3GPP TSG-CT WG1 Meeting #156	C1-254759

# Goteborg, Sweden, 25-29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.548 | CR | 0082 | rev | - | Current version: | 18.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor's notes under clause 6.2.3.10.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause 6.2.3.10.5 contains the following editor’s note, quote: Editor’s note: The SNRM client CoAP procedure is FFS. The specification (TS 24.548) fails to provide the SNRM client CoAP procedure for MBS session creation and MBS session announcement. Hence, it is needed to resolve the editor’s note so implementations based on CoAP are possible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MBS session announcement procedure and MBS session de-announcement procedure introduced at the SNRM client side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note remains in the specification when the release is frozen. Not complete specification as the necessary CoAP procedure (for MBS session creation and MBS session announcement) at the SNRM client side is not described. Not possible to implement the procedure by using CoAP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.10.5, (new) 6.2.3.10.5.1, (new) 6.2.3.10.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause A.3.2.2.3.3 is added by CR85 in (C1-254762). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.3.10.5	SNRM client CoAP procedure

* * * Next Change * * * *

###### 6.2.3.10.5.1	MBS session announcement procedure

Upon reception of a CoAP PUT request where the CoAP URI of the request identifies an MBS resource configuration resource as described in clause A.3.2.2.2.3.2, the SNRM-C:

a)	shall determine the identity of the sender of the received CoAP PUT request as specified in clause 6.2.1.2, and:

1)	if the identity of the sender of the received CoAP PUT request is not authorized to update the requested VAL group document, shall respond with a CoAP 4.03 (Forbidden) response to the CoAP PUT request and skip rest of the steps;

b)	shall support handling a CoAP PUT request from a SGM-C according to procedures specified in IETF RFC 7252 [23];

c)	shall create or update the MBS resource configuration resource pointed at by the CoAP URI with the content of "MbsResourceConfig" object received in the request and return a CoAP 2.01 (Created) or a CoAP 2.04 (Changed) response;

d)	if monitoring configuration is included in the "monitorConfig" object:

1)	if the "mbsListeningStatusNotify" attribute is present and is set to "true", the SNRM-C shall start sending listening status notification for the MBS session identified by the "mbsSessionId";

2)	if the "mbsListeningStatusNotify" attribute is not present or is present and is set to "false", the SNRM-C shall stop sending listening status notification of the MBS session;

3)	if the "mbsAnnouncementAck" attribute is present and is set to "true", the SNRM-C shall send an MBS announcement acknowledgement on receiving this announcement; and

4)	if the "mbsAnnouncementAck" attribute is not present or is present and is set to "false", the SNRM-C shall not send MBS announcement acknowledgement on receiving this announcement;

5)	if the "mbsSessionJoinNotify" attribute is present and is set to "true", the SNRM-C shall send session join notification for when the VAL user or UE joins the group;

6)	if the "mbsSessionJoinNotify" attribute is not present or is present and is set to "false", the SNRM-C shall not send send session join notification for when the VAL user or UE joins the group; and

e)	if session description protocol (SDP) information for MBS is included in "mbsSdp" attribute, shall use the SDP with media and application control information applicable to groups that can use the MBS session identified by the "mbsSessionId"; and

f)	if  announcement information for MBMS is included in the "mbmsAnnouncement" object, shall use it when attached to the LTE.

* * * Next Change * * * *

###### 6.2.3.10.5.2	MBS session de-announcement procedure

Upon reception of a CoAP DELETE request where the CoAP URI of the request identifies MBS resource configuration resource as described in clause A.3.2.2.2.3.3, the SNRM-C:

a)	shall determine the identity of the sender of the received CoAP DELETE request as specified in clause 6.2.1.2, and:

1)	if the identity of the sender of the received CoAP DELETE request is not authorized to delete the requested MBS resource configuration resource, shall respond with a CoAP 4.03 (Forbidden) response to the CoAP DELETE request and skip rest of the steps;

b)	shall support handling a CoAP DELETE request from a SNRM-S according to procedures specified in IETF RFC 7252 [23];

c)	shall delete the MBS resource pointed at by the CoAP URI;

d)	if monitoring configuration was included in the "monitorConfig" attribute, shall stop the monitoring accordingly; and

e)	shall check the condition for sending a listening status report.

* * * End of Changes * * * *