3GPP TSG-CT WG1 Meeting #156	C1-254981

# Goteborg, Sweden, 25-29 August 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4505 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction in faster service recovery_24301 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In CT1 meeting#154, C1-252204 was agreed to modify T3411 and T3402 for faster service recovery. However, there is an editorial correction required in sub-clause 5.2.2.3.3 (ATTACH-specific clause): if the timer T3402 is running, the UE detects the LongTimerEvent, the counter of attempts to tracking area updating while T3402 running is less than the MaxNumLongTimerReg; This CR proposes to do the correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial correction replacing “tracking area updating” with “attach” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial error shall remain in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.3.3	ATTEMPTING-TO-ATTACH

The UE:

-	shall initiate an attach or combined attach procedure on the expiry of timers T3411, T3402 or T3346 (see 3GPP TS 24.008 [13]);

-	may initiate an attach for emergency bearer services even if timer T3346 is running;

-	may initiate an attach if the UE is a UE configured to use AC11 - 15 in the selected PLMN, even if timer T3402, T3346 or timer T3447 is running;

-	shall initiate an attach or combined attach procedure when entering a new PLMN, if timer T3346 is running and the new PLMN is not equivalent to the PLMN where the UE started timer T3346, the PLMN identity of the new cell is not in one of the forbidden PLMN lists and the tracking area is not in one of the lists of forbidden tracking areas;

-	shall initiate an attach or combined attach procedure when the tracking area of the serving cell has changed, if timer T3346 is not running, the PLMN identity of the new cell is not in one of the forbidden PLMN lists and the tracking area of the new cell is not in one of the lists of forbidden tracking areas;

-	If the UE supports faster service recovery, is configured for faster service recovery in S1 mode as defined in the FasterRecovery leaf of the NAS configuration MO in 3GPP TS 24.368 [15A] and entry into this state was caused by abnormal cases aa or b defined in clause 5.5.1.2.6,

-	if the timer T3411 is running, the UE detects the ShortTimerEvent; and the counter of attempts to stop T3411 is less than the MaxNumShortTimerStop;

-	the UE may initiate the attach procedure and increment the counter of attempts to stop T3411.

-	if the timer T3402 is running, the UE detects the LongTimerEvent, the counter of attempts to attach while T3402 running is less than the MaxNumLongTimerReg;

-	the UE may re-enable the E-UTRA capability, initiate the attach procedure and increment the counter of attempts to attach while T3402 running.

NOTE 1:	When the counter of attempts to stop T3411 and the counter of attempts to attach while T3402 running reach its maximum value, UE follows the behaviour as if the UE is not configured with fast service recovery.

-	shall use requests for non-EPS services for non-emergency call from CM layers to trigger a combined attach procedure, if timer T3346 is not running (see clause 5.5.1.3), or to attempt to select GERAN or UTRAN radio access technology and proceed with the appropriate MM and CC specific procedures;

-	shall use requests for non-EPS services for emergency call from CM layers to attempt to select GERAN or UTRAN radio access technology and proceed with the appropriate MM and CC specific procedures;

-	may initiate an attach procedure upon receiving a request from upper layers to transmit user data related to an exceptional event and the UE is allowed to use exception data reporting (see the ExceptionDataReportingAllowed leaf of the NAS configuration MO in 3GPP TS 24.368 [15A]) if timer T3346 is not already running for "MO exception data" and even if timer T3402 or timer T3411 is running;

-	may initiate an attach procedure upon request of the upper layers to establish a PDN connection for emergency bearer services; and

-	may initiate an attach procedure upon request of the upper layers to establish a PDN connection for RLOS, if timer T3346 is not running.

* * * End of Change * * * *