3GPP TSG-CT WG1 Meeting #157	C1-256018

Sophia Antipolis, France, 13-17 October 2025

Source:	China Mobile

Title:	pCR on the paging collision and procedure collision

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This p-CR proposes to correct the requirement of AIoT device in the abnormal case of paging collision and solve the EN on the procedure collision in TS 24.369.

2. Reason for Change

According to RAN2#130 Meeting Report clause 8.2.2 in R2-2505002 and LS on parallel service request in R2-2504935:

8.2.2	A-IoT Paging

...

Agreements on parallel service request

- Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation.   Capture this in stage 2 specification.

- The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device.  Capture this in stage 3 specification.

- Send LS to RAN3 to notify them of agreements 1 and 2

- Parallel service request for overlapping reader scenario can be addressed in Rel-20

It is suggested to correct requirement of paging collision and remove the EN on procedure collision.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

5.2.5	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	Failure of authentication.

If an AIoT device fails the authentication procedure as specified in 3GPP TS 33.369 [6], the AIoT device shall not respond to the AIoT paging message.

b)	Collision of AIoT paging.

If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall abort the ongoing inventory procedure and process the new AIoT paging.

* * * Next Change * * * *

5.3.2.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The read command is unsuccessfully implemented (e.g. user memory operation error).

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

b)	The application data cannot be found in the requested place of the user memory.

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

* * * End of Changes * * * *