3GPP TSG-CT WG1 Meeting #157	C1-256027

Sophia-Antipolis, France, 13-17 October 2025

Source:	Samsung

Title:	Update SS_SmDataSourceRegistration open API for data source notification

Spec:	3GPP TS 24.550 v1.1.0

Agenda item:	19.42

Document for:	Agreement

1. Introduction

This p-CR proposes OPEN API changes for data source notification to 3GPP TS 24.550.

2. Reason for Change

In the current TS 24.550v1.1.0, in the SS_SmDataSourceRegistration open API the data source notification is yet to be defined. This pCR updates the SS_SmDataSourceRegistration open API with the data source notification changes agreed in tdoc C1-255513, that shall be invoked by the spatial map server to provide the data source notification to spatial map client.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.550 v1.1.0.

* * * First Change * * * *

# B.3	SS_SmDataSourceRegistration API

openapi: 3.0.0

info:

title: SM Server Data Source Registration and Notification Service

version: 1.0.0-alpha.0

description: >

API for Data Source Registration and Notification Service.

©2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA,

TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.550 V1.1.0; Digital asset, Spatial mapping and Spatial anchors

server - Service Enabler Architecture Layer for Verticals (SEAL); Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/24_series/24.550/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/sm_smds/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

paths:

/datasources-reg-lists:

post:

description: Register a new Data Source Registration.

operationId: DataSourceRegistration

tags:

- Data Source Registration List (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataSourceRegReq'

responses:

'201':

description: >

Created. The Data Source Registration List is successfully created and a representation of the created Individual Data Source Registration List resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/DataSourceRegReq'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

DataSourceNotification:

'{$request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataSourceNotification'

responses:

'204':

description: >

No Content. The Spatial Anchors Notification is successfully

received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/datasources-reg-lists/{dataSourceRegId}:

parameters:

- name: dataSourceRegId

in: path

description: Represents the identifier of the Individual Data Source Registration List resource.

required: true

schema:

type: string

put:

summary: Update an existing Individual Data Source Registration List.

operationId: UpdateIndDataSourceRegistrationList

tags:

- Individual Data Source Registration List (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataSourceRegReq'

responses:

'200':

description: >

OK. The Individual Data Source Registration List resource is successfully updated and

a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DataSourceRegReq'

'204':

description: >

No Content. The Individual Data Source Registration List resource is successfully

updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual Data Source Registration List.

operationId: ModifyIndDataSourceRegistrationList

tags:

- Individual Data Source Registration List (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataSourcePatchRegReq'

responses:

'200':

description: >

OK. The Individual Data Source Registration List resource is successfully modified and

a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DataSourceRegReq'

'204':

description: >

No Content. The Individual Data Source Registration List resource is successfully modified

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Data Source Registration List.

operationId: DeleteIndDataSourceRegistrationList

tags:

- Individual Data Source Registration List

responses:

'204':

description: >

No Content. The 'Individual Data Source Registration List' resource is successfully

deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

DataSourceRegReq:

description: >

Represents the Data Source Registration Request List.

type: object

properties:

requestorId:

type: string

description: Represents the identity of the requestor, either a VAL User or a VAL UE.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

valClientID:

type: string

description: Represents the identifier of the VAL client.

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

dsProfile:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

required:

- requestorId

- notificationDestination

- dsProfile

DataSourcePatchRegReq:

description: >

Represents the Data Source Patch Registration Request List.

type: object

properties:

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

dsProfile:

$ref: '#/components/schemas/DataSourceProfile'

DataSourceProfile:

description: Represents the profile information of the SM data source.

type: object

properties:

dsId:

type: string

description: Represents the identifier of the SM data source.

smInformation:

$ref: '#/components/schemas/SpatialMapInfoDetails'

required:

- dsId

- smInformation

SpatialMapInfoDetails:

description: Represents the Spatial Map Information.

type: object

properties:

smDataIdentifier:

type: string

description: Represents the identifier of the spatial map information.

smDataType:

type: string

description: Represents the type of the spatial map information provided by the VAL.

smRawDataFormat:

type: boolean

description: >

Indicates data format. When present, it shall be set as follows:

true: indicates the raw data format.

false: indicates processed data format.

Default value when omitted is "false".

smDataArea:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

smPosition:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/PositionInfo'

availabilityInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

dsUpdateIntervalInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- smDataIdentifier

- smDataType

- smDataArea

DataSourceNotification:

description: >

Represents the Data Source Notification.

type: object

properties:

requestorId:

type: string

description: >

Represents the identity of the requestor, either a VAL User or a VAL UE.

dataSourceRegId:

type: string

description: >

Represents the identifier of the SM data source registration resource.

dsNotificationInfo:

$ref: '#/components/schemas/DataSourceNotificationInfo'

required:

- requestorId

- dataSourceRegId

- dsNotificationInfo

DataSourceNotificationInfo:

description: >

Represents the detailed Data Source Notification information.

type: object

properties:

smDataIdentifier:

type: string

smDataType:

type: string

smRawDataFormat:

type: boolean

description: >

Indicates data format. When present, it shall be set as follows:

true: indicates the raw data format.

false: indicates processed data format.

Default value when omitted is "false".

smDurationInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

smFrequencyInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

smEndPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

required:

- smDataIdentifier

- smEndPt

* * * End of Changes * * * *