3GPP TSG-CT WG1 Meeting #157	C1-256050

Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Handling on collision of inventory and command procedure

Spec:	3GPP TS 24.369-100

Agenda item:	19.70

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

This CR proposes to solve the following EN in clause 5.3.2.6 of TS 24.369, add the collision cases as “abnormal case in the AIoT device” in clause 5.3.3.6 and clause 5.3.4.x,

Editor's note:	It is FFS the command and inventory procedure collision case.

This CR is based on the RAN2 agreement and SA3 progress as below:

In R2-2504935 (LS on parallel service request), RAN2 gives agreements on support of parallel service requests for overlapping reader scenario:

Agreements on parallel service request

- Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification.

- The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.

Besides, SA3 also points out in clause 5.2.2 of TS 33.369 that:

NOTE 1: An active attack may send a new paging request to the device while there is an ongoing procedure in device. The device will abort the ongoing procedure and respond to the new paging. The security measure to such denial-of-service attack is not specified in present document.

Both RAN2 and SA3 have agreed that if a new paging is received as an ongoing procedure in progress, the AIoT device will abort the ongoing procedure and respond to the new paging.

Based on the agreement of RAN2 and SA3, and the feasibility of different options without RAN2 impact, this CR proposes to abort the ongoing command procedure and process the new AIoT paging in case of collision.

Related DP is C1-256048.

Besides, in clause 5.3.5, the title format is wrong. The correct format should be 5.3.4.5, and this clause should be added as clause 5.3.2.x and clause 5.3.3.x

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 V1.0.0.

* * * First Change * * * *

5.3.2.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The read command is unsuccessfully implemented (e.g. user memory operation error).

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

b)	The application data cannot be found in the requested place of the user memory.

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

c)	Collision of AIoT paging and read command.

If a new AIoT paging is received while a read command procedure is in progress, the AIoT device shall abort the ongoing read command procedure and process the new AIoT paging.

* * * Next Change * * * *

5.3.3.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The write command is unsuccessfully implemented (e.g. user memory operation error)

The AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

b)	Collision of AIoT paging and write command

If a new AIoT paging is received while a write command procedure is in progress, the AIoT device shall abort the ongoing write command procedure and process the new AIoT paging. The rest handling is implementation-specific.

* * * Next Change * * * *

5.3.4.5	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of a command failure received as specified by 3GPP TS 38.413 [10], the AIOTF shall abort the command procedure.

a)	No response received from the AIoT device

The AIOTF shall consider the AIoT device has locally disabled the communication capability.

* * * Next Change * * * *

5.3.4.x	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	Collision of AIoT paging and permanent disable command.

If a new AIoT paging is received while a permanent disable command procedure is in progress, the AIoT device shall locally disable the communication capability, and ignore the new received inventory.

* * * Next Change * * * *

5.3.2.x	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of a command failure received as specified by 3GPP TS 38.413 [10], the AIOTF shall abort the command procedure.

* * * Next Change * * * *

5.3.3.x	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of a command failure received as specified by 3GPP TS 38.413 [10], the AIOTF shall abort the command procedure.

* * * End of Changes * * * *