3GPP TSG-CT WG1 Meeting #157	C1-256100

# Sophia-Antipolis, FR, 13 - 17 October 2025

Source:	Ericsson

Title:	Ericsson proposals to 6G SID on Study on optimal usage of CDDL specification for UE-to-network interfaces for verticals in 6G

Document for:	Discussion & Agreement

# Agenda Item:	20.2

# Work Item / Release:	6G/Rel-20

Abstract of the contribution:

# 1	Decision/action requested

Discuss the proposals to 6G SID on Study on optimal usage of CDDL specification for UE-to-network interfaces for verticals in 6G SID

# 2	References

[1]	3GPP TS 24.544 "Group Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification"

[2]	3GPP TS 24.543 "Data delivery management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol Specification"

[3]	3GPP TS 24.545 "Location Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification"

[4]	3GPP TS 24.548 "Network Resource Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification"

# 3	Rationale

CT1 starts discussions on the 6G Study(ies) and related scoping. That discussion paper aims to represent Ericsson vision on optimal usage of CDDL specification for UE-to-network interfaces for verticals.

# 4	Discussion

## 4.1	Current CDDL Naming Conventions and Extensibility Features

Currently in CT1 specs, there is no formal naming conventions or extensibility rules to follow. This discussion paper defines the issues with existing specs on CDDL and proposes a study to optimize the naming conventions and extensibility features of CDDL in CT1. There are different specifications in CT1 using CDDL:

| Working Group | Specification | Observations |
| --- | --- | --- |
| CT1 | Group Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification; TS 24.544 | Naming conventions: The naming conventions for attributes, data types, enumerations etc are consistent. This specification can be considered as a good example for naming conventions for CDDL. Extensibility: No extensibility is needed in the TS, because no new attributes are introduced in the later versions. Interoperability considerations are stated in the TS; however possible interoperability issues are identified. |
| CT1 | Data delivery management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; 24.543 | Naming conventions: The naming conventions for attributes, data types, enumerations etc are not consistent. Extensibility: The extensibility issues are found. For example: several attributes such as batAndPeriodicityCapability, l4sFeedbackCapability, transmisAssistInfo etc are new in v19. Interoperability considerations are stated in the TS; however possible interoperability issues are identified when a Rel-18 receiver parses and validates CBOR document compliant to Rel-19 CDDL schema and containing Rel-19 optional attributes, against Rel-18 CDDL schema (see C1-256162, C1-256159). |
| CT1 | Location Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification; 24.545 | Naming conventions: The naming conventions for attributes, data types, enumerations etc are not consistent. Extensibility: The extensibility issues are found. For example: several attributes such as VelocityInfo, Failure, verification, ConfirmLocationReport etc are new in v19. Interoperability considerations are stated in the TS; however possible interoperability issues are identified, similar as those for 24.543 |
| CT1 | Network Resource Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification; 24.548 | Naming conventions: The naming conventions for attributes, data types, enumerations etc are consistent. Extensibility: No extensibility is needed in the TS, because no new attributes are introduced in the later versions. Interoperability considerations are stated in the TS; however possible interoperability issues are identified. |

Observation 1: CT1 does not apply a common naming conventions for CDDL documentation in all TS. This leads to unharmonized specifications in CT1.

Observation 2: Even though CT1 defined "Applications must ignore any key-value pairs that they do not understand.", interoperability issues were identified in CDDL schema definition.

Considering the status of existing CT1 specification for CDDL, a study is proposed.

Proposal: CT1 should study the optimal usage of CDDL specification, including naming conventions and extensibility mechanisms for UE-to-network interface for verticals in 6G. The study of optimal usage of CDDL specification does not have any Stage 2 dependency.

# 5	Conclusion

Ericsson proposes to agree the Work Area with the related Work Tasks as described in section 4.1 and include in common or dedicated 6G SID.