3GPP TSG-CT WG1 Meeting #157	C1-256188

# Sophia Antipolis, France, 13-17 October 2025

Source:	Qualcomm Incorporated

Title:	Discussion on the NAS overhead for transport of IMS voice over GEO via control plane

Agenda item:	23

Document for:	DISCUSSION

# Background

SA2 is conducting a Rel-20 Study on Integration of satellite components in the 5G architecture Phase 4. The first work task in this study is “Support for IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC”. In the context of this work, SA2 has sent an LS to CT1 in C1-256067. This paper provides the background for the question directed at CT1 and proposes an answer. The proposed reply LS is provided in C1-256187.

SA2 asked following question to CT1:

Question 8: In addition to question 7 related to security, is there any further possibility to reduce the NAS overhead further than what the rel.19 NORDAT_CP WI provided in order to transport voice packets over NB-IoT NTN?

The context for question 8 is in question 7 to SA3:

Question 7: Considering that in the context of alternative solutions documented in TR 23.700-19 for the support of IMS voice over NB-IoT NTN connected to EPC, a specific SRB (i.e. via a dedicated EPS bearer for Data over NAS) will be used for transfer of voice media packets only, is there a concern to eliminate the 5 bytes of NAS layer security overhead?

Based on the above, the background for the question to CT1 is Solution 5 in the TR 23.700-19, which proposes to specify a new SRB dedicated to the transport of voice packets via control plane. SA2 would like to understand the minimum NAS layer overhead for this solution. The bearer mapping for this solution is shown in Figure 1 (Figure 6.5.1.4-1 in TR 23.700-19):

Figure 1 - Bearer mapping for IMS voice over GEO via NB-IoT NTN connected to EPC using CP CIoT for Solution 5 in TR 23.700-19

In this solution, the MME is made aware that a NAS PDU is received via SRBx, as per TR 23.007-19 clause 6.5.1.6.2: To associate the NAS PDU with SRBx, the eNB could insert an indication e.g. signalling bearer identity in the S1-AP NAS transport messages (similar to how the “EDT session IE” Boolean flag indicates the association with an EDT session in the S1-AP Initial UE message).

It is clear from Figure 1, and is also indicated in question 8, that the “specific SRB” (SRBx) is used for transfer of voice packets only.

Based on the above, we make the following observation:

Observation 1: MME can distinguish NAS PDUs carrying voice packets from other NAS PDUs.

# Discussion

In the Rel-19 work on NORDAT_CP, CT1 specified a new EMM TRANSPORT message with reduced overhead, shown in Figure 2:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Security header type |  |  |  | Protocol descriptor (EMM) |  |  |  | octet 1 |
| Message authentication code |  |  |  |  |  |  |  | octet 2 octet 5 |
| Sequence number |  |  |  |  |  |  |  | octet 6 |
| Data container |  |  |  |  |  |  |  | octet 7 octet n |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data type |  |  | DDX |  | EPS bearer identity |  |  | octet 1 |
| Data payload |  |  |  |  |  |  |  | octet 2 octet n |

Data container

Figure 2 – EMM TRANSPORT message

In this section we discuss the possibility of reducing the overhead in Figure 2 in the context of the solution in Section 1. We focus on the three fields in octet 7 (octet 1 in the Data container).

Data type field

Based on Observation 1, this field can be removed if the MME can leverage its ability to distinguish NAS PDUs carrying voice packets.

DDX field

Since this is voice service in which voice packets are sent continuously during the call, the DDX field is not needed.

EPS bearer identity field

There can be only one “specific SRB” (SRBx). Since the MME knows the EPS bearer identity assigned to SRBx, it may not be necessary to send the EPS bearer identity in the NAS message.

Observation 2: It could be possible to create a new message for the transport of voice packets using the “specific SRB” by eliminating octet 7 in the EMM TRANSPORT message.

# Conclusion

Observation 1: MME can distinguish NAS PDUs carrying voice packets from other NAS PDUs.

Observation 2: It could be possible to create a new message for the transport of voice packets using the “specific SRB” by eliminating octet 7 in the EMM TRANSPORT message.

Proposal: CT1 answers to SA2 that CT1 could consider further reduction of NAS overhead by one additional octet, if SA2 provides stage 2 requirements for such work.

The proposed reply LS is provided in C1-256187.