3GPP TSG-CT WG1 Meeting #157	C1-256198

# Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.572 | CR | 0126 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for multiple LCS UP connections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage 2 requirements for LCS UP in Release 19 have been updated to support multiple LCS UP connections per UE, with a maximum of one connection to each LMF. Stage 3 specifications need to be updated to support this functionality at the protocol level. To ensure compliance with the Stage 2 requirements, each LCS UP connection must be uniquely identifiable on both UE and LMF side, including during operations such as LMF relocation where both the old and new connections need to be referenced. It is proposed that the UE assigns a connection identifier during the establishment of each LCS UP connection. This identifier is provided to the LMF and will be used to reference subsequent operations related to that specific connection. The connection identifier should be assigned by the UE and must be unique within the scope of the UE. The method for selecting the identifier can be implementation-specific. Since the number of simultaneous LCS UP connections per UE is expected to be limited, the range of identifiers can also be constrained. In the case of LCS-UP connection modification, an abnormal scenario may occur where the UE cannot locate the LCS-UP connection identified by the source LCS-UP connection ID. In such cases, explicit error handling is required. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | LCS secured user plane connection identity is added to the user plane establish command message, the user plane establish complete message, and the user plane release command message. User plane connection modification procedure is added with using LCS secured user plane connection identity. New messages for user plane connection modification and new IE for LCS secured user plane connection identity are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 does not support functionality for muliple LCS secured user plane connections as required by stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2, 6.2.1.1.2, 6.2.1.1.3, 6.2.1.1.4, 6.2.1.2.2, 6.2.1.2.3, 6.2.1.x (new), 6.x.1.x.1(new), 6.2.1.x.2(new), 6.2.1.x.3(new), 6.2.1.x.4(new), 6.2.1.x.5 (new), 10.3.1.1, 10.3.1.x5 (new), 10.3.2.1, 10.3.2.x5 (new), 10.3.6.1, 10.3.6.x6 (new), 10.3.8.1, 10.3.8.x7 (new), 10.3.y (new), 10.3.y.1 (new), 10.3.y.2 (new), 10.3.y.3 (new), 10.3.z (new), 10.3.z.1 (new), 10.3.z1 (new), 10.3.z1.1 (new), 11.1.3, 11.3.x3 (new), 12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.1.2	Types of UPP-CM procedures

Two types of UPP-CM procedures can be distinguished:

a)	Procedures related to establishing the LCS secured user plane connection for LCS-UPP:

1)	Initiated by the network:

i)	network initiated user plane connection establishment procedure.

2)	Initiated by the UE:

i)	UE requested user plane connection establishment procedure.

b)	Procedures related to releasing the LCS secured user plane connection for LCS-UPP:

1)	Initiated by the network:

i)	network initiated user plane connection release procedure.

2)	Initiated by the UE:

i)	UE requested user plane connection release procedure.

c)	Procedure related to modifying the LCS secured user plane connection for LCS-UPP:

1)	Initiated by the network:

i)	network initiated user plane connection modification procedure.

* * * Next Change * * * *

##### 6.2.1.1.2	Network initiated user plane connection establishment procedure initiation by the network

The LMF initiates the network initiated user plane connection establishment procedure by sending the USER PLANE CONNECTION ESTABLISHMENT COMMAND message to the UE (see example in figure 6.2.1.1.2.1). The LMF shall:

a0)	allocate a unique LCS-UP binding ID value and associate the LCS-UP binding ID value with the UE identity (i.e., SUPI, GPSI, or both);

a)	create the USER PLANE CONNECTION ESTABLISHMENT COMMAND message according to clause 10.3.1;

b)	send the USER PLANE CONNECTION ESTABLISHMENT COMMAND message to the UE; and

c)	start a timer T5012 upon sending the USER PLANE CONNECTION ESTABLISHMENT COMMAND message.

If the LMF initiates the network initiated user plane connection establishment procedure for replacing an existing LCS secured user plane connection with the source LMF as specified in subclause 6.18.3 in 3GPP TS 23.273 [2], the LMF shall include the Source LCS secured user plane connection identity IE and set the value to the LCS secured user plane connection identity received from the source LMF.

Figure 6.2.1.1.2.1: Network initiated user plane connection establishment procedure

* * * Next Change * * * *

##### 6.2.1.1.3	Network initiated user plane connection establishment procedure accepted by the UE

Upon receipt of a USER PLANE CONNECTION ESTABLISHMENT COMMAND message from the LMF, the UE shall stop timer T5011, if running, and timer T5014, if running, delete the stored LCS-UP binding ID, if any, and store the LCS-UP binding ID and the LMF LCS-UP address received in the USER PLANE CONNECTION ESTABLISHMENT COMMAND message.

If the USER PLANE CONNECTION ESTABLISHMENT COMMAND message can be accepted, the UE shall:

a)	establish a PDU session providing PDU connectivity service between the UE and the LMF, if not available, as described in clause 4.2.2;

b)	establish a TLS connection between the UE and the LMF, as described in clause 4.3 after the required PDU connectivity service between the UE and the LMF is available;

c)	perform an LCS-UP connection binding procedure as described in clause 7.3.4 after the successful TLS connection establishment; and

d)	assign an LCS secured user plane connection identifier value to the newly established secured user plane connection, if the UE supports the multiple LCS secured user plane connections and the network has indicated that it supports the multiple LCS secured user plane connections as specified in clause 5.5.1.2 or clause 5.5.1.3 of 3GPP TS 24.501 [4].

NOTE:	The LCS secured user plane connection identifier value selection is up to the UE implementation.

After the LCS-UP connection binding procedure is completed successfully as described in clause 7.3.4, the UE shall:

a)	create the USER PLANE CONNECTION ESTABLISHMENT COMPLETE message according to clause 10.3.2;

b)	send the USER PLANE CONNECTION ESTABLISHMENT COMPLETE message to the LMF;

c)	consider the LCS secured user plane connection between the UE and the LMF is established, and

d)	if the Source LCS secured user plane connection identity IE is included in the USER PLANE CONNECTION ESTABLISHMENT COMMAND message, the UE shall associate all location sessions associated with the LCS secured user plane connection identified by the Source LCS secured user plane connection identity IE to the newly established LCS secured user plane connection, and stop using the LCS secured user plane connection identified by the Source LCS secured user plane connection identity IE.

* * * Next Change * * * *

##### 6.2.1.1.4	Network initiated user plane connection establishment procedure completion by the network

Upon receipt of a USER PLANE CONNECTION ESTABLISHMENT COMPLETE message from the UE, the LMF shall stop the timer T5012 and shall consider that the LCS secured user plane connection between the UE and the LMF is established.

If the USER PLANE CONNECTION ESTABLISHMENT COMPLETE message from the UE includes an LCS secured user plane connection identity IE, the LMF shall associate the value of the LCS secured user plane connection identity IE to the newly established LCS secured user plane connection between the UE and the LMF.

* * * Next Change * * * *

##### 6.2.1.2.2	Network initiated user plane connection release procedure initiation by LMF

The LMF initiates the network initiated user plane connection release procedure by sending the USER PLANE CONNECTION RELEASE COMMAND message to the UE, as shown in figure 6.2.1.2.2.1, the LMF:

a)	shall generate the USER PLANE CONNECTION RELEASE COMMAND message according to subclause 10.3.6;

b)	shall send the USER PLANE CONNECTION RELEASE COMMAND message to the UE; and

c)	shall start a timer T5010 upon sending the USER PLANE CONNECTION RELEASE COMMAND message.

The LMF shall include the LCS secured user plane connection identity IE in the USER PLANE CONNECTION RELEASE COMMAND message if the LCS secured user plane connection to be released is associated with an LCS secured user plane connection identity.

The LMF may include the Back-off timer value IE in the USER PLANE CONNECTION RELEASE COMMAND message.

Figure 6.2.1.2.2.1: Network initiated user plane connection release procedure

* * * Next Change * * * *

##### 6.2.1.2.3	Network initiated user plane connection release procedure accepted by UE

Upon receipt of a USER PLANE CONNECTION RELEASE COMMAND message from the LMF, the UE shall determine to which LCS secured user plane connection the message applies to as follows:

a) if the LCS secured user plane connection identity IE is included in the USER PLANE CONNECTION RELEASE COMMAND message, the message applies to the LCS secured user plane connection identified by the value of the LCS secured user plane connection identity IE; or

b) if the LCS secured user plane connection identity IE is not included in the USER PLANE CONNECTION RELEASE COMMAND message, the message applies to the LCS secured user plane connection established at the UE, if any.

NOTE:	The LCS secured user plane connection identity IE is not included in the USER PLANE CONNECTION RELEASE COMMAND message if the UE or the LMF or both do not support the multiple LCS secured user plane connections. In this case, there must be only one LCS secured user plane connection for the UE.

For the LCS secured user plane connection to which the USER PLANE CONNECTION RELEASE COMMAND message applies, the UE shall stop the timer T5013 if running and shall:

a)	stop sending LCS-UPP messages over the LCS secured user plane connection to be released;

b)	terminate the TLS connection used for the LCS secured user plane connection to be released;

c)	consider the LCS secured user plane connection between the UE and the LMF as released;

d)	create a USER PLANE CONNECTION RELEASE COMPLETE message according to clause 10.3.7; and

e)	send the USER PLANE CONNECTION RELEASE COMPLETE message to the LMF.

If the Back-off timer value IE is included in the USER PLANE CONNECTION RELEASE COMMAND message, the UE shall start the timer T5014 with the value provided in the Back-off timer value IE after sending the USER PLANE CONNECTION RELEASE COMPLETE message to the LMF. The UE shall not initiate the UE requested user plane connection establishment procedure as specified in subclause 6.2.2.1, until timer T5014 expires or timer T5014 is stopped.

Upon reception of a USER PLANE CONNECTION RELEASE COMPLETE message from the UE, the LMF shall stop the timer T5010 and shall consider the LCS secured user plane connection between the UE and the LMF as released.

* * * Next Change * * * *

#### 6.2.1.x	Network initiated user plane connection modification procedure

##### 6.2.1.x.1	General

The purpose of the network initiated user plane connection modification procedure is to enable the network to modify the LCS secured user plane connection between the UE and the LMF as described in clause 6.18.3 of 3GPP TS 23.273 [2]. The USER PLANE CONNECTION MODIFICATION COMMAND message is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message and the USER PLANE CONNECTION MODIFICATION COMPLETE message is encapsulated in the UPP-CMI container of the UL NAS TRANSPORT message as defined in 3GPP TS 24.501 [4].

* * * Next Change * * * *

##### 6.2.1.x.2	Network initiated user plane connection modification procedure initiation by LMF

The LMF initiates the network initiated user plane connection modification procedure by sending the USER PLANE CONNECTION MODIFICATION COMMAND message to the UE, as shown in figure 6.2.1.x.2.1. The LMF shall:

a)	generate the USER PLANE CONNECTION MODIFICATION COMMAND message according to subclause 10.3.y;

b)	send the USER PLANE CONNECTION MODIFICATION COMMAND message to the UE; and

c)	start a timer Txxxx upon sending the USER PLANE CONNECTION MODIFICATION COMMAND message.

The LMF may include the Source LCS secured user plane connection identity IE and the Target LCS secured user plane connection identity IE in the USER PLANE CONNECTION MODIFICATION COMMAND message as specified in clause 6.18.3 of 3GPP TS 23.273 [2].

Figure 6.2.1.x.2.1: Network initiated user plane connection modification procedure

* * * Next Change * * * *

##### 6.2.1.x.3	Network initiated user plane connection modification procedure accepted by UE

Upon receipt of a USER PLANE CONNECTION MODIFICATION COMMAND message, if the USER PLANE CONNECTION MODIFICATION COMMAND message can be accepted, the UE shall:c)	send the USER PLANE CONNECTION MODIFICATION COMPLETE message to the LMF.

a)	if the Source LCS secured user plane connection identity IE is included:

1)	identify the user plane connection using the source LCS secured user plane connection identity;

2)	associate all location sessions associated with the source LMF's user plane connection, if any, to the LCS secured user plane connection identified by the Target LCS secured user plane connection identity;

3)	stop sending the LCS-UPP messages via the LCS secured user plane connection identified by the source LCS secured user plane connection; and

4)	start sending LCS-UPP messages for the location sessions found in 2) via the LCS secured user plane connection identified by the Target LCS secured user plane connection identity;

b)	create a USER PLANE CONNECTION MODIFICATION COMPLETE message according to clause 10.3.z; and

Upon reception of a USER PLANE CONNECTION MODIFICATION COMPLETE message from the UE, the LMF shall stop the timer Txxxx and shall consider the modification of the LCS secured user plane connection between the UE and the LMF as successful.

* * * Next Change * * * *

##### 6.2.1.x.4	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of the timer Txxxx

The LMF shall, on the first expiry of the timer Txxxx, retransmit the USER PLANE CONNECTION MODIFICATION COMMAND message and shall reset and start timer Txxxx. This retransmission is repeated up to four times, i.e. on the fifth expiry of timer Txxxx, the LMF shall abort the ongoing network initiated user plane connection modification procedure on the LCS secured user plane connection.

* * * Next Change * * * *

##### 6.2.1.x.5	Abnormal cases on the UE side

The following abnormal cases can be identified:

a)	Source LCS secured user plane connection is not available

If the LCS secured user plane connection identified by the Source LCS secured user plane identity IE received from the LMF is not available in the UE, The UE shall generate the USER PLANE CONNECTION MODIFICATION FAILURE message according to clause 10.3.z, and include the Failure cause IE with the cause value set to #4 "User plane not available".

Upon reception of a USER PLANE CONNECTION MODIFICATION FAILURE message from the UE, the LMF shall stop the timer Txxxx, and consider the network initiated user plane connection modification procedure as not successful. If cause value #4 "User plane not available" is included in the USER PLANE CONNECTION MODIFICATION FAILURE message, the LMF may consider to re-initiate available LCS session(s) from the source LMF over the secured LCS user plane connection to the UE.

* * * Next Change * * * *

#### 10.3.1.1	Message definition

The USER PLANE CONNECTION ESTABLISHMENT COMMAND message is sent by the LMF to the UE to transport the user plane connection information and trigger the UE to establish the LCS secured user plane connection towards the LMF. See table 10.3.1.1.1.

Message type:	USER PLANE CONNECTION ESTABLISHMENT COMMAND

Significance:	dual

Direction:	network to UE

Table 10.3.1.1.1: USER PLANE CONNECTION ESTABLISHMENT COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION ESTABLISHMENT COMMAND message identity | Message type 11.1.3 | M | V | 1 |
|  | LCS-UP binding ID | LCS-UP binding ID 11.3.4 | M | LV | 5-256 |
|  | LMF LCS-UP address | LMF LCS-UP address 11.3.1 | M | LV | 3-256 |
| X2 | Source LCS secured user plane connection identity | LCS secured user plane connection identity 11.3.x3 | O | TLV | 3 |

* * * Next Change * * * *

#### 10.3.1.x5	Source LCS secured user plane connection identity

The LMF shall include this information element to the UE if the new LCS secured user plane connection shall replace an existing LCS secured user plane connection, as specified in subclause 6.18.3 in 3GPP TS 23.273 [2].

* * * Next Change * * * *

#### 10.3.2.1	Message definition

The USER PLANE CONNECTION ESTABLISHMENT COMPLETE message is sent by the UE to the LMF in response to a USER PLANE CONNECTION ESTABLISHMENT COMMAND message and confirms the availability of the required PDU connectivity service between UE and LMF. See table 10.3.2.1.1.

Message type:	USER PLANE CONNECTION ESTABLISHMENT COMPLETE

Significance:	dual

Direction:	UE to network

Table 10.3.2.1.1: USER PLANE CONNECTION ESTABLISHMENT COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION ESTABLISHMENT COMPLETE message identity | Message type 11.1.3 | M | V | 1 |
| x3 | LCS secured user plane connection identity | LCS secured user plane connection identity 11.3.x3 | O | TLV | 3 |

* * * Next Change * * * *

#### 10.3.2.x5	LCS secured user plane connection identity

If the UE supports the multiple LCS secured user plane connections and the network has indicated that it supports the multiple LCS secured user plane connections as specified in clause 5.5.1.2 or clause 5.5.1.3 of 3GPP TS 24.501 [4], the UE shall include this information element to the LMF.

* * * Next Change * * * *

#### 10.3.6.1	Message definition

The USER PLANE CONNECTION RELEASE COMMAND message is sent by the LMF to the UE to release the LCS secured user plane connection between the UE and the LMF. See table 10.3.6.1.1.

Message type:	USER PLANE CONNECTION RELEASE COMMAND

Significance:	dual

Direction:	network to UE

Table 10.3.6.1.1: USER PLANE CONNECTION RELEASE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION RELEASE COMMAND message identity | Message type 11.1.3 | M | V | 1 |
| 10 | Back-off timer value | GPRS timer 3 11.3.2 | O | TLV | 3 |
| x3 | LCS secured user plane connection identity | LCS secured user plane connection identity 11.3.x3 | O | TLV | 3 |

* * * Next Change * * * *

#### 10.3.6.x6	LCS secured user plane connection identity

The LMF shall include this information element to the UE, if the LCS secured user plane connection to be released is associated with an LCS secured user plane connection identity.

* * * Next Change * * * *

#### 10.3.8.1	Message definition

The USER PLANE CONNECTION RELEASE REQUEST message is sent by the UE to the LMF to request to release the LCS secured user plane connection between the UE and the LMF. See table 10.3.8.1.1.

Message type:	USER PLANE CONNECTION RELEASE REQUEST

Significance:	dual

Direction:	UE to network

Table 10.3.8.1.1: USER PLANE CONNECTION RELEASE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION RELEASE REQUEST message identity | Message type 11.1.3 | M | V | 1 |
| 21 | Failure cause | Failure cause 11.3.3 | O | TV | 2 |
| x3 | LCS secured user plane connection identity | LCS secured user plane connection identity 11.3.x3 | O | TLV | 3 |

* * * Next Change * * * *

#### 10.3.8.x7	LCS secured user plane connection identity

The UE shall include this information element, if the LCS secured user plane connection requested to be released is associated with an LCS secured user plane connection identity.

* * * Next Change * * * *

### 10.3.y	User plane connection modification command

#### 10.3.y.1	Message definition

The USER PLANE CONNECTION MODIFICATION COMMAND message is sent by the LMF to the UE to modify the LCS secured user plane connection between the UE and the LMF. See table 10.3.y.1.1.

Message type:	USER PLANE CONNECTION MODIFICATION COMMAND

Significance:	dual

Direction:	network to UE

Table 10.3.y.1.1: USER PLANE CONNECTION MODIFICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION MODIFICATION COMMAND message identity | Message type 11.1.3 | M | V | 1 |
| yy | Source LCS secured user plane connection identity | LCS secured user plane connection identity 11.3.x3 | O | TLV | 3 |
| yz | Target LCS secured user plane connection identity | LCS secured user plane connection identity 11.3.x3 | O | TLV | 3 |

* * * Next Change * * * *

#### 10.3.y.2	Source LCS secured user plane connection identity

The LMF shall include this information element to the UE for modification of user plane connection to indicate the source LCS secured user plane connection to be relocated, as specified in subclause 6.18.3 in 3GPP TS 23.273 [2].

* * * Next Change * * * *

#### 10.3.y.3	Target LCS secured user plane connection identity

The LMF shall include this information element to the UE for modification of user plane connection to indicate the target LCS secured user plane connection to be modified, as specified in subclause 6.18.3 in 3GPP TS 23.273 [2].

* * * Next Change * * * *

### 10.3.z	User plane connection modification complete

#### 10.3.z.1	Message definition

The USER PLANE CONNECTION MODIFICATION COMPLETE message is sent by the UE to the LMF to accept a USER PLANE CONNECTION MODIFICATION COMMAND message and indicates confirmation for the modification of the LCS secured user plane connection between the UE and the LMF. See table 10.3.z.1.1.

Message type:	USER PLANE CONNECTION MODIFICATION COMPLETE

Significance:	dual

Direction:	UE to network

Table 10.3.z.1.1: USER PLANE CONNECTION MODIFICATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION MODIFICATION COMPLETE message identity | Message type 11.1.3 | M | V | 1 |

* * * Next Change * * * *

### 10.3.z1	User plane connection modification failure

#### 10.3.z1.1	Message definition

The USER PLANE CONNECTION MODIFICATION FAILURE message is sent by the UE to the LMF in response to a USER PLANE CONNECTION MODIFICATION COMMAND message and indicates the failure of the modification of the LCS secured user plane connection between the UE and the LMF. See table 10.3.z1.1.1.

Message type:	USER PLANE CONNECTION MODIFICATION FAILURE

Significance:	dual

Direction:	UE to network

Table 10.3.z1.1.1: USER PLANE CONNECTION MODIFICATION FAILURE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION MODIFICATION COMPLETE message identity | Message type 11.1.3 | M | V | 1 |
|  | Failure cause | Failure cause 11.3.3 | M | V | 1 |

* * * Next Change * * * *

### 11.1.3	Message type

The Message type information element and its use are defined in 3GPP TS 24.007 [11]. Table 11.1.3.1 defines the value part of the message type information element used in the LCS-UPP and table 11.1.3.2 defines the value part of the message type information element used in the UPP-CM.

The Message type is a type 3 information element, with the length of 1 octet.

Table 11.1.3.1: Message type for LCS-UPP

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | - | - | - | - | - | - |  | LCS-UPP messages |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | UL LCS-UP transport |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | DL LCS-UP transport |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | LCS-UP connection binding request |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | LCS-UP connection binding accept |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | LCS-UP connection binding reject |

Table 11.1.3.2: Message type for UPP-CM

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 1 | 1 | - | - | - | - | - | - |  | UPP-CM messages |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | User plane connection establishment command |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | User plane connection establishment complete |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | User plane connection establishment failure |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | User plane connection establishment request |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | User plane connection establishment reject |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | User plane connection release command |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  | User plane connection release complete |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 |  | User plane connection release request |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |  | User plane connection modification command |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |  | User plane connection modification complete |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |  | User plane connection modification failure |

* * * Next Change * * * *

### 11.3.x3	LCS secured user plane connection identifier

The purpose of the LCS secured user plane connection identifier information element is to identify the LCS secured user plane connection between the UE and the LMF.

The LCS secured user plane connection identifier information element is coded as shown in figure 11.3.xx.1 and table 11.3.xx.1.

The LCS secured user plane connection identifier information element is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCS secured user plane connection identifier IEI |  |  |  |  |  |  |  | octet 1 |
| Length of LCS secured user plane connection identifier contents |  |  |  |  |  |  |  | octet 2 |
| LCS secured user plane connection identifier value |  |  |  |  |  |  |  | octet 3 |

Figure 11.3.xx.1: LMF identifier information element

Table 11.3.xx.1: LMF identifier information element

| LCS secured user plane connection identifier value (octet 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | reserved |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |
| to |  |  |  |  |  |  |  |  | possible values for the LCS secured user plane connection identifier |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |  |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | reserved |

* * * Next Change * * * *

## 12.3	Timers of UPP-CM

Timers of UPP-CM are shown in table 12.3.1 and table 12.3.2.

Table 12.3.1: Timers of UPP-CM – UE side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY (NOTE 1) |
| --- | --- | --- | --- | --- |
| T5011 | 16s | Transmission of USER PLANE CONNECTION ESTABLISHMENT REQUEST message | USER PLANE CONNECTION ESTABLISHMENT COMMAND message received USER PLANE CONNECTION ESTABLISHMENT REJECT message received | Retransmission of USER PLANE CONNECTION ESTABLISHMENT REQUEST message |
| T5013 | 16s | Transmission of USER PLANE CONNECTION RELEASE REQUEST message | USER PLANE CONNECTION RELEASE COMMAND message received | Retransmission of USER PLANE CONNECTION RELEASE REQUEST message |
| T5014 | NOTE 2 | USER PLANE CONNECTION ESTABLISHMENT REJECT received with a back-off timer value as specified in subclause 6.2.2.1.4. USER PLANE CONNECTION RELEASE COMMAND received with a back-off timer value and USER PLANE CONNECTION RELEASE COMPLETE sent as specified in subclause 6.2.1.2.3. | USER PLANE CONNECTION ESTABLISHMENT COMMAND message received | None |
| NOTE 1: Typically, the procedures are aborted on the fifth expiry of the relevant timer. Exceptions are described in the corresponding procedure description. NOTE 2: The value of this timer is provided by the network. |  |  |  |  |

Table 12.3.2: Timers of UPP-CM – LMF side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY (NOTE 1) |
| --- | --- | --- | --- | --- |
| T5012 | NOTE 2 | Transmission of USER PLANE CONNECTION ESTABLISHMENT COMMAND message | USER PLANE CONNECTION ESTABLISHMENT COMPLETE message received USER PLANE CONNECTION ESTABLISHMENT FAILURE message received | Retransmission of USER PLANE CONNECTION ESTABLISHMENT COMMAND message |
| T5010 | 16s | Transmission of USER PLANE CONNECTION RELEASE COMMAND message | A USER PLANE CONNECTION RELEASE COMPLETE message received | Retransmission of USER PLANE CONNECTION RELEASE COMMAND message |
| Txxxx | 16s | Transmission of USER PLANE CONNECTION MODIFICATION COMMAND message | USER PLANE CONNECTION MODIFICATION COMPLETE message received | Retransmission of USER PLANE CONNECTION MODIFICATION COMMAND message |
| NOTE 1: Typically, the procedures are aborted on the fifth expiry of the relevant timer. Exceptions are described in the corresponding procedure description. NOTE 2: The timer value of T5012 is implementation specific and set to be inclusive NAS procedure timer handling (e.g., for PDU session establishment), TLS connection establishment, and user plane connection binding procedure. |  |  |  |  |

* * * End of Change * * * *