3GPP TSG-CT WG1 Meeting #157	C1-256260

Sophia Antipolis, France, 13-17 October 2025

Source:	China Mobile

Title:	pCR on the EN for command not supported

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This p-CR proposes to solve the ENs on read and write command not supported cases in 5.3.2.5 and 5.3.3.5.

2. Reason for Change

The following ENs were added in TS 24.369 in CT1#156

5.3.2.5	Read command procedure not accepted by AIoT device

...

Editor's note:	Whether to use the READ COMMAND REJECT message to reject the READ COMMAND message when the read command not supported is FFS.

...

5.3.3.5	Write command procedure not accepted by AIoT device

...

Editor's note:	Whether to use the WRITE COMMAND REJECT message to reject the WRITE COMMAND message when the write command not supported is FFS.

...

According to the discussion in C1-256259, it is suggested to reuse READ COMMAND REJECT message and WRITE COMMAND REJECT message in the case command not supported.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if read command is not supported by the AIoT device, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

* * * Next Change * * * *

5.3.3.5	Write command procedure not accepted by AIoT device

Upon receiving a WRITE COMMAND message, if write command is not supported by the AIoT device, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

Upon receiving a WRITE COMMAND message, if the command type specific parameters (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

* * * End of Changes * * * *