3GPP TSG-CT WG1 Meeting #157	C1-256285

# Sophia Antipolis, France 13 – 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0851 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UE IP address IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE IP address IE is defined (in 24.554 CR0408) to accommodate both IPv4 and IPv6 addresses as 5G ProSe end UE IP address. However, it is not correct that the IE indicates to contain a link local IPv6 address if the address type is IPv6. According to clause 5.5.1.1 of TS 23.304, the UE obtains /64 IPv6 prefix via IPv6 Stateless Address auto configuration specified in RFC 4862. For link-local IPv6 address, it is formed locally based on RFC 4862 if the UE does not support the IPv6 IP address allocation mechanism, i.e. the IP Address Configuration indicates" address allocation not supported". In current specification, Link local IPv6 address IE is defined in clause 11.3.7 to represent UE link local IPv6 address. Based on above, the UE IP address IE needs to be fixed to contain 64/ IPv6 prefix other than link local IPv6 address. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the UE IP address information to contain a /64 IPv6 prefix in octet 4 to octet 11 and specify to set octet 12 to 19 all zero. Backward compatibility analysis: The change is backward compatible. The legacy UE will interpret the received "/64 IPv6 prefix + 64 bit all zero" address information as a normal IPv6 address which is not 128-bit link local IPv6 address. The legacy UE will ignore this information. The UE compliant with this CR will interpret the received link local IPv6 address as invalid information since the updated link local IPv6 address is delivered via link local IPv6 address IE if needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IPv6 prefix of the UE can not be delivered via relevant PC5 signalings. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.49 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

11.3.49	UE IP address

The purpose of the UE IP address information element is to indicate the IP address of the UE.

The UE IP address is a type 4 information element with minimum length of 7 octets and a maximum length of 19 octets.

The UE IP address information element is coded as shown in figure 11.3.49.1 and table 11.3.49.1.

| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE IP address IEI |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of UE IP address content |  |  |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare |  | AT |  | octet 3 |
| UE IP address information |  |  |  |  |  |  |  |  |  | octet 4 octet n |

Figure 11.3.49.1: UE IP address

Table 11.3.49.1: UE IP address

Address type (AT) (octet 3 bit 1): The AT field indicates the IP address type. Bits 1 0 IPv4 1 IPv6

UE IP address information (octet 4 to n) If the AT value indicates IPv4, the UE IP address information in octet 4 to octet 7 contains an IPv4 address. If the AT value indicates IPv6, the UE IP address information in octet 4 to octet 119 contains a /64 IPv6 prefix. Octet 12 to 19 are set to all zero.

* * * End of Changes * * * *