3GPP TSG-CT WG1 Meeting #157	C1-256312

Sophia Antipolis, France, 13-17 October 2025

Source:	Huawei, HiSilicon, LG Electronics

Title:	Pseudo-CR on Permanent disable indication to lower layers

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

Work has started in CT1 on AmbientIoT-CT and a new TS for AIoT NAS between AIoT device and AIOTF is to be developed.

2. Reason for Change

At permanent disable procedure, the command message is sent from AIOTF to AIoT device, which is then processed at NAS layer where it is interpreted as a request for permanent disable and subsequent action can be taken at the NAS layer. However, at permanent disable being accepted the information is also needed at the lower layer for appropriate handling and requirements at the AS/MAC layer. This cannot be determined at lower layer as both the command message and response message are transparent and their logical meaning not known. It is therefore proposed that the NAS layer informs lower layer of a permanent disable when the response message is provided.

3. Conclusions

A requirement to indicate permanent disable to lower layer is added when the NAS responds to a PERMANENT DISABLE COMMAND message.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0

* * * First Change * * *

#### 5.3.4.3	Permanent disable command procedure accepted by AIoT device

Upon receiving the PERMANENT DISABLE COMMAND message, the AIoT device shall indicate permanent disable to lower layers, send a PERMANENT DISABLE COMPLETE message to the AIOTF and disable the communication capability of the AIoT device.

* * * End of Changes * * *