3GPP TSG-CT WG1 Meeting #157	C1-256390

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.543 | CR | 0114 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor's note under clause A.4.4.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_ph2_App, SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification contains the following editor’s note under clause A.4.4.5.2, quote: Editor's note [WID: XRM_Ph2_App, CR#: 0095]: The definition of multimodalSealddFlowId is FFS. Last meeting (CT1#156), the need of having a multimodalSealddFlowId attribute for the response message to an SEALDD multi-modal transmission connection establishment request was questioned. This resulted in the editor note above being added to the specification. Having a dedicated multi‑modal flow identifier is useful when messages have to reference, correlate, or manage the entire group of SEALDD flows as a single logical object. However, it is unnecessary if each response always contains exactly one grouping of sealdd flows and there is no reference to that grouping from elsewhere (any other message). Analysis stage-2 requirements from TS 23.433, the a dedicated multi‑modal flow identifier is firstly optional. Furthermore, it is only indicated by the server to the client (direction server->client) and never from the client to the server. Also, for the the response message to an SEALDD multi-modal transmission connection establishment request only one grouping of sealdd flows is possible per message. However, the server can optionally reference a grouping of seal flows by using multi‑modal flow identifier in the SEALDD client policy configuration request (clause 9.10.3.7 of TS 23.433) and the Transmission quality measurement subscription request (clause 9.7.3.6 of TS 23.433). Hence, there is optinally reference to multi-modal grouping of sealdd flows from other messages, so if the multi‑modal flow identifier is provided in those messages, it could be beneficial to manage an entire group of sealdd flows as a single logical object using an identifier at the client side (UE). It is therefore proposed to support a dedicated multi‑modal flow identifier as per stage-2 description. Anyhow, note that an implementation can decide whether to use that provided multi‑modal flow identifier or use any other implementation method internally at the client side (UE) to reference a group of sealdd flows (e.g., memory pointers). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The specification is updated as per described in the reason for change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note remains in the specification after freezing the release 19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### A.4.4.5.2	CDDL document

;;; XRInformRequest

;;+ Represents a request for performing SDDM XR transmission connection inform.

XRInformRequest = {

requestorId: RequestorId

valUeIds: [* ValTargetUe]

status: Status

? valServiceId: string

}

;;; XRInformResponse

;;+ Represents the response of a request for performing SDDM XR transmission connection inform.

XRInformResponse = {

result: ResultOp

? cause: Cause

}

;;; XREstablishmentRequest

;;+ Represents a request for establishing an XR transmission connection.

XREstablishmentRequest = {

sealClientId: string

sealddFlowsInfos: SealddFlowsInfo

valTgtUe: ValTargetUe

serverId: ServerId

valServiceId: string

}

;;; XREstablishmentResponse

;;+ Represents a response for establishing an XR transmission connection.

XREstablishmentResponse = {

result: ResultOp

sealddMultimodalFlow SealddMultimodalFlowInfo

sealddmultimodalFlow: SealddMultiModalFlowInfo

? cause: Cause

? protocolDescriptorInfo: ProtocolDescriptorInfo

}

;;; SealddFlowsInfo

;;+ Represents the information of the SDDM flows.

SealddFlowsInfo = {

sealddFlowId: Uinteger

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

}

;;; SealddMultiModalFlowInfo

;;+ Represents the information of the multi-modal SDDM flow.

SealddMultiModalFlowInfo = {

sealddMultimodalFlowId: Uinterger

sealddFlowsInfos: SealddFlowsInfo

}

;;; ProtocolDescriptorInfo

;;+ Represents the information of the protocol of the VAL traffic.

ProtocolDescriptorInfo = {

? headerExtType: string

? headerExtId: Uinteger

? packetizationIndication: boolean

? payloadType: string

? payloadFormat: string

}

;;; RequestorId

;;+ Indicates requestor of an operation.

RequestorId = "SEALDDCLIENT" / "SEALDDSERVER"

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Status

;;+ Identifies the UE-to-UE direct communication status.

Status = "ESTABLISHED" / "RELEASED"

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

* * * End of Changes * * * *