3GPP TSG-CT WG1 Meeting #157	C1-256397

Sophia Antipolis, France, 13-17 October 2025	was C1-255301

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.572 | CR | 0115 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use routing information to map the user plane connection in the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2 agreed to support the multiple LCS-UPP connections per UE in Rel19. Since there can be more than one LCS secured user plane connections in the UE, when the UE receives an UPP-CM message, UE needs to identify the UPP-CM message is for which LCS secured user plane connection. Considering that when a LCS secured user plane connection is established, the routing information of the LMF which is for the UPP-CM message is always included in the Additional information IE of the UL NAS TRANSPORT message and DL NAS TRANSPORT message, and since only one LCS secured user plane connection is supported per LMF, the routing information can be used for UE to identify the LCS secured user plane connection. It is proposed to use the routing information for the UE to identify if the UPP-CM message is for which LCS secured user plane connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification about the UE uses the routing information included in the Additional information IE of the DL NAS TRANSPORT message to identify the established LCS secured user plane connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE has no clue about the UPP-CM message encapsulated in the DL NAS TRANSPORT message is sent to which LCS secured user plane connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1.3, 6.2.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.1.1.3	Network initiated user plane connection establishment procedure accepted by the UE

Upon receipt of a USER PLANE CONNECTION ESTABLISHMENT COMMAND message from the LMF, the UE shall stop timer T5011, if running, and timer T5014, if running, delete the stored LCS-UP binding ID, if any, and store the LCS-UP binding ID and the LMF LCS-UP address received in the USER PLANE CONNECTION ESTABLISHMENT COMMAND message.

If the USER PLANE CONNECTION ESTABLISHMENT COMMAND message can be accepted, the UE shall:

a)	establish a PDU session providing PDU connectivity service between the UE and the LMF, if not available, as described in clause 4.2.2;

b)	establish a TLS connection between the UE and the LMF, as described in clause 4.3 after the required PDU connectivity service between the UE and the LMF is available; and

c)	perform an LCS-UP connection binding procedure as described in clause 7.3.4 after the successful TLS connection establishment.

After the LCS-UP connection binding procedure is completed successfully as described in clause 7.3.4, the UE shall:

a)	create the USER PLANE CONNECTION ESTABLISHMENT COMPLETE message according to clause 10.3.2;

b)	send the USER PLANE CONNECTION ESTABLISHMENT COMPLETE message to the LMF;

c)	consider the LCS secured user plane connection between the UE and the LMF is established; and

d)	associate the routing information provided by the lower layer with the established LCS secured user plane connection if the multiple LCS secured user plane connections is supported.

* * * Next Change * * * *

##### 6.2.1.2.3	Network initiated user plane connection release procedure accepted by UE

Upon receipt of a USER PLANE CONNECTION RELEASE COMMAND message from the LMF, the UE shall:

a0)	identify the LCS secured user plane connection to be released by using the routing information provided by the lower layer if the multiple LCS secured user plane connections is supported;

a1)	stop the timer T5013 of the LCS secured user plane connection to be released if running;

a)	stop sending LCS-UPP messages over the LCS secured user plane connection to be released;

b)	terminate the TLS connection used for the LCS secured user plane connection to be released;

c)	consider the LCS secured user plane connection to be released between the UE and the LMF as released;

d)	create a USER PLANE CONNECTION RELEASE COMPLETE message according to clause 10.3.7; and

e)	send the USER PLANE CONNECTION RELEASE COMPLETE message to the LMF.

If the Back-off timer value IE is included in the USER PLANE CONNECTION RELEASE COMMAND message, the UE shall start the timer T5014 with the value provided in the Back-off timer value IE after sending the USER PLANE CONNECTION RELEASE COMPLETE message to the LMF. The UE shall not initiate the UE requested user plane connection establishment procedure as specified in subclause 6.2.2.1, until timer T5014 expires or timer T5014 is stopped.

Upon reception of a USER PLANE CONNECTION RELEASE COMPLETE message from the UE, the LMF shall stop the timer T5010 and shall consider the LCS secured user plane connection between the UE and the LMF as released.

* * * End of Changes * * * *