3GPP TSG-CT WG1 Meeting #157	C1-256459

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7055 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for reporting connection information for active non-3GPP devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, it has been agreed that the UE updates the SMF with connection information of one or more non-3GPP devices, including non-3GPP device identifier and the corresponding connection information, during the PDU session modification procedure. However, the current specification is unclear when the UE needs to update the network regarding the status of these devices. Since a connected non-3GPP device may become inactive and no longer require differentiated QoS, the network needs to be explicitly informed about only the "active" non-3GPP devices that currently need this service differentiation to ensure optimal resource management and correct policy enforcement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that the UE provides connection information only for “active” non-3GPP devices that require differentiated QoS in the PDU session modification procedure. This ensures the SMF has an up-to-date list of devices for which QoS policy applies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network would not have accurate, up-to-date information on which non-3GPP devices currently require QoS differentiation and would apply unnecessary QoS resources or policies to devices that have become inactive. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

## 4.30	QoS differentiation of traffic for non-3GPP device identifier

A 5GS can support QoS differentiation of traffic for non-3GPP devices connecting through the UE, as specified in 3GPP TS 23.501 [8]. The UE and the network may support QoS differentiation of traffic for non-3GPP devices.

The UE shall bind a non-3GPP device identifier to a non-3GPP device, for the traffic of the non-3GPP device that requires differentiated QoS (see 3GPP TS 23.501 [8] and 3GPP TS 23.316 [6D]). A non-3GPP device identifier shall be bound to no more than one non-3GPP device simultaneously. This binding enables the 5GS to distinguish between the traffic generated by different active non-3GPP devices connected through the same UE.

NOTE 1:	QoS differentiation of traffic for the non-3GPP device identifier also applies to the wireline access when a non-3GPP device is connecting to the 5G-RG.

NOTE 2:	To support QoS differentiation of traffic for active non-3GPP device(s) connecting through the UE, how the UE determines which PDU session to use for the non-3GPP device traffic is up to UE implementation.

The UE enables QoS differentiation of traffic from one or more non-3GPP devices associated with non-3GPP device identifiers by providing each non-3GPP device identifier and the corresponding connection information to the SMF in the PDU session modification procedure. The UE shall provide the non-3GPP device connection information for each active non-3GPP device depending on the PDU session type of the PDU session, as follows:

a)	for Ethernet PDU session type the UE shall include the non-3GPP device identifier and the Ethernet connection information;

b)	for IPv4 PDU session type the UE shall include the non-3GPP device identifier and the IPv4 connection information;

c)	for IPv6 PDU session type the UE shall include the non-3GPP device identifier and the IPv6 connection information; and

d)	for IPv4v6 PDU session type the UE shall include the non-3GPP device identifier and:

1)	the IPv4 connection information;

2)	the IPv6 connection information; or

3)	both 1) and 2).

If a previously reported active non-3GPP device becomes inactive or no longer requires differentiated QoS, the UE shall update the SMF by omitting the device identifier and its connection information from the subsequent PDU session modification procedure.

NOTE 3:	For an IPv4v6 PDU session type, it is up to UE implementation to decide whether to use IPv4 connection information, IPv6 connection information, or a combination of both. This decision can be based on the traffic associated with the non-3GPP device and the IP address allocation by the network for that PDU session.

***** Change ends *****