3GPP TSG-CT WG1 Meeting #157	C1-256467

Sophia Antipolis, France, 13-17 October 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Resolving editor’s note in protocol overview clause

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

Work has started in CT1 on AmbientIoT-CT and a new TS for AIoT NAS between AIoT device and AIOTF is to be developed.

2. Reason for Change

In the AIoT NAS protocol overview clause, there is an editor’s note related to security of the protocol. As the stage 2 security is specified in 33.369 the principles are sufficiently stable and it is proposed to remove the Editor’s note and add a sentence similar to the corresponding sentence in 24.501.

3. Conclusions

The security related editor’s note in the protocol overview clause can be deleted and a sentence referring to clause 4.2 can be added.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0

* * * First Change * * *

## 4.1	Overview

The AIoT NAS protocol described in the present document provide signalling between the AIoT device and the AIOTF, for 5GS. The AIoT NAS as part of the protocol stacks for support of AIoT services in 5GS is described in 3GPP TS 23.369 [2].

The main function of the AIoT NAS protocol is to support the AIoT procedures (see 3GPP TS 23.369[2]):

a)	inventory procedure; and

b)	command procedure.

For the support of the above functions, the procedures are supplied within procedures for AIoT NAS, in clause 5.

The AIoT NAS messages are transported between the AIoT device and the AIOTF via AIOT1 reference point, and this transport does not involve N1 reference point. The AIoT NAS protocol specified in the present document is unrelated to the 5GMM NAS and 5GSM NAS protocols specified in 3GPP TS 24.501 [4].

Principles for the handling of AIoT NAS security, including ciphering and integrity protection of AIoT NAS messages, are provided in subclause 4.2.

The AIoT NAS protocol for 5GS follows the protocol architecture model for layer 3 as described in 3GPP TS 24.007 [3].