3GPP TSG-CT WG1 Meeting #157	C1-256469

Sophia Antipolis, France, 13-17 October 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Inventory procedure updates

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

Work has started in CT1 on AmbientIoT-CT and a new TS for AIoT NAS between AIoT device and AIOTF is to be developed.

2. Reason for Change

Based on further SA3 progress for AIoT security as captured in 33.369 v1.0.0 specification of the Inventory procedure can be updated and aligned.

- Following 33.369, when privacy protection of AIoT device identifier is not used, the AIoT device includes its permanent identifier in the INVENTORY REPORT message:

When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2.

- Requirements for privacy protection have been added to 33.369 which impacts the NAS layer requirements to determine a match from the lower layer provided AIoT identification information in the case of individual paging. Depending on what type of Single device identity is included in the AIoT identification information (see for the AIoT device to compare to for a match, different cases apply. These requirements need to be captured for the case when “the AIoT identification information includes an AIoT device identifier” in the matching part of the inventory procedure.

Additionally, due to clashing pCRs in CT#156, a change was missed that corrected the order of requirements in the inventory procedure such that these are captured in the logical order which they occur. This is proposed to be corrected as well.

3. Conclusions

Updates of the inventory procedure are needed for:

- individual paging; and

- AIoT inclusion of permanent ID in INVENTORY REPORT.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0

* * * First Change * * *

### 5.2.3	Inventory procedure completion

The AIoT device determines to respond to the AIoT paging:

a)	based on the indication from lower layer as specified in 3GPP TS 38.391 [8]; or

b)	based on whether the AIoT identification information received from the lower layer is matched or not.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

The AIoT device shall include the AIoT device identity IE in the INVENTORY REPORT message.

Editor's note:	Details of the AIoT device identity IE is FFS and depends on SA3 progress.

The AIoT device determines whether the AIoT identification information is matched or not as follows:

Editor's note:	It is FFS how to determine the component of AIoT identification information for AIoT device, and this may need coordination with RAN2/3.

a)	if the AIoT identification information includes an AIoT device identifier:

1)	the AIoT device determines the AIoT identification information is matched if the AIoT device identifier included in the AIoT identification information is a:

A)	permanent device identifier and it matches the permanent identifier of the AIoT device;

B)	stored T-ID and it matches the stored T-ID of the AIoT device;

C)	concealed T-ID based on T-ID and it matches the concealed value calculated from the stored T-ID of the AIoT device; or

D)	concealed T-ID based on permanent identifier and it matches the concealed value calculated from the permanent identifier of the AIoT device;

2)	otherwise, the AIoT device determines the AIoT identification information is not matched; or

Editor's note:	Further alignments in details of AIoT device identifier matching depend on SA3 progress and updates in RAN2, SA2 and CT4 for AIoT identification information and definition of T-ID.

b)	if the AIoT identification information includes filtering information (see clause 31.3 of 3GPP TS 23.003 [5]):

1)	based on ID type field included in the filtering information, the AIoT device determines the AIoT identification information is matched if:

A)	the PLMN ID field is not included in the filtering information, or the PLMN ID field is included in the filtering information and:

i)	the PLMN ID field is included in the AIoT device permanent identifier; and

ii)	the PLMN ID field in the AIoT device permanent identifier is the same as the PLMN ID field within the filtering information;

B)	the NID field is not included in the filtering information, or the NID field is included in the filtering information and:

i)	the NID field is included in the AIoT device permanent identifier; and

ii)	the NID field in the AIoT device permanent identifier is the same as the NID field within the filtering information;

C)	the third party ID field is not included in the filtering information, or the third party ID field is included in the filtering information and:

i)	the third party ID field is included in the AIoT device permanent identifier; and

ii)	the third party ID field in the AIoT device permanent identifier is the same as the third party ID field within the filtering information; and

D)	the identification information filters field is not included in the filtering information, or the identification information filters field is included in the filtering information and for each identification information filter, the component value within the identification information filter is the same as the corresponding bitstring from the AIoT device permanent identifier based on offset field and length field; or

2)	otherwise, the AIoT device determines the AIoT identification information is not matched.

NOTE 1:	The AIoT identification information corresponds to paging ID, as defined in 3GPP TS 38.391 [8]).

Editor's note:	The reference to filtering information (e.g. IE name) will be updated according to CT4 and RAN2 progress.

Based on whether the AIoT identification information is matched:

a)	if the AIoT identification information is matched, the AIoT device shall indicate to the lower layer that the AIoT identification information is matched, and determine to respond to the AIoT paging; or

b)	otherwise, the AIoT device shall ignore the AIoT identification information, indicate to the lower layer that the filtering information is not matched, and determine not to respond to the AIoT paging.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT_root and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

If privacy protection of AIoT device identifier is not used, the AIoT device shall include the AIoT device permanent identifier in the AIoT device identity IE in the INVENTORY REPORT message.

Editor's note:	Details of the AIoT device identity IE is FFS and depends on SA3 progress.

Upon all the selected NG-RAN indicating the inventory procedure has been completed, the inventory procedure is considered as completed by the AIOTF as specified in 3GPP TS 23.369 [2].

NOTE:	How NG-RAN indicates the inventory procedure has been completed is defined in 3GPP TS 38.300 [7].

Editor's note:	The reference to TS 38.300 may be updated according to RAN2 progress.

Editor's note:	AIoT .device authentication of the network is FFS.

* * * Next Change * * *

##### 7.1.2.1	Message definition

The INVENTORY REPORT message is sent by the AIoT device to the AIOTF to report the AIoT device identity.

See table 7.1.2.1-1.

Message type:	INVENTORY REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | Security parameters | Security parameters 7.2.3 | M | V | n |
|  | RANDAIOT_d | RAND 7.2.10 | M | V | TBD |
|  | RESAIOT | Authentication response parameter 7.2.7 | M | V | TBD |
| XY | AIoT device identity | AIoT device identity 7.2.6 | O | TV | n |

* * * Next Change * * *

##### 7.1.2.1	AIoT device identity

The AIoT device shall include this IE when privacy protection of AIoT device identifier is not used.

* * * End of Changes * * *