3GPP TSG-CT WG1 Meeting #159	C1-260104

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0869 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Differentiating security materials used for PC5 direct discovery for multi-hop UE-to-UE relay (for both Model A discovery and Model B discovery) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2026-01-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The security procedures and requirements for multi-hop UE-to-UE relay procedures follow the same security procedures and requirements for the non multi-hop UE-to-UE relay procedures, as per security spec TS 33.503. That means, the 5G ProSe multi-hop end UEs can be provisioned with distinct sets of discovery security materials of potential 5G ProSe multi-hop UE-to-UE relay UE(s) in different PLMN(s), which also leads to the need to include the PLMN ID in the PC5 Discovery messages for multi-hop UE-to-UE relay and in cleartext, for a receiving UE to identify which discovery security materials to use. The corresponding stage-3 requirements need to be implemented which is done in this CR, for all multi-hop UE-to-UE relay discovery procedures, for both model A and model B discovery. Note: some of the needed changes were already done in a previous agreed CR (C1-256796) where the PLMN ID was included in the following two messages: "PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for IP data unit type" and "PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for IP data unit type". Hence this CR continues the remaining changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the PLMN ID in the PC5 Discovery messages for multi-hop UE-to-UE relay for both model A and model B discovery and in cleartext, and introducing any corresponding requirements at the sender UE and receiver UE. It is worth to note that, there are no changes needed for security provisioning procedures over the PC3a nor PC8 interfaces, since the existing procedures are reused as highlighted in the spec. Backward compatibility analysis: The CR is not backward compatible since it adds a field (PLMN ID) as a Mandatory IE in some PC5 discovery messages, hence a legacy receiver may fail in decoding the message. However the change is essential as explained in this CR. |  |  |  |  |  |  |  |  |
| G |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1- No possibility to differentiate security materials for the same RSC for different PLMNs. 2- Misalignment and inconsistency across the different procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8c.2.1.2.2.2.2, 8c.2.1.2.2.3.2, 8c.2.1.3.3.3, 8c.2.1.3.3.4.2, 10.2.1, 11.2.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

8c.2.1.2.2.2.2	Announcing UE procedure for multi-hop layer-3 UE-to-UE relay discovery initiation

The UE is authorised to perform the announcing UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type if:

a)	the UE is authorised to act as a 5G ProSe multi-hop layer-3 UE-to-UE relay UE in the PLMN or the subscribed SNPN as specified in clause 5.2.9, and

1)	the UE is served by NG-RAN and is authorised to perform 5G ProSe multi-hop layer-3 UE-to-UE relay discovery for IP data unit type in the PLMN or the subscribed SNPN as specified in clause 5; or

2)	the UE is authorised to perform 5G ProSe multi-hop layer-3 UE-to-UE relay discovery for IP data unit type when not served by NG-RAN as specified in clause 5 and intends to use the provisioned radio resources for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery for IP data unit type; and

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be announced as specified in clause 5.2.9 and the indicated security procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery for IP data unit type for the relay service code is supported by the UE as per clause 6.1.3.5.3.1 of 3GPP TS 33.503 [34]; and

2)	the user info ID for the 5G ProSe multi-hop layer-3 UE-to-UE relay discovery as specified in clause 5.2.9;

otherwise, the UE is not authorised to perform the announcing UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type.

Figure 8c.2.1.2.2.2.2.1 illustrates the interaction of the UEs in the announcing UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type.

Figure 8c.2.1.2.2.2.2.1: Announcing UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type

When a UE is triggered by the upper layers to announce the availability of an IP data unit type connectivity service provided by a multi-hop layer-3 UE-to-UE relay, if the UE is authorised to perform the announcing UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages for 5G ProSe multi-hop UE-to-UE relay discovery announcement for IP data unit type as specified in 3GPP TS 38.331 [13], shall perform a service request procedure or mobility registration procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time as specified in clause 11.2.5;

c)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type according to clause 10.2.1. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type, the UE

1)	shall set the announcer info parameter to the User info ID configured for the multi-hop layer-3 UE-to-UE relay discovery for IP data unit type, as specified in clause 5.2.9;

2)	shall set the relay service code parameter to the relay service code configured for the connectivity service to be announced, as specified in clause 5.2.9

3)	shall include the MIC field computed as described in 3GPP TS 33.503 [34], by using the UTC-based counter and the DUIK contained in the <UUR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used;

4)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

5)	shall set the ProSe direct discovery PC5 message type IE and ProSe direct discovery PC5 message content type extensions IE as specified in table 11.2.1.1 and table 11.2.1a.1; and

6)	shall include the PLMN ID set to its HPLMN ID and in cleartext;

NOTE 0:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 0A:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

d) 	shall apply the DUIK, DUSK or DUCK with the associated encrypted bitmask for the relay service code for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery for IP data unit type, along with the UTC-based counter, to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in clause 6.1.3.5.3.1 of 3GPP TS 33.503 [34];

e)	shall set the destination layer-2 ID to the default destination layer-2 ID as specified in clause 5.2.9 and self-assign a source layer-2 ID for sending the multi-hop layer-3 UE-to-UE relay discovery announcement for IP data unit type; and

NOTE 1:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2, is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2 and is different from any other self-assigned source layer-2 ID in use for a simultaneous 5G ProSe direct discovery procedure over PC5 with a different discovery model as specified in clause 6.2.14.2.2.2, clause 6.2.15.2.2.2, clause 8.2.1.3.1.2, clause 8a.2.1.3.2, clause 8b.2.1.3 and clause 8c.2.1.2.3.

f)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type to the lower layers for transmission over the PC5 interface with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery.

The UE shall maintain the association(s) between the self-assigned source layer-2 ID and relay service code once generating the PROSE PC5 DISCOVERY message. Each self-assigned source layer-2 ID can be associated with only one relay service code.

The UE shall ensure that it keeps on passing the same PROSE PC5 DISCOVERY message along with the same source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission, until the UE is triggered by the upper layers to stop announcing the availability of a connectivity service provided by the multi-hop layer-3 UE-to-UE relay, or until the UE stops being authorised to perform the announcing UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type. How this is achieved is left up to the UE implementation.

==============Next change==============

8c.2.1.2.2.3.2	Monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery initiation for IP data unit type

The UE is authorised to perform the monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery if:

a)	the following is true:

1)	the UE is not served by NG-RAN, is authorised to perform 5G ProSe multi-hop layer-3 UE-to-UE relay discovery using monitoring when the UE is not served by NG-RAN and is configured with the radio parameters to be used for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery for IP data unit type when not served by NG-RAN;

2)	the UE is served by NG-RAN and is authorised to perform 5G ProSe layer-3 UE-to-UE relay discovery monitoring in at least one PLMN or the subscribed SNPN; or

3)	the UE is:

i)	in 5GMM-IDLE mode, in limited service state as specified in 3GPP TS 23.122 [14] and the reason for the UE being in limited service state is one of the following:

A)	the UE is unable to find a suitable cell in the selected PLMN or the subscribed SNPN as specified in 3GPP TS 38.304 [15];

B)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #11 "PLMN not allowed" as specified in 3GPP TS 24.501 [11]; or

C)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #7 "5GS services not allowed" as specified in 3GPP TS 24.501 [11]; and

ii)	authorised to perform 5G ProSe multi-hop layer-3 UE-to-UE relay discovery using monitoring when the UE is not served by NG-RAN, and:

A)	configured with the radio parameters to be used for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery when not served by NG-RAN; and

b)	the UE is configured with the relay service code parameter identifying the connectivity service to be monitored, as specified in clause 5.2.9;

otherwise, the UE is not authorised to perform the monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type.

Figure 8c.2.1.2.2.3.2.1 illustrates the interaction of the UEs in the monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type.

Figure 8c.2.1.2.2.3.2.1: Monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type

When the UE, acting as a 5G ProSe multi-hop layer-3 UE-to-UE relay or as a 5G ProSe multi-hop end UE, is triggered by the upper layers to monitor the proximity of a connectivity service provided by another 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type, and if the UE is authorised to perform the monitoring UE procedure for multi-hop UE-to-UE relay discovery for IP data unit type, then the UE shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages with the default destination layer-2 ID as specified in clause 5.2.9.

Upon the reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to discover, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to discover, the UE shall obtain a valid UTC time from the lower layers and generate a UTC-based counter corresponding to this UTC time. The UE shall use the associated DUSK, if received from the 5G DDNMF (if security procedure over control plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used) or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used) and the UTC-based counter associated with the PROSE PC5 DISCOVERY message to unscramble the PROSE PC5 DISCOVERY message as described in clause 6.1.3.5.3.1 of 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF (if security procedure over control plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used) or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used), the UE shall use the DUCK and the UTC-based counter associated with the PROSE PC5 DISCOVERY message to decrypt the configured message-specific confidentiality-protected portion, as described in clause 6.1.3.5.3.1 of 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF (if security procedure over control plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used) or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used), the UE shall use the DUIK and the UTC-based counter associated with the PROSE PC5 DISCOVERY message to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for UE-to-UE relay discovery announcement.

Then if the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type is the same as the relay service code parameter configured as specified in clause 5 for the connectivity service being monitored, the UE shall consider that the connectivity service the UE seeks to monitor has been discovered.

NOTE: 	This procedure may be used by a 5G ProSe multi-hop end UE or a 5G ProSe multi-hop layer-3 UE-to-UE relay to measure the signal strength of the PROSE PC5 DISCOVERY message from a 5G ProSe multi-hop layer-3 UE-to-UE relay. The measurements could be considered for 5G ProSe multi-hop layer-3 UE-to-UE relay selection/reselection.

==============Next change==============

###### 8c.2.1.3.3.3	Discoverer relay UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery

For Ethernet and unstructured data unit type, the procedure specified in clause 8c.2.1.2.3.3 applies with following differences and clarification:

a)	if the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type is the same as the relay service code parameter configured as specified in clause 5.2.9 for the connectivity service, the discoverer relay UE determines to send the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type upon receipt of a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type from the source 5G ProSe multi-hop layer-3 end UE or other 5G ProSe multi-hop layer-3 UE-to-UE relay UE.

For the received List of multi-hop UE-to-UE relay paths IE, if:

1)	the value of the hop count field is less than the hop limit as described in clause 6.3.2.6.3.2 of 3GPP TS 23.304 [2]; and

2)	the user info ID of the discoverer relay UE is not listed in the "Path of relays" field, if any,

the UE shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type and set the PROSE PC5 DISCOVERY message as specified in clause 8c.2.1.3.3.2.2 with following differences:

i)	shall update received List of multi-hop UE-to-UE relay paths IE as follows:

-	the Hop count field is increased by 1;

-	add the user info ID of the discoverer relay UE to the "Path of relays" as described in table 11.2.24.2;

-	update the value of the length of the path of relays field accordingly;

-	if the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type is from the source 5G ProSe multi-hop layer-3 end UE, the UE shall set the SL2I bit to "Source layer-2 ID of the discoverer end UE is present" and include the source layer-2 ID of the the source 5G ProSe multi-hop layer-3 end UE in the "Source layer-2 ID of the discoverer end UE" field; and

ii)	shall include the updated List of multi-hop UE-to-UE relay paths IE in the PROSE PC5 DISCOVERY message.

otherwise, UE shall drop the received PROSE PC5 DISCOVERY message;

b)	if the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type is the same as the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type, and the user info ID of the discoverer relay UE is listed in the "Path of relays" field, the UE shall forward PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type as specified in 6.3.2.6.3.2 of 3GPP TS 23.304 [2].

The UE shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages for multi-hop layer-3 UE-to-UE relay discovery response from the discoveree end UE or other discoverer relay UE.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type, for the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF, the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF, the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type.

If the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type is the same as the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit,

then the UE:

a)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

b)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type. In the PROSE PC5 DISCOVERY message, the UE:

1)	shall set the List of multi-hop UE-to-UE relay paths IE to the List of multi-hop UE-to-UE relay paths as included in the received PROSE PC5 DISCOVERY message for UE-to-UE relay discovery response;

2)	shall set the Discoveree info to the configured user info ID for the 5G ProSe multi-hop layer-3 UE-to-UE relay discovery, as specified in clause 5.2.9;

3)	shall set the Discoverer info to the Discoverer info as included in the received PROSE PC5 DISCOVERY message for UE-to-UE relay discovery response;

4)	shall set the relay service code parameter to the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type;

5)	shall include the MIC field computed as described in 3GPP TS 33.503 [34];

6)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

7)	shall set the ProSe direct discovery PC5 message type parameter and the ProSe direct discovery PC5 message content type extensions as specified in table 10.2.1.24; and

8)	shall include the PLMN ID set to its HPLMN ID and in cleartext;

NOTE 1:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 2:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

c)	shall apply the DUIK, DUSK, or DUCK with the associated encrypted bitmask for the relay service code for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more of the above parameters, as specified in 3GPP TS 33.503 [34]; and

d)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

==============Next change==============

8c.2.1.3.3.4.2	Discoveree end UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery initiation

The UE is authorised to perform the discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery if:

a)	the UE is authorised to act as a 5G ProSe multi-hop layer-3 end UE in the PLMN or the subscribed SNPN indicated by the serving cell, and

1)	the UE is served by NG-RAN; or

2)	the UE is not served by NG-RAN and intends to use the provisioned radio resources for multi-hop layer-3 5G ProSe UE-to-UE relay discovery; and

b)	the UE is configured with:

1)	the relay service code parameter identifying the non-IP connectivity service to be responded to as specified in clause 5.2.9 and the indicated security procedure for the relay service code is supported by the UE;

otherwise, the UE is not authorised to perform the discoveree end UE procedure for multi-hop UE-to-UE relay discovery.

Figure 8c.2.1.3.3.4.2.1 illustrates the interaction of the UEs in the discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery.

Figure 8c.2.1.3.3.4.2.1: Discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery

When the UE is triggered by the upper layers to start responding to solicitation on proximity of a connectivity service provided by a 5G ProSe multi-hop layer-3 UE-to-UE relay UE and if the UE is authorised to perform the discoveree end UE procedure for multi-hop layer-3 UE-to-UE relay discovery, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11]; and

b)	shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop layer-3 UE-to-UE relay discovery solicitation, if:

a)	the received PLMN ID in the message is not associated with the relay service code parameter configured as specified in clause 5.2.9 for the connectivity service, the UE shall ignore the message. If the received PLMN ID in the message is associated with the relay service code parameter configured as specified in clause 5.2.9 for the connectivity service and the relay service code parameter of the received message is the same as the relay service code parameter configured as specified in clause 5.2.9 for the connectivity service; and

b)	the Target discoveree info IE containing the User info ID of the discoveree end UE in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type is the same as the User info ID of the UE provided by upper layers;

the UE:

a)	shall obtain a valid UTC time for the discovery transmission from the lower layers, generate the UTC-based counter corresponding to this UTC time. Then the UE shall use the generated the UTC-based counter to recover the UTC-based counter associated with the PROSE PC5 DISCOVERY message and the UTC-based counter associated with the list of protected user infos in the received List of multi-hop UE-to-UE relay paths IE;

b)	shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF and the UTC-based counter associated with the PROSE PC5 DISCOVERY message to unscramble the PROSE PC5 DISCOVERY message as described in clause 6.1.3.5.3.2 of 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF, the UE shall use the DUCK and the UTC-based counter associated with the PROSE PC5 DISCOVERY message to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF, the UE shall use the DUIK and the UTC-based counter associated with the PROSE PC5 DISCOVERY message to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for UE-to-UE relay discovery solicitation;

c)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type in which the UE:

1)	shall include the List of multi-hop UE-to-UE relay paths IE in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type;

2)	shall set the relay service code parameter to the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type;

3)	shall include the MIC field computed as described in 3GPP TS 33.503 [34];

4)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

5)	shall set the ProSe direct discovery PC5 message type IE and ProSe direct discovery PC5 message content type extensions IE as specified in table 11.2.1.1 and table 11.2.1a.1;

6)	shall set the Discoveree info to the configured user info ID for the 5G ProSe multi-hop layer-3 UE-to-UE relay discovery, as specified in clause 5.2.9;

7)	shall set the Discoverer info to the Discoverer info as included in the received PROSE PC5 DISCOVERY message for UE-to-UE relay discovery response; and

8)	shall include the PLMN ID set to the PLMN ID for which the used security parameters are associated and in cleartext;

NOTE:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

d)	shall set the destination layer-2 ID to the source layer-2 ID from the 5G ProSe multi-hop layer-3 UE-to-UE relay UE used in the transportation of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type and self-assign a source layer-2 ID for sending the multi-hop UE-to-UE relay discovery response message for Ethernet and unstructured data unit type; and

NOTE:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2 and is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2.

e)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

==============Next change==============

### 10.2.1	Message definition

This message is sent by the UE over the PC5 interface for open 5G ProSe direct discovery and restricted 5G ProSe direct discovery. See table 10.2.1.1, table 10.2.1.2, table 10.2.1.3, table 10.2.1.4, table 10.2.1.5, table 10.2.1.6, table 10.2.1.7, table 10.2.1.8, table 10.2.1.9, table 10.2.1.10, table 10.2.1.11, table 10.2.1.12, table 10.2.1.13, table 10.2.1.14, table 10.2.1.15, table 10.2.1.16, table 10.2.1.17, table 10.2.1.18 table 10.2.1.19, table 10.2.1.20, table 10.2.1.21, table 10.2.1.22, table 10.2.1.23 and table 10.2.1.24.

Message type:	PROSE PC5 DISCOVERY

Significance:	dual

Direction:	UE to peer UE

Table 10.2.1.1: PROSE PC5 DISCOVERY message content for open 5G ProSe direct discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe application code | ProSe application code 11.2.2 | M | V | 23 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Open discovery" and the content type is set to "Announcement". |  |  |  |  |  |

Table 10.2.1.2: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe restricted code | ProSe restricted code 11.2.3 | M | V | 23 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "Announcement". |  |  |  |  |  |

Table 10.2.1.3: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe query code | ProSe restricted code 11.2.3 | M | V | 23 |
| 2A | Discoveree user info | Application layer ID 11.2.15 | O | TLV | 3-257 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "Solicitation". |  |  |  |  |  |

Table 10.2.1.4: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe response code | ProSe restricted code 11.2.3 | M | V | 23 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "response". |  |  |  |  |  |

Table 10.2.1.5: PROSE PC5 DISCOVERY message for group member discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-257 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery announcement/group member discovery response" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.6: PROSE PC5 DISCOVERY message for group member discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-256 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
| 28 | Target user info | User info ID 11.2.7 | O | TV | 7 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.7: PROSE PC5 DISCOVERY message for group member discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-256 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery announcement/group member discovery response" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.8: PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 2) | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery announcement/UE-to-network relay discovery response" and the discovery model is set to "Model A". NOTE 2: If the announcing UE works as a 5G ProSe Layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.9: PROSE PC5 DISCOVERY message for UE-to-network relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 29 | Target discoveree info | User info ID 11.2.7 | O | TV | 7 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.10: PROSE PC5 DISCOVERY message for UE-to-network relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 2) | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery announcement/UE-to-network relay discovery response" and the discovery model is set to "Model B". NOTE 2: If the discoveree UE works as a 5G ProSe Layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.11: PROSE PC5 DISCOVERY message for relay discovery additional information

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 51 | Relay TAI | TAI 11.2.10 | O | TV | 4 |
| 10 | NID | NID 11.2.23 | O | TLV | 8 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Relay discovery additional information" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.12: PROSE PC5 DISCOVERY message for UE-to-UE relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 15-8323 |
| 2D | Status indicator | Status indicator 11.2.9 | O | TV | 2 |
| 2C | UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-UE relay discovery announcement" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.13: PROSE PC5 DISCOVERY message for UE-to-UE relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 12-520 |
| 2C | UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| 2D | Status indicator | Status indicator 11.2.9 | O | TV | 2 |
| 8- | Announce prohibited indication | Announce prohibited indication 11.2.19 | O | TV | 1 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-UE relay discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.14: PROSE PC5 DISCOVERY message for UE-to-UE relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 12-520 |
| 2C | UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| 2D | Status indicator | Status indicator 11.2.9 | O | TV | 2 |
| 8- | Announce prohibited indication | Announce prohibited indication 11.2.19 | O | TV | 1 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-UE relay discovery response" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.15: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 3) | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
|  | Hop count | Hop count 11.2.21 | M | V | 1 |
| 10 | Hop limit | Hop limit 11.2.22 | O | TV | 1 |
| 11 | Root relay info | User info ID 11.2.7 | O | TV | 7 |
| 12 | Accumulated QoS | PC5 QoS flow descriptions 11.2.27 | O | TLV | 6-65538 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-network relay discovery announcement”. NOTE 3: If the announcing UE works as a 5G ProSe multi-hop layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.16: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Hop count | Hop count 11.2.21 | M | V | 1 |
| 8- | Hop limit | Hop limit 11.2.22 | O | TV | 1 |
| 29 | Root relay info | User info ID 11.2.7 | O | TV | 7 |
| 52 | NCGI of multi-hop UE-to-network relay | NCGI 11.2.12 | O | TV | 9 |
| 10 | NCGI info set of intermediate UE-to-network relay | NCGI info set 11.2.25 | O | TLV | 16 |
| 51 | Relay TAI | TAI 11.2.10 | O | TV | 4 |
| 53 | NID | NID 11.2.23 | O | TLV | 8 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-network relay discovery additional information”. |  |  |  |  |  |

Table 10.2.1.17: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 29 | Target discoveree info | User info ID 11.2.7 | O | TV | 7 |
| 20 | Multi-hop U2N path info | List of user info IDs and layer-2 IDs 11.3.48 | O | TLV | 13-257 |
| 21 | Intermediate relays | List of user info ID 11.2.26 | O | TLV | 8-257 |
| 22 | Hop count | Hop count 11.2.21 | O | TV | 1 |
| 23 | Hop limit | Hop limit 11.2.22 | O | TV | 1 |
| 24 | Accumulated QoS | PC5 QoS flow descriptions 11.2.27 | O | TLV | 6-65538 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to "Multi-hop UE-to-network relay discovery solicitation". |  |  |  |  |  |

Table 10.2.1.18: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
|  | Multi-hop U2N path info | List of user info IDs and layer-2 IDs 11.3.48 | M | LV | 12-256 |
| 10 | Hop count | Hop count 11.2.21 | O | TV | 1 |
| 12 | Accumulated QoS | PC5 QoS flow descriptions 11.2.27 | O | TLV | 6-65538 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to "Multi-hop UE-to-network relay discovery response". |  |  |  |  |  |

Table 10.2.1.19: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery announcement for IP data unit type”. |  |  |  |  |  |

Table 10.2.1.20: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for IP data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 11-519 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 29 | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery solicitation for IP data unit type”. |  |  |  |  |  |

Table 10.2.1.21: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for IP data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | M | V | 6 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 11-519 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery response for IP data unit type”. |  |  |  |  |  |

Table 10.2.1.22: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for Ethernet and unstructured data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | List of multi-hop UE-to-UE relay paths | List of multi-hop UE-to-UE relay paths 11.2.24 | M | LV-E | 2-65537 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery announcement for Ethernet and unstructured unit type”. |  |  |  |  |  |

Table 10.2.1.23: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 10 | List of multi-hop UE-to-UE relay paths | List of multi-hop UE-to-UE relay paths 11.2.24 | O | TLV-E | 3-65538 |
| 29 | Target discoveree info | User info ID 11.2.7 | O | TV | 7 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type”. |  |  |  |  |  |

Table 10.2.1.24: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | List of multi-hop UE-to-UE relay paths | List of multi-hop UE-to-UE relay paths 11.2.24 | M | LV-E | 2-65537 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type”. |  |  |  |  |  |

==============Next change==============

### 11.2.20	PLMN ID

The PLMN ID information element is used to indicate the PLMN ID of the 5G ProSe UE-to-network relay UE, the 5G ProSe multi-hop UE-to-network relay UE, the 5G ProSe multi-hop remote UE, the 5G ProSe intermediate UE-to-network relay UE , the 5G ProSe UE-to-UE relay UE, the 5G ProSe end UE, the 5G ProSe multi-hop UE-to-UE relay UE, or the 5G ProSe multi-hop end UE for which the relay service code is associated, and is coded as the PLMN identity information element specified in clause 9.11.3.85 of 3GPP TS 24.501 [11] starting from octet 3 to octet 5.

==============End of change==============