3GPP TSG-CT WG1 Meeting #159	C1-260198

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.581 | CR | 0132 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solve the problem in 10.1.14 of ETSI plugtest R19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc18 |  |  |  |  |  | Date: |  |  | 2026-01-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | ETSI Plug test indicates the following problem: 10.1.14 Ambiguities in SSRCs to be used in different MCVideo transmission control messages 3GPP TS 24.581 Section 4.2.2 states that an MCVideo call uses 3 different and unique SSRC values. One for audio, one for video and another one for transmission control. It also states that the transmission participant is uniquely identified in the MCVideo call using the 'SSRC value'. This statement doesn't clarify which of the 3 SSRCs it is referring to. Even assuming this statement means the “SSRC used for Transmission Control” further issues may appear. If the SSRC values for each 'm=' stream in the MCVideo call are different then it is unclear how the 'Active SSRC List' in section 6.3.7 is expected to function. This stores the SSRC of the participant who has been granted to send to. Section 6.3.7.4.3 states that the SSRC value of RTP packets to forward are compared to this list and forwarded if found. It is unclear how SSRC values should be added to this list to make it work. Should the SSRC values for both audio and video of the originator be added and how would they be learnt by the terminator when the Transmission Notification message only contains a single SSRC value is unclear. Plugtests participants propose (apart from internal clarification in Plugtests Test Case Document ETSI TS 103.564) for 3GPP CT1 to add an appendix to TS 24.581 with example use case for the 'Active SSRC List' as defined in section 6.3.7. Furthermore, they kindly suggest that, when SSRC values are referred to in 3GPP TS 24.581, they should be qualified with exactly which of the SSRCs is being referred to (i.e. originating audio SSRC, terminating transmission control SSRC, etc). Reference to: https://portal.etsi.org/Portals/0/TBpages/CTI/Docs/9th_ETSI_MCX_Plugtests_Report.pdf This CR is trying to solve this problem. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify which SSRC is stored in the active SSRC list. Backward compatibility analysis: This CR is to backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.3.7.1	General

The reception control interface towards the MCVideo client in the transmission control server shall behave according to the state diagram and state transitions specified in this clause.

Figure 6.3.7.1-1 shows the states and state transitions for an associated transmission participant in the transmission control server.

Figure 6.3.7.1-1: Transmission control server state transition diagram for basic reception control operation towards the transmission participant

The reception control interface towards the MCVideo client in the transmission control server shall create one instance of the 'basic reception control operations' state machine towards the MCVideo client for every transmission participant served by the transmission control server as follows:

1.	For pre-arranged group call and adhoc group call in case of an originating MCVideo call, the 'basic reception control operation towards the transmission participant' state machine shall be created when the MCVideo server sends the SIP 200 (OK) response towards the originating MCVideo client.

2.	For pre-arranged group call and adhoc group call in case of a terminating MCVideo call, the 'basic reception control operation towards the transmission participant' state machine shall be created when the transmission control server receives the SIP 200 (OK) response.

3.	For chat group call the 'basic reception control operation state machine towards the transmission participant' shall be created when the MCVideo server sends the SIP 200 (OK) response to the received initial SIP INVITE request.

The transmission participant associated to the 'basic reception control operation towards the transmission participant' state machine is here referred to as the "associated transmission participant".

The external inputs to the state machine are:

1.	directives coming from the reception control arbitration logic;

2.	transmission control messages sent by the transmission participants;

3.	media; and

4.	in certain cases, SIP messages used for call handling.

If transmission control messages or RTP media packets arrives in a state where there is no procedure specified in the following clauses, the transmission control interface towards the MCVideo client in the transmission control server:

1.	shall discard the transmission control message;

2.	shall request the network media interface in the MCVideo server to discard any received RTP media packet; and

3.	shall remain in the current state.

State details are explained in the following clauses.

In MCVideo calls, each transmitting participant has three unique SSRC values: Audio SSRC, Video SSRC, and transmission control SSRC (see clause 4.2.2).

The Active SSRC List maintained by a receiving participant shall store only the transmission control SSRC of participants who have been granted permission to transmit. Audio and Video SSRCs are implicitly associated with the corresponding Transmission Control SSRC through session signaling.

Upon reception of RTP packets, the receiver shall determine the transmitting participant associated with each Audio or Video SSRC and forward the packet only if the participant’s Transmission Control SSRC is present in the Active SSRC List.

==============End of change==============