3GPP TSG-CT WG1 Meeting #159	C1-260246

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.369 | CR | 0024 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of T-ID due to lower layer failue |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The AIoT device can receives new T-ID in READ command or WRITE command and send a COMPLETE message to the AIOTF. When the AIOTF receives COMPLETE message, then network shall using new T-ID for upcoming procedures e.g., AIoT paging procedure. In unsuccessful case: If AIOTF does not receive COMPLETE message for a command including the new T-ID, then the AIOTF is not aware whether the command message is really received by the AIoT device or not. Furthermore, the AIOTF cannot know which T-ID is used by the AIoT device and which T-ID should be used for upcoming AIoT pagings. If AIOTF uses new T-ID while the UE is still using the old T-ID, or vise versa, then UE does not respond to AIoT paging. Backward compatibility analysis: The change is backward compatible since not changing the signaling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When lower layer failure occurs, the AIOTF shall consider both old T-ID and new T-ID as valid until the old T-ID can be considered as invalid. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AIOTF cannot reach the AIOT device by using inappropriate T-ID in AIOT paging procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.7, 5.3.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 5.3.2.7	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of a command failure received as specified by 3GPP TS 38.413 [10], the AIOTF shall abort the read command procedure and consider both the old T-ID and the new T-ID as valid until the old T-ID can be considered as invalid by the AIOTF.

b)	Expiry of timer T1

The AIOTF shall abort the read command procedure, consider the read command procedure is unsuccessful and consider both the old T-ID and the new T-ID as valid until the old T-ID can be considered as invalid by the AIOTF.

==============Next change==============

#### 5.3.3.7	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of a command failure received as specified by 3GPP TS 38.413 [10], the AIOTF shall abort the write command procedure and consider both the old T-ID and the new T-ID as valid until the old T-ID can be considered as invalid by the AIOTF.

b)	Expiry of timer T2

The AIOTF shall abort the write command procedure, consider the write command procedure is unsuccessful and consider both the old T-ID and the new T-ID as valid until the old T-ID can be considered as invalid by the AIOTF.

==============End of change==============