3GPP TSG-CT WG1 Meeting #159	C1-260250

# Goa, India , 09-13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7125 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update requested WUS IE inclusion conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc19 |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the following statement quoted from clause 5.5.1.3.2 of TS 24.501, it can be concluded that the requested WUS assistance information IE shall not be included if the UE is performing a registration procedure for periodic registration update". " The UE shall set the WUSA bit to "WUS assistance information reception supported" in the 5GMM capability IE for all cases except case b, if the UE supports WUS assistance information. The UE may include its UE paging probability information in the Requested WUS assistance information IE if the UE has set the WUSA bit to "WUS assistance information reception supported" in the 5GMM capability IE and does not have an active emergency PDU session. " However, the condition description for inclusion of requested WUS assistance information IE in the registration request message as shown below is incomplete. The condition "the UE is not performing a registration procedure for periodic registration update" should be reflected. " 8.2.6.30 Requested WUS assistance information The UE may include this IE if: - the UE supports WUS assistance information; - the UE is not performing initial registration for emergency services; and - the UE does not have an active emergency PDU session. " |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add "the UE is not performing a registration procedure for periodic registration update" in the list of condition bullets for inclusion of requested WUS assistance information IE in the registration request message. Backward compatibility analysis: The change is backward compatible since the missing condition description aligns the statement as specified in clause 5.5.1.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete description on the condition of inclusion of requested WUS assistance information IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 8.2.6.30	Requested WUS assistance information

The UE may include this IE if:

-	the UE supports WUS assistance information;

-	the UE is not performing initial registration for emergency services;

-	the UE does not have an active emergency PDU session; and

-	the UE is not performing a registration procedure for periodic registration update.

==============End of change==============