3GPP TSG-CT1 Meeting #159	C1-260353

Goa, India , 9-13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.571 | CR | 0110 | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Option C: Usage of Extended Facility IE in all LCS-CP procedures. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicion |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For more details and background on the issue please see discussion paper (C1-260380) The Facility information element is a type 4 element with a length field of 1 byte. The maximum size it can carry is 255 bytes. This has been the size of the IE from rel-8. For 2G, 3G and 4G this size seemed enough. But for 5G, the component can carry a much higher size and for that it is required to extend the Facility information element. As an example, According to ASN.1 encoding in 24080-i50-ASN1 These 2 fields themselves can go over 255 bytes. So the procedures that are using these parameters ( eg: LCS-PeriodicTriggeredInvokeArg , lcs-MSCancelDeferredLocation ) will have to be extended to carry messages longer than 255 bytes. Similarly, there are other procedures, (eg: SL mobile terminating location, MO-LR) where the message can be longer than 255 bytes. So the procedures are updated with the possibility to use the new Ext Facility IE when there is a need to send a message larger than 255 bytes. When the length of the content is less than 255 bytes the old Facility IE can be used by the sender. So changes are needed for procedures to clarify that the Extended Facility IE needs to be used for larger message content. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new extended Facility IE for larger messages in the LCS procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect messages and wrong coding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3.2, 5.2.1.4.2, 5.2.1.8.2, 5.2.2.1.2, 5.2.2.2.2, 5.2.2.4.2, 5.2.2.11.2, 5.4, 5.4.1, 5.4.1.1, 5.4.1.2, 5.4.2, 5.4.2.1, 5.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 24.080.. CR .0129. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.1.3.2	Normal operation

The LMF sends a REGISTER message to the UE containing lcs-PeriodicTriggeredInvoke invoke component as defined in 3GPP TS 24.080 [5]. The REGISTER message is transported to the UE via the serving AMF as described in figure 5.2.1.3.1.1.

If the UE can support the periodic or triggered location, the UE returns a RELEASE COMPLETE message to the LMF containing an lcs-PeriodicTriggeredInvoke return result component. The RELEASE COMPLETE message is transported to the LMF via the serving AMF as described in figure 5.2.1.3.1.1.

For supporting of the location events reporting over user plane connection, as described in clause 6.16.1 of 3GPP TS 23.273 [2], the LMF provides the UE with the following:

a)	the endpoint address for the location reporting over user plane connection;

b)	the security information for the location reporting over user plane connection;

c)	the cumulative event report timer, if available; or

d)	the maximum number of events reporting counter, if available.

If the UE receives the cumulative event report timer or the maximum number of events reporting counter from the LMF, the UE shall perform the UE initiated Event Reporting procedure as defined in subclause 5.2.2.4 with including the indication for the cumulative event report.

If the UE is unable to process or support the request received from the network, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component or reject component. Error values are specified in 3GPP TS 24.080 [5].

During the supplementary services periodic or triggered location operation, the network shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the UE. If this timer expires the network shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message.

Figure 5.2.1.3.2.1 illustrates the signalling for normal operation between the UE and the network.

When the Facility element in the REGISTER message carries a content length of 255 bytes or less, the signalling is as follows:

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility (Invoke = lcs-PeriodicTriggeredInvoke (referenceNumber, qoS, mapped-Qos, reportingPLMNList, periodicLocation, areaEventReporting, eventReportAllowedArea, motionEventReporting, referenceNumberExt, h-gmlc-callBackUri, supportedGADShapes, deferredRoutingIdentifier, reportingAccessTypes, multiplePositioningProtocolPDUs, controlPlane-CIoT-5GS-Optimisation, scheduledLocTime, userPlaneReportAFAddr, cumulativeReportCriteria, h-gmlc-address, reportingInd))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility (Return result =lcs-PeriodicTriggeredInvoke)

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

When the Facility element in the REGISTER message carries content longer than 255 bytes and if the UE and the NW supports the usage of ExtendedFacility information element as defined in 3GPP TS 24.080 [5], the Facility IE shall be replaced with an ExtendedFacility information element, the signalling is as follows:

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Extended Facility (Invoke = lcs-PeriodicTriggeredInvoke (referenceNumber, qoS, mapped-Qos, reportingPLMNList, periodicLocation, areaEventReporting, eventReportAllowedArea, motionEventReporting, referenceNumberExt, h-gmlc-callBackUri, supportedGADShapes, deferredRoutingIdentifier, reportingAccessTypes, multiplePositioningProtocolPDUs, controlPlane-CIoT-5GS-Optimisation, scheduledLocTime, userPlaneReportAFAddr, cumulativeReportCriteria, h-gmlc-address, reportingInd))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Extended Facility (Return result =lcs-PeriodicTriggeredInvoke)

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Extended Facility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Extended Facility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Figure 5.2.1.3.2.1: Periodic or Triggered Location Invocation

NOTE 1:	The mappedLcs-QoS is obtained by the UE in 5GS and used for location service continuity from 5GS to EPS for periodic or triggered location event reporting.

NOTE 2:	If h-gmlc-address as defined in 3GPP TS 24.080 [5] is received in the lcs-PeriodicTriggeredInvoke invoke component, it is used for a EPC-(H)GLMC during the procedure of location service continuity between EPS and 5GS.

NOTE 3:		If reportingIndication as defined in 3GPP TS 24.080 [5] is received in the lcs-PeriodicTriggeredInvoke invoke component, the reportingIndication is used for determining whether the Event Reporting procedure is triggered when the UE is inside or outside the eventReportAllowedArea.

* * * Next Change * * * *

##### 5.2.1.4.2	Normal operation

The AMF invokes a cancel deferred location procedure by sending a REGISTER message containing an lcs-CancelDeferredLocation invoke component to the UE as defined in 3GPP TS 24.080 [5].

The UE shall terminate the ongoing periodic or triggered location if this can be identified from the information in the lcs-CancelDeferredLocation invoke component.

The UE shall then return a RELEASE COMPLETE message containing an lcs-CancelDeferredLocation return result component (see figure 5.2.1.4.2.1).

If the UE is unable to process the request received from the network or cannot identify the ongoing periodic or triggered location to be terminated, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the supplementary services cancel deferred location operation, the network shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the UE. If this timer expires the network shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message.

When the Facility element in the REGISTER message carries a content length of 255 bytes or less, the signalling is as follows:

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility (Invoke = lcs-CancelDeferredLocation (referenceNumberExt, h-gmlc-callBackUri))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility (Return result = lcs-CancelDeferredLocation)

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

When the Facility element in the REGISTER message carries content longer than 255 bytes and if the UE and the NW supports the usage of ExtendedFacility information element as defined in 3GPP TS 24.080 [5], the Facility IE shall be replaced with an ExtendedFacility information element, the signalling is as follows:

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Extended Facility (Invoke = lcs-CancelDeferredLocation (referenceNumberExt, h-gmlc-callBackUri))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Extended Facility (Return result = lcs-CancelDeferredLocation)

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Extended Facility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Extended Facility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Figure 5.2.1.4.2.1: Cancel Deferred Location

* * * Next Change * * * *

##### 5.2.1.8.2	Normal operation

In order to initate the sidelink mobile terminating location request procedure, the LMF shall send a REGISTER message to the UE containing the lcs-SLMTLR invoke component as defined in 3GPP TS 24.080 [5] as described in clause 6.20.2, clause 6.20.3, clause 6.20.4 or clause 6.20.5 of 3GPP TS 23.273 [2]. In the lcs-SLMTLR invoke component, the LMF:

-	shall include the relatedUEInfo IE indicating information of candidate located UE(s) when the required location result is absolute location or indicating information of candidate SL reference UE(s) when the required location result is relative location;

-	shall include the locatedUEselect IE indicating whether the target UE or the LMF is to select the located UE(s) when the required location result is absolute location; and

-	may include the coordinateID IE indicating local coordinate(s) when the required location result is absolute location. The coordinateID IE is used to determine the target UE location in local coordinates.

When the lcs-SLMTLR invoke component is applied for periodic-triggered location events as described in clause 6.20.4 of 3GPP TS 23.273 [2], the LMF additionally:

-	shall include the requestedRangingResult IE indicating information of the requested location results (e.g. absolute locations, relative locations, distances, directions, velocities and relative velocities) for the target UE;

-	shall include the h-gmlc-address IE indicating the address for the (H)GMLC;

-	shall include the referenceNumber IE indicating the LDR reference number;

-	shall include periodicLocation IE, areaEventReporting IE, or motionEventReporting IE indicating the reporting criteria;

-	shall include the deferredRoutingIdentifier IE indicating identification of a new LMF, if any,which is to serve the UE; and

-	may include the lcs-QoS IE indicating the QoS for the requested location results.

If the UE accepts the lcs-SLMTLR invoke component, the UE shall send a RELEASE COMPLETE message to the LMF containing the lcs-SLMTLR return result component. In the lcs-SLMTLR return result component, the UE:

-	shall include the relatedUEInfo IE containing information of the related UEs for the ranging and sidelink positioning discovered by the UE, if the lcs-SLMTLR invoke component includes the locatedUEselect IE indicating that the LMF is to select the located UE(s);

-	shall include the relatedUEInfo IE containing information of the related UEs for the ranging and sidelink positioning selected by the UE, if the lcs-SLMTLR invoke component includes the locatedUEselect IE indicating that target UE is to select the located UE(s); and

-	may include the rangingSLPPList IE containing the embedded SLPP message providing sidelink positioning capabilities of the related UEs as defined in 3GPP TS 38.355 [4a].

If the UE is unable to process or support the request received from the network, the UE shall send a RELEASE COMPLETE message containing a return error component or reject component. Error values are specified in 3GPP TS 24.080 [5].

During the sidelink mobile terminating location request operation, the network shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the UE. If this timer expires the network shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message.

Figure 5.2.1.8.2.1 illustrates the signalling for normal operation between the UE and the network.

When the Facility element in the REGISTER message carries a content length of 255 bytes or less, the signalling is as follows:

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility (Invoke = lcs-SLMTLR (slmtlr-Type, supportedGADShapes, relatedUEInfo, locatedUEselect, coordinateID, lcs-QoS, requestedRangingResult, referenceNumber, h-gmlc-address, periodicLocation, areaEventReporting, motionEventReporting, deferredRoutingIdentifier))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility (Return result = lcs-SLMTLR(relatedUEInfo, rangingSLPPList))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

When the Facility element in the REGISTER message carries content longer than 255 bytes and if the UE and the NW supports the usage of ExtendedFacility information element as defined in 3GPP TS 24.080 [5], the Facility IE shall be replaced with an ExtendedFacility information element, the signalling is as follows:

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Extended Facility (Invoke = lcs-SLMTLR (slmtlr-Type, supportedGADShapes, relatedUEInfo, locatedUEselect, coordinateID, lcs-QoS, requestedRangingResult, referenceNumber, h-gmlc-address, periodicLocation, areaEventReporting, motionEventReporting, deferredRoutingIdentifier))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Extended Facility (Return result = lcs-SLMTLR(relatedUEInfo, rangingSLPPList))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Extended Facility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Extended Facility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Figure 5.2.1.8.2.1: Sidelink mobile terminating location request procedure

NOTE 1:	Only the following IEs defined in lcs-SLMTLR operation in 3GPP TS 24.080 [5] are used for ranging and sidelink positioning:

-	slmtlr-Type

-	supportedGADShapes

-	relatedUEInfo

-	locatedUEselect

-	coordinateID

-	rangingSLPPList

-	lcs-QoS

-	requestedRangingResult

-	referenceNumber

-	h-gmlc-address

-	periodicLocation

-	areaEventReporting

-	motionEventReporting

-	deferredRoutingIdentifier

NOTE 2:	The rangingSLPPList IE is added to the lcs-SLMTLR return result component to allow for passing multiple UE positioning information SLPP messages (e.g. UE capabilities) to the LMF. Its ASN.1 description is given in 3GPP TS 24.080 [5].

* * * Next Change * * * *

##### 5.2.2.1.2	Normal operation

The UE invokes an MO-LR by sending a REGISTER message to the network containing an lcs-MOLR invoke component. SS Version Indicator value 1 or above shall be used.

The receiving network entity shall initiate the handling of location request in the network. The network shall pass the result of the location procedure to the UE by sending a FACILITY message to the UE containing an lcs-MOLR return result component. When location estimate is kept in the network entity and this information satisfies the requested accuracy and the requested maximum age of location, then the network may reuse this information and the positioning measurement procedure may be skipped.

The network shall pass the result of the location procedure to the UE only if the location estimate is given in a format that the UE supports, as indicated by either the presence (and content) or the absence of the parameter supportedGADShapes, which may be sent by the UE in the lcs-MOLR invoke component.

The UE may terminate the dialogue by sending a RELEASE COMPLETE message in the case of single location request (see figure 5.2.2.1.1-1). The UE may also initiate another location request operation by sending a FACILITY message to the network containing an lcs-MOLR invoke component (see figure 5.2.2.1.1-2). After the last location request operation the UE shall terminate the dialogue by sending a RELEASE COMPLETE message.

If the network is unable to successfully fulfil the request received from the UE (e.g. to provide a location estimate or location assistance information), it shall clear the transaction by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5]. If the network is unable to provide a location estimate due to lack of support in the UE for the type of shape of the location estimate, then it shall use the error Facility Not Supported.

If the network has returned a result to the UE in a FACILITY message but, after some PLMN administered time period has elapsed, has not received either a new location request operation in a FACILITY message or a RELEASE COMPLETE message from the UE, the network may clear the transaction by sending a RELEASE COMPLETE message.

During the MO-LR operation the UE shall run a timer T(LCSL). This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message, and shall inform the user of the failure.

When the Facility element in the REGISTER or FACILITY message carries a content length of 255 bytes or less, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = lcs-MOLR (molr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, gpsAssistanceData, supportedGADShapes, lcsServiceTypeID, ageOfLocationInfo, locationType, pseudonymIndicator, h-gmlc-address, locationEstimate, velocityEstimate, referenceNumber, periodicLDRInfo, locationUpdateRequest, sequenceNumber, terminationCause, mo-lrShortCircuit, ganssAssistanceData, multiplePositioningProtocolPDUs, locationInfo, scheduledLocTime))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

When the Facility element in the REGISTER or FACILITY message carries content longer than 255 bytes and if the UE and the NW supports the usage of ExtendedFacility information element as defined in 3GPP TS 24.080 [5], the Facility IE shall be replaced with an ExtendedFacility information element, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Extended Facility (Invoke = lcs-MOLR (molr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, gpsAssistanceData, supportedGADShapes, lcsServiceTypeID, ageOfLocationInfo, locationType, pseudonymIndicator, h-gmlc-address, locationEstimate, velocityEstimate, referenceNumber, periodicLDRInfo, locationUpdateRequest, sequenceNumber, terminationCause, mo-lrShortCircuit, ganssAssistanceData, multiplePositioningProtocolPDUs, locationInfo, scheduledLocTime))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

Extended Facility (Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Reject (Invoke_problem))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Figure 5.2.2.1.2-1: Single mobile originated location request

When the Facility element in the REGISTER or FACILITY message carries a content length of 255 bytes or less, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = lcs-MOLR (molr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, gpsAssistanceData, supportedGADShapes, lcsServiceTypeID, ageOfLocationInfo, locationType, pseudonymIndicator, h-gmlc-address, locationEstimate, velocityEstimate, referenceNumber, periodicLDRInfo, locationUpdateRequest, sequenceNumber, terminationCause, mo-lrShortCircuit, ganssAssistanceData, multiplePositioningProtocolPDUs, locationInfo, scheduledLocTime))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

(Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

FACILITY

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = lcs-MOLR (molr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, gpsAssistanceData, supportedGADShapes, lcsServiceTypeID, ageOfLocationInfo, locationType, pseudonymIndicator, h-gmlc-address, locationEstimate, velocityEstimate, referenceNumber, periodicLDRInfo, locationUpdateRequest, sequenceNumber, terminationCause, mo-lrShortCircuit, ganssAssistanceData, multiplePositioningProtocolPDUs, locationInfo))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

(Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

When the Facility element in the REGISTER or FACILITY message carries content longer than 255 bytes, the Facility information element shall be replaced with an ExtendedFacility information element, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Extended Facility (Invoke = lcs-MOLR (molr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, gpsAssistanceData, supportedGADShapes, lcsServiceTypeID, ageOfLocationInfo, locationType, pseudonymIndicator, h-gmlc-address, locationEstimate, velocityEstimate, referenceNumber, periodicLDRInfo, locationUpdateRequest, sequenceNumber, terminationCause, mo-lrShortCircuit, ganssAssistanceData, multiplePositioningProtocolPDUs, locationInfo, scheduledLocTime))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

(Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Reject (Invoke_problem))

FACILITY

------------------------------------------------------------------------------------------------------------------------>

Extended Facility (Invoke = lcs-MOLR (molr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, gpsAssistanceData, supportedGADShapes, lcsServiceTypeID, ageOfLocationInfo, locationType, pseudonymIndicator, h-gmlc-address, locationEstimate, velocityEstimate, referenceNumber, periodicLDRInfo, locationUpdateRequest, sequenceNumber, terminationCause, mo-lrShortCircuit, ganssAssistanceData, multiplePositioningProtocolPDUs, locationInfo))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

(Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Reject (Invoke_problem))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Figure 5.2.2.1.2-2: Multiple mobile originated location requests

NOTE 1:	Only the following IEs defined in lcs-MOLR operation in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	molr-Type

-	lcs-QoS

-	lcsServiceTypeID

-	ageOfLocationInfo

-	locationType

-	mlc-Number

-	lcsClientExternalID

-	pseudonymIndicator

-	supportedGADShapes

-	multiplePositioningProtocolPDUs

-	locationEstimate

-	add-LocationEstimate

-	h-gmlc-address

-	decipheringKeys

-	scheduledLocTime

-	velocityEstimate

NOTE 2:	The multiplePositioningProtocolPDUs IE is added to the lcs-MOLR invoke component to allow for passing multiple UE positioning information LPP messages (e.g. UE location measurements or UE capabilities) to the LMF for NG-RAN LCS. Its ASN.1 description is given in 3GPP TS 24.080 [5], where the maximum number of LPP messages is specified. There is one or more types of multiple LPP messages can be encapsulated in one multiplePositioningProtocolPDUs IE.

* * * Next Changes * * * *

##### 5.2.2.2.2	Normal operation

The UE invokes a cancel deferred location procedure by sending a REGISTER message containing an lcs-MSCancelDeferredLocation invoke component to the LMF as defined in 3GPP TS 24.080 [5].

The LMF shall terminate the ongoing periodic or triggered location if this can be identified from the information in the lcs-MSCancelDeferredLocation invoke component.

The LMF shall then return a RELEASE COMPLETE message containing an lcs-MSCancelDeferredLocation return result component (see Figure 5.2.2.2.2-1).

NOTE:	If the UE has requested the cancellation of the deferred location procedure for the location events reporting over user plane connection and the UE receives the acknowledgment of the cancellation from the LMF, the UE releases the user plane connection to the LCS Client or AF if the release has not been yet initiated.

If the LMF is unable to process the request received from the UE or can not identify the ongoing periodic or triggered location to be terminated, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated cancel deferred location operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed, The UE may terminate the dialogue by sending a RELEASE COMPLETE message, and informs the user of the failure.

When the Facility element in the REGISTER message carries a content length of 255 bytes or less, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = lcs-MSCancelDeferredLocation (referenceNumberExt, h-gmlc-callBackUri))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = lcs-MSCancelDeferredLocation)

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

When the Facility element in the REGISTER message carries content longer than 255 bytes and if the UE and the NW supports the usage of ExtendedFacility information element as defined in 3GPP TS 24.080 [5], the Facility information element shall be replaced with an ExtendedFacility information element, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Extended Facility (Invoke = lcs-MSCancelDeferredLocation (referenceNumberExt, h-gmlc-callBackUri))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Extended Facility (Return result = lcs-MSCancelDeferredLocation)

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.2.2-1: Cancel Deferred Location

NOTE:	Only the following IEs defined in lcs-MSCancelDeferredLocation operation in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	referenceNumberExt

-	h-gmlc-callBackUri

* * * Next Changes * * * *

##### 5.2.2.4.2	Normal operation

The UE invokes an EventReport procedure by sending a REGISTER message containing an lcs-EventReport invoke component to the LMF as defined in 3GPP TS 24.080 [5].

If the LMF can handle this event report, the LMF shall return a RELEASE COMPLETE message containing an lcs-EventReport return result component (see Figure 5.2.2.4.2-1).

If the LMF is unable to process the request received from the UE or cannot identify the ongoing periodic or triggered location to be terminated, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated event reporting procedure operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed. The UE may terminate the dialogue by sending a RELEASE COMPLETE message, and informs the user of the failure.

When the Facility element in the REGISTER message carries a content length of 255 bytes or less, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = lcs-EventReport (eventType, referenceNumberExt, h-gmlc-callBackUri, lcs-QoS, locationInfo, supportedGADShapes, multiplePositioningProtocolPDUs, terminationCause, cumulativeEventReport))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = lcs-EventReport(deferredRoutingIdentifier, terminationCause))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

When the Facility element in the REGISTER message carries content longer than 255 bytes and if the UE and the NW supports the usage of ExtendedFacility information element as defined in 3GPP TS 24.080 [5], the Facility IE shall be replaced with an ExtendedFacility information element, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Extended Facility (Invoke = lcs-EventReport (eventType, referenceNumberExt, h-gmlc-callBackUri, lcs-QoS, locationInfo, supportedGADShapes, multiplePositioningProtocolPDUs, terminationCause, cumulativeEventReport))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Extended Facility (Return result = lcs-EventReport(deferredRoutingIdentifier, terminationCause))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.4.2-1: EventReport

NOTE:	Only the following identifiers defined in lcs-EventReport operation in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	eventType

-	referenceNumberExt

-	h-gmlc-callBackUri

-	lcs-QoS

-	locationInfo

-	supportedGADShapes

-	multiplePositioningProtocolPDUs

-	terminationCause

-	cumulativeEventReport

* * * Next Changes * * * *

##### 5.2.2.11.2	Normal operation

In order to initiate the UE initiated RSPP supplementary information transport procedure, the initiating UE shall send a REGISTER message to the LMF containing the lcs-ULRSPPTransport invoke component as defined in 3GPP TS 24.080 [5].

In the lcs-ULRSPPTransport invoke component included in the REGISTER message, the UE shall include rangingSLPPList IE. In the rangingSLPPList, the UE:

a)	may include the embedded SLPP message as defined in 3GPP TS 38.355 [4a] for the initiating UE; and

b)	shall include the embedded SLPP message(s) as defined in 3GPP TS 38.355 [4a] for the related SL reference UE(s) or the related located UE(s) with the associated application layer ID(s).

NOTE:	The SLPP message(s) sent by the initiating UE include the SLPP message(s) to provide capability, to request assistance data or to provide location information as defined in 3GPP TS 38.355 [4a].

The LMF shall then return a RELEASE COMPLETE message containing an lcs-ULRSPPTransport return result component (see Figure 5.2.2.11.2-1) if the LMF accepts the lcs-ULRSPPTransport invoke component.

If the LMF does not accept the lcs-ULRSPPTransport invoke component, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated RSPP supplementary information transport operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed. The UE may terminate the dialogue by sending a RELEASE COMPLETE message, and informs the user of the failure.

Figure 5.2.2.11.2.1 illustrates the signalling for normal operation between the UE and the network.

When the Facility element in the REGISTER message carries a content length of 255 bytes or less, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = lcs-ULRSPPTransport (rangingSLPPList))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = lcs-ULRSPPTransport())

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

When the Facility element in the REGISTER message carries content longer than 255 bytes and if the UE and the NW supports the usage of ExtendedFacility information element as defined in 3GPP TS 24.080 [5], the Facility IE shall be replaced with an ExtendedFacility information element, the signalling is as follows:

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Extended Facility (Invoke = lcs-ULRSPPTransport (rangingSLPPList))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Extended Facility (Return result = lcs-ULRSPPTransport())

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Extended Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.11.2-1: UE initiated RSPP supplementary information transport

NOTE 1: Only the following IEs defined in lcs-ULRSPPTransport operations in 3GPP TS 24.080 [5] are used for ranging and sidelink positioning:

-	rangingSLPPList

NOTE 2:	The rangingSLPPList IE is added to lcs-ULRSPPTransport invoke component to allow for passing multiple SLPP messages for the UE connecting with the LMF and/or the other related UE. Its ASN.1 description is given in 3GPP TS 24.080 [5].

* * * next change* * * *

## 5.4	Negotiation on ExtendedFacility information element capability

This clause specifies the procedures needed by the the network and the UE to negotiate whether they support the ExtendedFacility information element through the CapabilityInformation supplementary service defined in 3GPP TS 24.080 [5].

#### 5.4.1	Negotiation on ExtendedFacility information element capability initiated by the network

##### 5.4.1.1	General

If the network supports the use of ExtendedFacility information element and determines that the length of the message content to be sent to the UE is above 255 bytes and the network does not know whether the UE supports the use of ExtendedFacility information element, the network may request the UE using the CapabilityInformation service to indicate the support of the use of ExtendedFacility information element as shown in the Figure 5.4.1.1-1. If the UE supports the use of ExtendedFacility information element, the UE shall respond to the network with indicating the support. Upon receiving the indication from the UE that the UE supports the use of ExtendedFacility information elemenet, the network shall encode the message content using the ExtendedFacility information element as defined in 3GPP TS 24.080 [5].

Figure 5.4.1.1-1: NAS signaling transport for Network initiated CapabilityInformation service

##### 5.4.1.2	Normal operation

The LMF invokes a capability exchange procedure by sending a REGISTER message containing capabilityInformation invoke component to the UE as defined in 3GPP TS 24.080[5]. The REGISTER message is transported to the UE via the serving AMF as described in figure 5.4.1.2-1. The REGISTER message contains information indicating whether the network supports the ExtendedFacility information element.

If the UE can support the CapabilityInformation supplementary service and support the ExtendedFacility information element, the UE returns a RELEASE COMPLETE message to the LMF containing CapabilityInformation return result component. The RELEASE COMPLETE message is transported to the LMF via the serving AMF as described in figure 5.4.1.1-1.

If the network requests the UE to indicate the support of the use of ExtendedFacility information element and the UE returns an error, then the network determines that the UE does not support the use of ExtendedFacility information element and the network shall use Facility IE to communicate with the UE.

If the UE is unable to process or support the request received from the network, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component or reject component. Error values are specified in 3GPP TS 24.080 [5].

During the supplementary services capabilityInformation operation, the network shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the UE. If this timer expires the network shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message.

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility (Invoke =capabilityInformation (extendedFacilityInd))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility (Return result = capabilityInformation (extendedFacilityInd))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Figure 5.4.1.2-1: CapabilityInformation

#### 5.4.2	Negotiation on ExtendedFacility information element capability initiated by the UE

##### 5.4.2.1	General

If the UE supports the use of ExtendedFacility information element and determines that the length of the message content to be sent to the network is above 255 bytes and the UE does not know whether the network supports the use of ExtendedFacility information element, the UE may request the UE using the CapabilityInformation service to indicate the support of the use of ExtendedFacility information element as shown in the Figure 5.4.2.1-1. If the network supports the use of ExtendedFacility information element, the network shall respond to the UE with indicating the support. Upon receiving the indication from the network that the network supports the use of ExtendedFacility Information element, the UE shall encode the message content using the ExtendedFacility information element as defined in 3GPP TS 24.080 [5].

Figure 5.4.2.1-1: NAS signaling transport for UE initiated CapabilityInformation service

##### 5.4.2.2	Normal operation

The UE invokes a capability exchange procedure by sending a REGISTER message containing capabilityInformation invoke component to the network as defined in 3GPP TS 24.080[5]. The REGISTER message contains information indicating whether the UE supports the ExtendedFacility information element.

If the LMF can support the CapabilityInformation supplementary service and support the ExtendedFacility information element, the LMF returns a RELEASE COMPLETE message to the UE containing CapabilityInformation return result component as described in figure 5.4.2.2-1.

If the UE requests the network to indicate the support of the use of ExtendedFacility information element and the network returns an error, then the UE determines that the network does not support the use of ExtendedFacility information element and the UE shall use Facility IE to communicate with the UE.

If the LMF is unable to process or support the request received from the UE, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component or reject component. Error values are specified in 3GPP TS 24.080 [5].

During the supplementary services capabilityInformation operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = capabilityInformation (extendedFacilityInd))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = capabilityInformation (extendedFacilityInd))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Figure 5.4.2.2-1: Single mobile originated location request

* * * End of Changes * * * *