3GPP TSG-CT WG1 Meeting #157	C1-256535

Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Clarification on inventory procedure not accepted by AIoT device

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

When the disable command is completed by an AIoT device, the AIoT device is not expected to process any AIoT NAS procedure. However, in clause 5.2.4, it states a AIoT device shall not respond to the AIoT paging message (as copied below).

5.2.4	Inventory procedure not accepted by AIoT device

If the AIoT device has completed a disable command procedure, the AIoT device shall not respond to the AIoT paging message.

This gives the impression that the AIoT MAC will provide the AIoT identification information to AIoT NAS layer, and NAS layer determines to reject the procedure due to the disable command. However, in our view, once the disable command is completed, both AIoT NAS layer and AIoT MAC layer is shut down. That is, the AIoT MAC layer will no longer to listen to the broadcast message. Hence, the above description is misleading.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

* * * Next Change * * * *

5.2.4	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	Failure of authentication.

If an AIoT device fails the authentication procedure as specified in 3GPP TS 33.369 [6], the AIoT device shall not respond to the AIoT paging message.

b)	Collision of AIoT paging.

If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall proceed with the ongoing inventory procedure and shall process the new AIoT paging after the completion of the inventory procedure.

* * * Next Change * * * *

5.2.5	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of an inventory failure received as specified by 3GPP TS 38.413 [10] from all the selected NG-RAN(s), the AIOTF shall abort the inventory procedure.

In the case the lower layer indication of no inventory response from all the selected NG-RAN(s), the AIOTF shall abort the inventory procedure.

* * * End of Changes * * * *