3GPP TSG-CT WG1 Meeting #157	C1-256802

Sophia Antipolis, France, 13-17 October 2025	(revision of C1-256473)

Source:	China Mobile

Title:	Pseudo-CR on update on DC application profiles downloading on UE

Spec:	3GPP TS 24.392 v1.0.0

Agenda item:	19.62

Document for:	Approval

1. Introduction

This pCR is proposed to update the procedures of DC application profiles downloading on UE.The updates include the following aspects:

1.	The procedures for Obtaining the Root application and obtaining the DC application profile(s), which were previously covered in a single clause, have now been separated into two distinct clauses. Each clause includes descriptions of the procedures for both the MMTel Enabler client and the MMTel Enabler server. This is done to make the procedure descriptions clearer and to facilitate referencing a specific procedure.

2.	Add the requirement of zip package included in the Root Application response to the procedure of Obtaining the Root application.

3.	Add a general clause to describe the overall procedures.

2. Reason for Change

update on DC application profiles downloading on UE.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.392 v1.0.0.

* * * First Change * * * *

6.2.1	DC application profiles downloading on UE

6.2.1.1	General

This clause specifies the following procedures:

1.	the UE’s obtaining the Root application is specified in clause 6.2.1.2; and

2.	the UE’s obtaining the DC application profile(s) is specified in clause 6.2.1.3.

6.2.1.2	Obtaining the Root application

6.2.1.2.1	Procedure at the MMTel enabler client

In order to obtain the Root application, the MMTel enabler client shall send an HTTP GET request to the MMTel enabler server for the Root application. In this HTTP GET request, the MMTel enabler client shall include a Request-URI set to ("/") with the following query parameters:

a)	shall include a <device-vendor> element to indicate the vendor of the terminal device;

b)	shall include a <device-type> element to indicate the model of the terminal device; and

c)	may include a <root-application-version> element to indicate the version of the Root application in the terminal device;

Upon receiving an HTTP 200 (OK) from the MMTel enabler server, the MMTel enabler client shall parse the response message received from the MMTel enabler server and updates the local Root application and Root application validity to the value received in <root-application-validity> element. If received an HTTP 304 (Not Modified) from the MMTel enabler server, the MMTel enabler client does not update the local Root application.

6.2.1.2.2	Procedure at the MMTel enabler server

Upon receiving the HTTP GET request from the MMTel enabler client where the Request-URI of the HTTP GET request is set to ("/"), the MMTel enabler server shall check whether the Root application version included in the query parameters is the latest version:

- if the Root application version is not the latest version or the Root application version is not included in the request, the MMTel enabler server shall generate an HTTP 200 (OK) response containing:

- a Content-Type header field set to "application/json";

- shall include the information elements as specified in 3GPP TS 23.392 [2] encoded in JSON format as specified in clause 7.1.1, in the HTTP payload:

- shall include an <update-needed> element to indicate that the Root application is needed to be updated by the MMTel enabler client in the UE;

- may include a <root-application-version> element to indicate the newest version of the Root application;

- may include a <root-application-validity> element to indicate the validity of the Root application; and

- may include a <root-application> element to indicate the Root application provided by the MMTel service provider; and

- shall include a zip package in the HTTP payload. The zip package contains the HTML file that serves as the entry point of the Root application, along with supporting CSS and JS scripts, which together form an executable HTML5 page. The HTML file shall be placed in the root directory of the zip package, and its filename must be fixed as index.html.

NOTE:	The details of the zip package included in the HTTP payload is implementation specific.

- if the Root application version is the latest version, the MMTel enabler server shall generate an HTTP 304 (Not Modified) response;

6.2.1.3	Obtaining the DC application profile(s)

6.2.1.3.1	Procedure at the MMTel enabler client

In order to obtain the DC application profile(s), the MMTel enabler client shall send an HTTP GET request to the MMTel enabler server for DC application profiles. In this HTTP GET request, the MMTel enabler client shall include a Request-URI set to ("/applicationlist") with the following query parameters:

a)	shall include a <MMTel-enabler-client-version> element to indicate the version of the MMTel enabler client;

b)	may include an <application-type> element to indicate the type of the application that UE wants to request;

c)	may include a <begin-index> element to indicate the starting index of the first Data Channel Application required in the Data Channel Application list; and

d)	may include a <number-of-application-profile-required> element to indicate the total number of DC application profiles required in this request;

Upon receiving the HTTP GET response for the DC application profiles from the MMTel enabler server, the MMTel enabler client shall

1.	parse the response message received from the MMTel enabler server;

2.	locally stores the downloaded DC application profiles

3.	updates the DC application profile validity to the value received in <application-validity> element; and

4.	displays the related application information from the DC application profile on the Root application.

NOTE:	The UX aspect is out of scope of the present document.

6.2.1.3.2	Procedure at the MMTel enabler server

Upon receiving the HTTP GET request from the MMTel enabler client where the Request-URI of the HTTP GET request is set to ("/applicationlist"), the MMTel enabler server shall determine the list of DC application profiles for the response based on the query parameters included in the Request-URI and generate an HTTP 200 (OK) response containing:

- Content-Type header field set to "application/json"; and

- shall include the information elements as specified in 3GPP TS 23.392 [2] encoded in JSON format as specified in clause 7.1.2, in the HTTP payload:

- shall include a <DC-application-profile number> element to indicate the total number of the DC Application profile included in the list;

- shall include a <begin-index> element to indicate the starting index of the first Data Channel Application responded in the Data Channel Application list;

- shall include a <DC-application-profile-list> element, each element in this list contains a DC application profile of this Data Channel Application. The cardinality of <DC-application-profile-list> shall exactly match the value of the <DC-application-profile-number>. For each DC application profile:

- shall include an <app-id> element to identify a Data Channel Application;

- may include an <application-name> element to indicate the name of the Data Channel Application;

- may include a <service-type> element to help the user understand the type of services provided by the application;

- may include an <application-icon> element to indicate the icon of the Data Channel Application. This element includes the URL of the application icon;

- may include an <application-version> element to indicate the version of the Data Channel Application;

- may include an <application-validity> element to indicate the latest validity of the Data Channel Application;

- may include an <application-loading-phase> element to indicate when this Data Channel Application is allowed to be used; When present, shall accept one of the following enumerated values

-	Precall only and

-	Incall only;

- may include an <autoload> element to indicate whether this Data Channel Application is needed to be load to the UE automatically;

- may include an <autolaunch> element to indicate whether this Data Channel Application is needed to be downloaded to the UE and run automatically;

- may include an <if-work-without-peer-DC> element to indicate whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call;

- may include a <supported-scenario> element to indicate the supported media type required for this Data Channel Application This element, when present, shall accept one of the following enumerated values:

-	Voice call only;

-	Video call only; and

-	Voice and video call;

- may include a <condition> element to indicate whether this Data Channel Application can be used under this condition. The conditions may include Service area, Time period, etc.;

- may include a <3gpp-qos-hint> element to indicate the QoS requirement of this Data Channel Application; The value of this element is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application.

- may include a <personal-data-collection> element to indicate whether this Data Channel Application will collect the user personal data; and

may include a <personal-data-collection-info-URL> element to indicate the URL of the description of the purpose of mandatory and optional personal data to be collected.