3GPP TSG-CT WG1 Meeting #157	C1-256540

# Sophia Antipolis, France, 13-17 October 2025

Source:	Ericsson, vivo, Lenovo, Samsung

Title:	Pseudo-CR on privacy protection

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

2. Reason for Change

2.1 Stage-2 requirements

As states in 33.369:

When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2.

and

The AIOTF shall support a mechanism for the use of temporary IDs and it is optional for network to use.

Thus, whether the privacy protection is used or not, is decision of the network, i.e. operator of the network.

Further stage-2 requirements for privacy as in 33.369 clause 5.4.2 and clause 5.4.3.

2.2 Information providing in paging message and INVENTORY REPORT

If the privacy protection is used:

-	and the network performs paging with filtering information, then the AIoT device does not include the AIoT device permanent identifier in INVENTORY REPORT, and the network derives AIoT device permanent identifier from RESAIOT (see in 33.369 clause 5.4.2).

-	and the network performs paging with a single device, then the network includes T-ID related information (33.369 clause 5.4.3) in paging message and the AIoT device does not include the AIoT device permanent identifier in INVENTORY REPORT, and the network derives AIoT device permanent identifier from RESAIOT (see in 33.369 clause 5.4.3).

-	and the network performs paging for all AIoT devices, then solution for performing paging with filtering information can be reused.

If the privacy protection is not used:

-	and the network performs paging with filtering information, then the AIoT device includes the AIoT device permanent identifier in INVENTORY REPORT.

-	and the network performs paging with a single device, then the AIoT device includes the AIoT device permanent identifier in INVENTORY REPORT (see 33.369 clause 5.4.1). It is proposed that the network includes the AIoT device permanent identifier in paging message too, since the AIoT device permanent identifier will anyway be disclosed in INVENTORY REPORT.

-	and the network performs paging for all device, then solution for performing paging with filtering information can be reused.

2.3 When AIoT device determining whether privacy is used or not

If indication that the privacy protection is used, was included in a paging message and was blindly followed by the AIoT device, then given that the paging message is not integrity protected and the AIoT device does not authenticate the network at time of handling the AIoT device, such solution would enable an attacker to send a paging message without indicating the privacy protection is used. For such paging message, AIoT device would disclose its AIoT device permanent identifier. Privacy would not be ensured. Hence, solution relying on indicating the privacy protection is used in paging message does not work.

If it is pre-configured in the device that the privacy protection is used, and the privacy protection is used is enforced by the AIoT device when handling paging message, then the AIoT device will not disclose its AIoT device permanent identifier, even if a paging message is sent by an attacker.

3. Conclusions

It is proposed to add the clarification about the privacy protection is based on the operator policy for a given AIoT device, define a configuration parameter in AIoT device indicating whether the privacy protection is used or not, and:

- when the privacy protection is not used, the AIoT device permanent identifier is included in the paging message for individual paging and the INVENTORY REPORT message for group paging and paging all.

- when the privacy protection is used, the AIoT device permanent identifier is included in neither the paging message nor the INVENTORY REPORT message.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369.

***** change *****

### 4.2.6	Privacy of AIoT device identifiers

The AIoT NAS protocol supports privacy of AIoT device identifiers by enabling the use of Temporary Identifiers (T-IDs), as specified in 3GPP TS 33.369 [6].

The network shall support that the mechanism for privacy protection and usage of privacy protection is based on the network policy and deployment.

Editor's note:	It is FFS how the AIoT device determines whether privacy protection is used.

When privacy protection is not used:

-	the AIoT device permanent identifier is included as the AIoT identification information in the paging message for individual paging; and

-	the AIoT device permanent identifier is included in the INVENTORY REPORT message sent as response to any paging message.

When privacy protection is used:

-	T-ID is included as the AIoT identification information in the paging message for individual paging; and

-	No AIoT device identity is included in the INVENTORY REPORT message sent as response to any paging message.

***** change *****

### 5.2.3	Inventory procedure completion

The AIoT device determines to respond to the AIoT paging:

a)	based on the indication from lower layer as specified in 3GPP TS 38.391 [8]; or

b)	based on whether the AIoT identification information received from the lower layer is matched or not.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

The AIoT device determines whether the AIoT identification information is matched or not as follows:

Editor's note:	It is FFS how to determine the component of AIoT identification information for AIoT device, and this may need coordination with RAN2/3.

a)	if the AIoT identification information includes an AIoT device identifier:

Editor's note:	Details of AIoT device identifier should be aligned with security consideration from SA3. How to match the AIoT device identifier depends on the AIoT device identifier included.

b)	if the AIoT identification information includes filtering information (see clause 31.3 of 3GPP TS 23.003 [5]):

1)	based on ID type field included in the filtering information, the AIoT device determines the AIoT identification information is matched if:

A)	the PLMN ID field is not included in the filtering information, or the PLMN ID field is included in the filtering information and:

i)	the PLMN ID field is included in the AIoT device permanent identifier; and

ii)	the PLMN ID field in the AIoT device permanent identifier is the same as the PLMN ID field within the filtering information;

B)	the NID field is not included in the filtering information, or the NID field is included in the filtering information and:

i)	the NID field is included in the AIoT device permanent identifier; and

ii)	the NID field in the AIoT device permanent identifier is the same as the NID field within the filtering information;

C)	the third party ID field is not included in the filtering information, or the third party ID field is included in the filtering information and:

i)	the third party ID field is included in the AIoT device permanent identifier; and

ii)	the third party ID field in the AIoT device permanent identifier is the same as the third party ID field within the filtering information; and

D)	the identification information filters field is not included in the filtering information, or the identification information filters field is included in the filtering information and for each identification information filter, the component value within the identification information filter is the same as the corresponding bitstring from the AIoT device permanent identifier based on offset field and length field; or

2)	otherwise, the AIoT device determines the AIoT identification information is not matched.

NOTE 1:	The AIoT identification information corresponds to paging ID, as defined in 3GPP TS 38.391 [8]).

Editor's note:	The reference to filtering information (e.g. IE name) will be updated according to CT4 and RAN2 progress.

Based on whether the AIoT identification information is matched:

a)	if the AIoT identification information is matched, the AIoT device shall indicate to the lower layer that the AIoT identification information is matched, and determine to respond to the AIoT paging; or

b)	otherwise, the AIoT device shall ignore the AIoT identification information, indicate to the lower layer that the filtering information is not matched, and determine not to respond to the AIoT paging.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

If privacy protection is not used, the AIoT device shall include the AIoT device permanent identifier in the AIoT device identity IE in the INVENTORY REPORT message.

If privacy protection is used, the AIoT device shall not include the AIoT device identity IE in the INVENTORY REPORT message.

Upon all the selected NG-RAN indicating the inventory procedure has been completed, the inventory procedure is considered as completed by the AIOTF as specified in 3GPP TS 23.369 [2].

NOTE:	How NG-RAN indicates the inventory procedure has been completed is defined in 3GPP TS 38.300 [7].

Editor's note:	The reference to TS 38.300 may be updated according to RAN2 progress.

Editor's note:	AIoT .device authentication of the network is FFS.

***** change *****

##### 7.1.2.1	Message definition

The INVENTORY REPORT message is sent by the AIoT device to the AIOTF to reply to the AIoT paging.

See table 7.1.2.1-1.

Message type:	INVENTORY REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | Security parameters | Security parameters 7.2.3 | M | V | n |
|  | RANDAIOT_d | RAND 7.2.10 | M | V | TBD |
|  | RESAIOT | Authentication response parameter 7.2.7 | M | V | TBD |
| XX | AIoT device identity | AIoT device identity 7.2.6 | O | TLV | 15-31 |

***** change *****

### 7.2.6	AIoT device identity

The purpose of the AIoT device identity information element is to provide the network with the AIoT device identity of the AIoT device.

The AIoT device identity information element is coded as shown in figure 7.2.6-1 and table 7.2.6-1.

The AIoT device identity is a type 4 information element with minimum length of 15 octets and maximum length of 31 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIoT device identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of AIoT device identity contents |  |  |  |  |  |  |  | octet 2 |
| AIoT device identity |  |  |  |  |  |  |  | octet 3 octet m |

Figure 7.2.6-1: AIoT device identity information element

Table 7.2.6-1: AIoT device identity

AIoT device identity(octets 3 to m)

This field is encoded as the structure of AIoT device permanent identifier as specified in clause 31.2 in 3GPP TS 23.003 [5]. When the ID Type is set to "NID part is included", bits 5 to 8 of the last octet of the NID field are coded as zero.