3GPP TSG-CT WG3 Meeting #143	C3-254037

# Sophia-Antipolis, France, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1707 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Device Location in NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CEWiT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 23.369 clause 7.4.2 and 7.4.3, location information requested was added as the optional parameter in the Nnef_AIoT_Inventory service request and Nnef_AIoT_Command service request. In clause 7.4.4, Location information of the AIOT Device was added as the optional parameter in the Nnef_AIoT_Notify service request. This CR intends to add this location information requested attribute in the InventoryReq data type and CommandReq data type. Also, this CR intends to add locationof the AIoT device in the AIoTNotif data type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of the attribute “deviceLocInd” with “boolean” datatype in clause 5.45.5.2.2. - Addition of the attribute “deviceLocInd” with “boolean” datatype in clause 5.45.5.2.4. - Addition of the attribute “deviceLoc” with “string” datatype in clause 5.45.5.2.6. - OpenAPI updation accordingly at A.43 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, there will be gap between Stage 2 and Stage 3 spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.45.5.2.2, 5.45.5.2.4, 5.45.5.2.6, A.43 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29569_Naiotf_AIoT.yaml TS29522_AIoT.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 5.45.5.2.2	Type: InventoryReq

Table 5.45.5.2.2-1: Definition of type InventoryReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| targetArea | ExtAIoTArea | C | 0..1 | Contains the target AIoT Service Area within which the requested Inventory operation shall apply. (NOTE) |  |
| targetDevices | AIoTDevices | C | 0..1 | Contains the target AIoT device(s) related information. (NOTE) |  |
| numDevices | Uinteger | O | 0..1 | Contains the approximative number of the targeted AIoT device(s). |  |
| timeInterval | DurationSec | O | 0..1 | Contains the time interval to be used for results aggregation. |  |
| notifUri | Uri | M | 1 | Contains the URI via which the AIoT Inventory operation related notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.45.6. This attribute shall be present only when feature negotiation is required. |  |
| deviceLocInd | boolean | O | 0..1 | Contains the Indication to request the device location. - "true" indicates that location of the device is requested - "false" indicates that location of the device is not requested. - The default value is "false" when this attribute is omitted. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** Next Change ***

##### 5.45.5.2.4	Type: CommandReq

Table 5.45.5.2.4-1: Definition of type CommandReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| commandType | CommandType | M | 1 | Contains the type of the requested command. |  |
| targetArea | ExtAIoTArea | C | 0..1 | Contains the target AIoT Service Area within which the requested command operation shall apply. (NOTE) |  |
| targetDevices | AIoTDevices | C | 0..1 | Contains the target AIoT device(s) related information. (NOTE) |  |
| numDevices | Uinteger | O | 0..1 | Contains the approximative number of the targeted AIoT device(s). |  |
| msgSize | Uinteger | O | 0..1 | Contains the approximative message size in units of Bytes. This attribute may be present only if the "commandType" attribute is set to "READ". |  |
| offset | Uinteger | C | 0..1 | Contains the offset, expressed in units of bytes. This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the offset from which to read the application data. - If the "commandType" attribute is set to "WRITE", this attribute contains the offset from which to write the application data. |  |
| length | Uinteger | C | 0..1 | Contains the length of application data, expressed in units of bytes (i.e., byte length). This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the length of application data to read. - If the "commandType" attribute is set to "WRITE", this attribute contains the length of application data to write. |  |
| data | Bytes | C | 0..1 | Contains the application data to write. This attribute shall be present only if the "commandType" attribute is set to "WRITE". |  |
| notifUri | Uri | M | 1 | Contains the URI via which the AIoT Command operation related notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.45.6. This attribute shall be present only when feature negotiation is required. |  |
| deviceLocInd | boolean | O | 0..1 | Contains the Indication to request the device location. - "true" indicates that location of the device is requested - "false" indicates that location of the device is not requested. - The default value is "false" when this attribute is omitted. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** Next Change ***

##### 5.45.5.2.6	Type: AIoTNotif

Table 5.45.5.2.6-1: Definition of type AIoTNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afTransId | string | M | 1 | Contains the identifier of the AF transaction to which the notification is related. |  |
| devicesRepData | array(DevicesRepInfo) | C | 1..N | Contains the AIoT device(s) related reporting information. (NOTE) |  |
| lastRepInd | boolean | C | 0..1 | Contains the Last Report Indication, i.e., indicates whether this is the last reporting from the AIOTF for the AIoT service operation identified by the "transId" attribute. - "true" indicates that this is the last report. - "false" indicates that this is not the last report. - The default value is "false" when this attribute is omitted. (NOTE) |  |
| deviceLoc | string | C | 0..1 | This attribute shall be present when the NF service consumer has requested device location by setting "deviceLocInd" as "true" in the inventory or command request. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** Next Change ***

# A.43	AIoT API

openapi: 3.0.0

info:

title: 3gpp-aiot

version: 1.0.0-alpha.2

description: |

API for UE Address service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

**************** Text Skipped for Clarity *****************

STRUCTURED DATA TYPES

#

InventoryReq:

description: Represents the AIoT Inventory request.

type: object

properties:

afId:

type: string

targetArea:

$ref: '#/components/schemas/ExtAIoTArea'

targetDevices:

$ref: 'TS29569_Naiotf_AIoT.yaml#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

deviceLocInd:

type: boolean

default: false

description: >

Contains the indiation to request the location of the device.

true indicates that the device location is requested.

false indicates thet the device location is not requested.

The default value is false when this attribute is omitted.

required:

- afId

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

InventoryResp:

description: Represents the AIoT Inventory response.

type: object

properties:

afTransId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afTransId

CommandReq:

description: Represents the AIoT Command request.

type: object

properties:

afId:

type: string

commandType:

$ref: '#/components/schemas/CommandType'

targetArea:

$ref: '#/components/schemas/ExtAIoTArea'

targetDevices:

$ref: 'TS29569_Naiotf_AIoT.yaml#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

msgSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

offset:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

length:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

data:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

deviceLocInd:

type: boolean

default: false

description: >

Contains the indiation to request the location of the device.

true indicates that the device location is requested.

false indicates thet the device location is not requested.

The default value is false when this attribute is omitted.

required:

- afId

- commandType

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

CommandResp:

description: Represents the AIoT Command response.

type: object

properties:

afTransId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afTransId

AIoTNotif:

description: Represents the AIoT Operations Notification.

type: object

properties:

afTransId:

type: string

devicesRepData:

type: array

items:

$ref: 'TS29569_Naiotf_AIoT.yaml#/components/schemas/DevicesRepInfo'

minItems: 1

lastRepInd:

type: boolean

default: false

description: >

Contains the Last Report Indication, i.e., indicates whether this is the last reporting

from the AIOTF for the AIoT service operation identified by the transId attribute.

true indicates that this is the last report.

false indicates that this is not the last report.

The default value is false when this attribute is omitted.

deviceLoc:

type: string

required:

- afTransId

anyOf:

- required: [devicesRepData]

- required: [lastRepInd]

ExtAIoTArea:

description: Represents the AIoT area.

type: object

properties:

extAreaIds:

type: array

items:

$ref: '#/components/schemas/ExtAiotAreaId'

minItems: 1

geographicAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

civicAddresses:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

minItems: 1

oneOf:

- required: [extAreaIds]

- required: [geographicAreas]

- required: [civicAddresses]

#

# SIMPLE DATA TYPES

#

ExtAiotAreaId:

description: Represents the External AIoT Area identifier.

type: string

#

# ENUMERATIONS

#

CommandType:

anyOf:

- type: string

enum:

- READ

- WRITE

- PERMANENT_DISABLE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the type of AIoT Command.

Possible values are:

- READ: Indicates that the AIoT Command is Read (i.e., retrieve information).

- WRITE: Indicates that the AIoT Command is Write (i.e., provision information).

- PERMANENT_DISABLE: Indicates that the AIoT Command is Permanent Disable (i.e., disable

the capability to transmit information).

#

# Data types describing alternative data types or combinations of data types

#

# *** End of Changes ***