3GPP TSG CT WG3 Meeting #143	C3-254038

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.548 | CR | 0058 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete the definition of the multi-modal flow type within the Synchronization policy |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | The definition of the SyncPolicy data structure needs to be completed by completing the definition of the encoding of the MultiModalFlowType data structure so that the related Editor's Note can be resolved. In order to cover all the use cases, it is proposed to define a new MultiModalFlowTypeInfo structured data type that contains 2 mutually-exclusive attributes representing 2 options, either a standardized list of values via the MultiModalFlowTypeInfo enumeration data type (with the 3 values that all the involved companies agreed on) or a generic string-encoded option to enable application-specific values. This way, there is no restriction in stage 3 on the values that the Multi-modal flow type can take and any evolutions in the related stage 1 or stage 2 requirements can be natively supported without the need to apply any updates in stage 3. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed necessary updates/corrections to complete the definition of the Synchronization Policy. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed necessary updates/corrections are not addressed and the Synchronization Policy definition is not complete in stage 3. |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.6.1, 6.5.6.2.14, 6.5.6.2.20 (new clause), 6.5.6.3.5, A.6 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces a backwards compatible new feature to the OpenAPI descriptions of the following APIs: - TS29548_SDD_PolicyConfiguration.yaml |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

#### 6.5.6.1	General

This clause specifies the application data model supported by the API.

Table 6.5.6.1-1 specifies the data types defined for the SDD_PolicyConfiguration API.

Table 6.5.6.1-1: SDD_PolicyConfiguration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AlignmentPolicy | 6.5.6.2.15 | Represents the multi-modal flows alignment policy. | XRMApp |
| BdwCtrlPolicy | 6.5.6.3.3 | Represents the bandwidth control policy. |  |
| GeofencingArea | 6.5.6.2.8 | Represents the geofencing area. | SEALDD_2 |
| GeofencingPolicy | 6.5.6.2.7 | Represents the geofencing policy. | SEALDD_2 |
| GeoPolAction | 6.5.6.3.4 | Represents the geofencing policy action. | SEALDD_2 |
| MultiModalFlowType | 6.5.6.3.5 | Represents the Muti-modal flow type. | XRMApp |
| MultiModalFlowTypeInfo | 6.5.6.2.20 | Represents the Muti-modal flow type information. | XRMApp |
| MultiModalSealddPolicy | 6.5.6.2.9 | Represents the Multi-modal SEALDD Policy. | XRMApp |
| Non3gppAccessMeasPol | 6.5.6.2.11 | Represents the non-3GPP access measurement policy. | SEALDD_2 |
| QualGuarPolicy | 6.5.6.2.5 | Represents the quality guarantee policy. |  |
| QualGuarThresh | 6.5.6.2.6 | Represents the quality guarantee related thresholds. |  |
| PolicyConfig | 6.5.6.2.2 | Represents a SEALDD Policy Configuration. |  |
| PolicyConfigPatch | 6.5.6.2.3 | Represents the parameters to request the modification of a SEALDD Policy Configuration. |  |
| ProximityThresholds | 6.5.6.2.17 | Represents the proximity thresholds for entering/leaving the UE-to-UE direct communication mode. | XRMApp |
| QoEThresholds | 6.5.6.2.19 | Represents the QoE thresholds for entering/leaving the UE-to-UE direct communication mode. | XRMApp |
| QoSThresholds | 6.5.6.2.18 | Represents the QoS thresholds for entering/leaving the UE-to-UE direct communication mode. | XRMApp |
| SealddPolicy | 6.5.6.2.4 | Represents a SEALDD Policy. |  |
| SignalStrength | 6.5.6.2.12 | Represents the signal strength value. | SEALDD_2 |
| SignalStrengthThreshold | 6.5.6.2.13 | Represents the signal strength threshold. | SEALDD_2 |
| SyncPolicy | 6.5.6.2.14 | Represents the multi-modal synchronization policy. | XRMApp |
| TempPolicy | 6.5.6.2.10 | Represents the temporal policy. | SEALDD_2 |
| UEToUEPolicy | 6.5.6.2.16 | Represents the multi-modal UE-to-UE policy. | XRMApp |

Table 6.5.6.1-2 specifies data types re-used by the SDD_PolicyConfiguration API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SDD_PolicyConfiguration API.

Table 6.5.6.1-2: SDD_PolicyConfiguration API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BitRate | 3GPP TS 29.571 [18] | Represents a bit rate. | XRMApp |
| DateTimeRo | 3GPP TS 29.122 [2] | Represents a date and a time with the "read-only" property. |  |
| DurationMilliSec | 3GPP TS 29.514 [19] | Represents a time duration in units of milliseconds. | XRMApp |
| GeographicArea | 3GPP TS 29.572 [22] | Represents a geographical area. | SEALDD_2 |
| MatchingDirection | 3GPP TS 29.520 [20] | Represents a threshold matching direction. | SEALDD_2 |
| MeasurementId | Clause 6.4.6.3.3 | Represents the transmission quality measurement type. |  |
| MultiModalId | 3GPP TS 29.514 [19] | Represents the identifier of a multi-modal service. | XRMApp |
| PacketErrRate | 3GPP TS 29.571 [18] | Represents the packet error rate. | XRMApp |
| PacketLossRate | 3GPP TS 29.571 [18] | Represents the packet loss rate. | XRMApp |
| ScheduledCommunicationTime | 3GPP TS 29.122 [2] | Represents a time schedule. | SEALDD_2 |
| Snssai | 3GPP TS 29.571 [18] | Represents a S-NSSAI. | XRMApp |
| SupportedFeatures | 3GPP TS 29.571 [18] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| ThresholdHandlingMode | 3GPP TS 29.549 [15] | Represents threshold handling mode. | SEALDD_2 |
| TransQualMeasCriteria | Clause 6.4.6.2.7 | Represents the transmission quality measurement reporting criteria. |  |
| TimeWindow | 3GPP TS 29.122 [2] | Represents a time window. | SEALDD_2 |
| Uint32 | 3GPP TS 29.571 [18] | Represents an unsigned 32-bit integer. | XRMApp |
| Uinteger | 3GPP TS 29.571 [18] | Represents an unsigned integer. | XRMApp |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |
| ValTargetUe | 3GPP TS 29.549 [15] | Represents the identifier of the targeted VAL UE or VAL user. |  |

* * * * Next changes * * * *

##### 6.5.6.2.14	Type: SyncPolicy

Table 6.5.6.2.14-1: Definition of type SyncPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flowType1 | MultiModalFlowTypeInfo | M | 1 | Contains the first multi-modal flow type. (NOTE) |  |
| flowType2 | MultiModalFlowTypeInfo | M | 1 | Contains the second multi-modal flow type. (NOTE) |  |
| syncThreshold | DurationMilliSec | M | 1 | Contains the maximum tolerable time delay, expressed in milliseconds, by which the multi-modal flow type identified by the "flowType1" attribute can be delayed compared to the multi-modal flow type identified by the "flowType2" attribute of the application, such that they can be perceived as being synchronous. |  |
| NOTE: These attributes shall not be set to the same value. |  |  |  |  |  |

* * * * Next changes * * * *

##### 6.6.6.2.20	Type: MultiModalFlowTypeInfo

Table 6.4.6.2.20-1: Definition of type MultiModalFlowTypeInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| type | MultiModalFlowType | C | 0..1 | Contains the Multi-modal flow type. |  |
| custType | string | C | 0..1 | Contains the customized Multi-modal flow type. |  |
| NOTE: These attributes are mutually exclusive and only one of them shall be present. |  |  |  |  |  |

* * * * Next changes * * * *

##### 6.5.6.3.5	Enumeration: MultiModalFlowType

The enumeration MultiModalFlowType represents the Multi-modal flow type. It shall comply with the provisions defined in table 6.5.6.3.5-1.

Table 6.5.6.3.5-1: Enumeration MultiModalFlowType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AUDIO | Indicates that the Multi-modal flow type is audio. |  |
| VIDEO | Indicates that the Multi-modal flow type is video. |  |
| VOICE | Indicates that the Multi-modal flow type is voice. |  |

* * * * Next changes * * * *

# A.6	SDD_PolicyConfiguration API

openapi: 3.0.0

info:

title: SEALDD Server Policy Configuration Service

version: 1.1.0-alpha.3

description: |

SEALDD Server Policy Configuration Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.548 V19.4.0; Service Enabler Architecture Layer for Verticals (SEAL);

SEAL Data Delivery (SEALDD) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.548/

servers:

- url: '{apiRoot}/sdd-pc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/configurations:

post:

summary: Request the creation of a Policy Configuration.

operationId: CreatePolicyConfig

tags:

- Policy Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'201':

description: >

Created. The Policy Configuration is successfully created and a representation of

the created Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

headers:

Location:

description: >

Contains the URI of the created Individual Policy Configuration resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/configurations/{configId}:

parameters:

- name: configId

in: path

description: >

Represents the identifier of the Individual Policy Configuration resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Policy Configuration resource.

operationId: GetIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'200':

description: >

OK. The requested Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Policy Configuration resource.

operationId: UpdateIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Policy Configuration resource.

operationId: ModifyIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PolicyConfigPatch'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully modified and

no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Policy Configuration resource.

operationId: DeleteIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

PolicyConfig:

description: >

Represents a SEALDD Policy Configuration.

type: object

properties:

appTrafficIds:

type: array

items:

type: string

minItems: 1

valTargetId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

sliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

multimodsddPol:

$ref: '#/components/schemas/MultiModalSealddPolicy'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- appTrafficIds

anyOf:

- required: [sealddPol]

- required: [multimodsddPol]

PolicyConfigPatch:

description: >

Represents the requested modifications to a SEALDD Policy Configuration.

type: object

properties:

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

multimodsddPol:

$ref: '#/components/schemas/MultiModalSealddPolicy'

SealddPolicy:

description: >

Represents a SEALDD Policy.

type: object

properties:

qualGuarPol:

$ref: '#/components/schemas/QualGuarPolicy'

qualOptPol:

$ref: '#/components/schemas/QualGuarPolicy'

bdwCtrlSets:

type: array

items:

$ref: '#/components/schemas/BdwCtrlPolicy'

minItems: 1

geoPol:

type: array

items:

$ref: '#/components/schemas/GeofencingPolicy'

minItems: 1

tempPol:

$ref: '#/components/schemas/TempPolicy'

non3gppAccMeasPol:

$ref: '#/components/schemas/Non3gppAccessMeasPol'

anyOf:

- required: [qualGuarPol]

- required: [qualOptPol]

- required: [bdwCtrlSets]

- required: [geoPol]

- required: [tempPol]

- required: [non3gppAccMeasPol]

QualGuarPolicy:

description: >

Represents the quality guarantee policy.

type: object

properties:

thresholds:

$ref: '#/components/schemas/QualGuarThresh'

required:

- thresholds

QualGuarThresh:

description: >

Represents the quality guarantee related thresholds.

type: object

properties:

measId:

type: array

items:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/MeasurementId'

minItems: 1

measThesh:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/TransQualMeasCriteria'

required:

- measId

- measThesh

GeofencingPolicy:

description: Represents a geofencing policy.

type: object

properties:

valServiceId:

type: string

area:

$ref: '#/components/schemas/GeofencingArea'

action:

$ref: '#/components/schemas/GeoPolAction'

required:

- valServiceId

- area

- action

GeofencingArea:

description: Represents a geofencing area.

type: object

properties:

geoAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

anyOf:

- required: [geoAreas]

MultiModalSealddPolicy:

description: >

Represents a Multi-modal SEALDD Policy.

type: object

properties:

syncPol:

type: array

items:

$ref: '#/components/schemas/SyncPolicy'

minItems: 1

description: >

Contains the synchronization policy.

The key of the map shall be set to the multi-modal flow type provided within the

"flowType" attribute of the corresponding map value encoded using the SyncPolicy data

structure.

alignmentPol:

$ref: '#/components/schemas/AlignmentPolicy'

ueToUePol:

$ref: '#/components/schemas/UEToUEPolicy'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

anyOf:

- required: [syncPol]

- required: [alignmentPol]

- required: [ueToUePol]

TempPolicy:

description: >

Represents a temporal policy.

type: object

properties:

schedule:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

oneOf:

- required: [schedule]

- required: [timeWindow]

Non3gppAccessMeasPol:

description: >

Represents the non-3GPP access measurement policy.

type: object

properties:

accessIds:

type: array

items:

type: string

minItems: 1

thresholds:

type: array

items:

$ref: '#/components/schemas/SignalStrengthThreshold'

minItems: 1

thrHandling:

$ref: 'TS29549_SS_NetworkResourceMonitoring.yaml#/components/schemas/ThresholdHandlingMode'

timeThr:

$ref: '#/components/schemas/TempPolicy'

anyOf:

- required: [accessIds]

- required: [thresholds]

- required: [timeThr]

SignalStrength:

description: >

Represents the signal strength value.

type: object

properties:

rssi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

anyOf:

- required: [rssi]

SignalStrengthThreshold:

description: >

Represents the signal strength threshold.

type: object

properties:

value:

$ref: '#/components/schemas/SignalStrength'

direction:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- value

- direction

SyncPolicy:

description: >

Represents the synchronization policy.

type: object

properties:

flowType1:

$ref: '#/components/schemas/MultiModalFlowTypeInfo'

flowType2:

$ref: '#/components/schemas/MultiModalFlowTypeInfo'

syncThreshold:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/DurationMilliSec'

required:

- flowType1

- flowType2

- syncThreshold

AlignmentPolicy:

description: >

Represents the multi-modal flows alignment policy.

type: object

properties:

multiModalId:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/MultiModalId'

maxAlignTime:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/DurationMilliSec'

anyOf:

- required: [multiModalId]

- required: [maxAlignTime]

UEToUEPolicy:

description: >

Represents the multi-modal UE-to-UE policy.

type: object

properties:

proxThresh:

$ref: '#/components/schemas/ProximityThresholds'

qoSThresh:

$ref: '#/components/schemas/QoSThresholds'

qoEThresh:

$ref: '#/components/schemas/QoEThresholds'

anyOf:

- required: [proxThresh]

- required: [qoSThresh]

- required: [qoEThresh]

ProximityThresholds:

description: >

Represents the proximity thresholds for entering/leaving the UE-to-UE direct communication

mode.

type: object

properties:

minUeToUedist:

type: number

format: double

minimum: 0

avgUeToUedist:

type: number

format: double

minimum: 0

maxUeToUedist:

type: number

format: double

minimum: 0

anyOf:

- required: [minUeToUedist]

- required: [avgUeToUedist]

- required: [maxUeToUedist]

QoSThresholds:

description: >

Represents the QoS thresholds for entering/leaving the UE-to-UE direct communication mode.

type: object

properties:

minLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

avgBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

avgPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

maxPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

minPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

avgPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

maxPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

minJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

avgJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

anyOf:

- required: [minLatency]

- required: [avgLatency]

- required: [maxLatency]

- required: [minBitRate]

- required: [avgBitRate]

- required: [maxBitRate]

- required: [minPackLossRate]

- required: [avgPackLossRate]

- required: [maxPackLossRate]

- required: [minPackErrRate]

- required: [avgPackErrRate]

- required: [maxPackErrRate]

- required: [minJitter]

- required: [avgJitter]

- required: [maxJitter]

QoEThresholds:

description: >

Represents the QoE thresholds for entering/leaving the UE-to-UE direct communication mode.

type: object

properties:

mos:

type: integer

minimum: 1

maximum: 5

anyOf:

- required: [mos]

MultiModalFlowTypeInfo:

description: >

Represents the Muti-modal flow type information.

type: object

properties:

type:

$ref: '#/components/schemas/MultiModalFlowType'

custType:

type: string

oneOf:

- required: [type]

- required: [custType]

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

BdwCtrlPolicy:

anyOf:

- type: string

enum:

- REALLOCATE_DL

- REALLOCATE_UL

- NOT_REALLOCATE_DL

- NOT_REALLOCATE_UL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the bandwidth control policy.

Possible values are:

- REALLOCATE_DL: Indicates that the bandwidth control action is to reallocate the

bandwidth limit between different VAL users for DL traffic.

- REALLOCATE_UL: Indicates that the bandwidth control action is to reallocate the

bandwidth limit between different VAL users for UL traffic.

- NOT_REALLOCATE_DL: Indicates that the bandwidth control action is to not reallocate

the bandwidth limit between different VAL users for DL traffic.

- NOT_REALLOCATE_UL: Indicates that the bandwidth control action is to not reallocate

the bandwidth limit between different VAL users for UL traffic.

GeoPolAction:

anyOf:

- type: string

enum:

- ALLOW

- BLOCK

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the geofencing policy action.

Possible values are:

- ALLOW: Indicates that the geofencing policy is to allow.

- BLOCK: Indicates that the geofencing policy is to block.

MultiModalFlowType:

anyOf:

- type: string

enum:

- AUDIO

- VIDEO

- VOICE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the Multi-modal flow type.

Possible values are:

- AUDIO: Indicates that the Multi-modal flow type is audio.

- VIDEO: Indicates that the Multi-modal flow type is video.

- VOICE: Indicates that the Multi-modal flow type is voice.

# Data types describing alternative data types or combinations of data types:

#

* * * * End of changes * * * *