3GPP TSG CT WG3 Meeting #143	C3-254044

# Sophia-Antipolis, FR, 13th – 17th October, 2025

Source:	Huawei

Title:	Pseudo-CR on various additional updates and corrections

Spec:	3GPP TS 29.566

Agenda item:	19.60 (EnergySys)

Document for:	Agreement

1. Introduction

There are still a few minor issues (e.g., editorials, respect of the drafting rules) that need to be corrected in the general and service description clauses of this new TS 29.566.

2. Reason for Change

Update the general and service description clauses of this new TS to apply these necessary corrections.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.566 V 1.1.0.

* * * * Start of changes * * * *

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Neif Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the EIF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [19].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

* * * * Next changes * * * *

# 4	Overview

In the frame of Energy Efficiency and Energy Saving functionalities and services, the Energy Information Function (EIF) provides services to NF service consumers (e.g., NEF, AF) via the Neif service-based interface. The EIF supports for this purpose the functionalities defined in 3GPP TS 23.502 [3] to manage Energy information and its exposure to applications.

Figures°4-1 and 4.2 depict the Energy related reference architecture of the EIF respectively in SBI representation and reference point representation.

Figure 4-1: Reference model for the EIF Services – SBI representation

Figure 4-2: Reference Model for the EIF Services – Reference point representation

* * * * Next changes * * * *

##### 5.2.2.2.2	Energy Event Exposure Subscription Creation

Figure 5.2.2.2.2-1 depicts a scenario where an NF service consumer requests to create an Energy Event Exposure Subscription (see also clause 4.29 of 3GPP TS 23.502 [3]).

Figure 5.2.2.2.2-1: Energy Event Exposure Subscription Creation

1.	To subscribe to energy consumption related event(s) reporting, the NF service consumer shall send an HTTP POST request targeting the URI of the "Energy Event Exposure Subscriptions" collection resource, with the request body including the EnergyEeSubsc data structure.

2a.	Upon success, the EIF shall respond to the NF service consumer with an HTTP "201 Created" status code with the response body containing the representation of the created resource within the EnergyEeSubsc data structure, and an HTTP "Location" header field containing the URI of the created "Individual Energy Event Exposure Subscription" resource.

2b.	On failure, the EIF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code.

* * * * Next changes * * * *

##### 5.2.2.2.3	Energy Event Exposure Subscription Update

Figure 5.2.2.2.3-1 depicts a scenario where an NF service consumer requests to update an existing Energy Event Exposure Subscription (see also clause 4.29 of 3GPP TS 23.502 [3]).

Figure 5.2.2.2.3-1: Energy Event Exposure Subscription Update

1.	In order to request the update of an existing subscription to energy consumption related event(s) reporting, the NF service consumer shall send an HTTP PUT/PATCH request to the EIF, targeting the URI of the corresponding "Individual Energy Event Exposure Subscription" resource, with the request body including either:

-	the updated representation of the resource within the EnergyEeSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the EnergyEeSubscPatch data structure, in case the HTTP PATCH method is used.

2a.	Upon success, the EIF shall update the targeted "Individual Energy Event Exposure Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Energy Event Exposure Subscription" resource within the EnergyEeSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the EIF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code.

* * * * Next changes * * * *

##### 5.2.2.4.2	Energy Event Exposure Notification

Figure 5.2.2.4.2-1 depicts a scenario where the EIF sends a request to notify a previously subscribed NF service consumer on energy consumption information related event(s) (see also clause 4.29 of 3GPP TS 23.502 [3]).

Figure 5.2.2.4.2-1: Energy Event Exposure Notification

1.	In order to notify a previously subscribed NF service consumer on energy consumption information related event(s), the EIF shall send an HTTP POST request to the NF service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the NF service consumer during the creation/update of the corresponding Energy Event Exposure Subscription using the procedures defined in clause 5.2.2.2, and the request body including the EnergyEeNotif data structure.

2a.	Upon success, the NF service consumer shall respond to the EIF with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the NF service consumer shall take proper error handling actions, as specified in clause 6.1.7, and respond to the EIF with an appropriate error status code.

* * * * Next changes * * * *

### 6.1.1	Introduction

The Neif_EventExposure Service shall use the Neif_EventExposure API.

The API URI of the Neif_EventExposure API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "neif-ee".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clauses 6.1.3 and 6.1.4.

* * * * Next changes * * * *

### 6.1.10	HTTP redirection

An HTTP request may be redirected to a different Neif_EventExposure service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different EIF producer instance will return the NF Instance ID of the new EIF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an EIF redirects a service request to a different EIF using an HTTP "307 Temporary Redirect" or "308 Permanent Redirect"’ status code, the identity of the new EIF towards which the service request is redirected shall be indicated in the "3gpp-Sbi-Target-Nf-Id"’ header of the HTTP "307 Temporary Redirect" or "308 Permanent Redirect" response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

* * * * Next changes * * * *

# B.2	Neif_EventExposure API

The API versions listed in table B.2-1 are withdrawn for the Neif_EventExposure API.

Table B.2-1: Withdrawn API versions of the Neif_EventExposure service

| API version number | Remarks |
| --- | --- |

* * * * Next changes * * * *

Annex C (normative):
ABNF grammar for 3GPP SBI HTTP custom headers

# C.1	General

This Annex contains a self-contained set of ABNF rules, comprising the re-used rules from IETF RFCs, and the rules defined by the 3GPP custom headers defined in this specification (see clause 6.1.2.3).

This grammar may be used as input to existing tools to help implementations to parse 3GPP custom headers.

* * * * End of changes * * * *