3GPP TSG CT WG3 Meeting #143	C3-254045

# Sophia-Antipolis, FR, 13th – 17th October, 2025

Source:	Huawei

Title:	Pseudo-CR on updates and corrections to the API definition clauses

Spec:	3GPP TS 29.566

Agenda item:	19.60 (EnergySys)

Document for:	Agreement

1. Introduction

The following issues have been identified in the definition of the API definition clauses of the new Neif_EventExposure API defined in this also new TS 29.566:

-	The data model still needs to be further refined to complete the support of subscribing to multiple Energy related events via the same subscription and thus also notifying with potentially several per-event reports.

-	In order to achieve this and also to align with the upstream Nnef_EventExposure API, there should be only a single enumeration data type with the full list of events, which is fully aligned with the related stage 2 requirements. Having a single event with a list of granularities is not a good approach especially from extensibility point of view, e.g., each time there is a new set of events, we would need to define new sets of attributes and the related data types for them, instead of simply extending a single enumeration data type.

-	Moreover, and though it is a separate interface, having a single event with a list of granularities means that the NEF will need to do an additional mapping work between the events subscribed by the AF and the event + granularity to be used to create the corresponding subscription at the EIF, and the same would apply in the opposite direction for notifications. The NEF can do such mapping/translation work, but if we can avoid it with a simpler solution, it is highly preferable, especially that the NEF may expose several APIs with many of them requiring unavoidable translation work (e.g., mapping from internal to external UE IDs, mapping from external location information to network internal location information).

-	Various issues, e.g., missing clauses in the data model part, drafting rules related issues, remaining unnecessary text from the SBI TS Skeleton, editorial corrections, also need to be addressed.

2. Reason for Change

Address the above-detailed issues to complete the definition of the API definition clauses of the new Neif_EventExposure API.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.566 V 1.1.0.

* * * * Start of changes * * * *

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Neif_EventExposure API.

Figure 6.1.3.1-1: Resource URI structure of the Neif_EventExposure API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods for the Neif_EventExposure API.

.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Energy Event Exposure Subscriptions | /subscriptions | POST | Create a new Energy Event Exposure Subscription. |
|  |  | GET | Retrieve all the Energy Event Exposure Subscription(s) managed by the EIF. |
| Individual Energy Event Exposure Subscription | /subscriptions/{subId} | GET | Retrieve an existing "Individual Energy Event Exposure Subscription" resource. |
|  |  | PUT | Update an existing "Individual Energy Event Exposure Subscription" resource. |
|  |  | PATCH | Modify an existing "Individual Energy Event Exposure Subscription" resource. |
|  |  | DELETE | Delete an existing "Individual Energy Event Exposure Subscription" resource. |

* * * * Next changes * * * *

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/neif-ee/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |

* * * * Next changes * * * *

###### 6.1.3.2.3.1	POST

The HTTP POST method enables an NF service consumer to request the creation of an Energy Event Exposure Subscription at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.2.3.2-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.1.3.2.3.2-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | Represents the parameters to request the creation of a new Energy Event Exposure Subscription. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | 201 Created | Successful case. The Energy Event Exposure Subscription is successfully created and a representation of the created "Individual Energy Event Exposure Subscription" resource shall be returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/neif-ee/<apiVersion>/subscriptions/{subId} |

* * * * Next changes * * * *

###### 6.1.3.2.3.2	GET

The HTTP GET method enables an NF service consumer to retrieve all the Energy Event Exposure Subscription(s) managed by the EIF.

This method shall support the URI query parameters specified in table 6.1.3.2.3.2-1.

Table 6.1.3.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.1.3.2.3.2-3.

Table 6.1.3.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(EnergyEeSubsc) | M | 0..N | 200 OK | Successful case. All the active "Individual Energy Event Exposure Subscription" resource(s) managed by the EIF are returned. If there are no active "Individual Energy Event Exposure Subscription" resource(s) at the EIF, an empty array is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target EIF (service) instance towards which the request is redirected. |

Table 6.1.3.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target EIF (service) instance towards which the request is redirected. |

* * * * Next changes * * * *

###### 6.1.3.3.3.1	GET

The HTTP GET method enables an NF service consumer to retrieve an existing "Individual Energy Event Exposure Subscription" resource at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Energy Event Exposure Subscription" resource is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected |

* * * * Next changes * * * *

###### 6.1.3.3.3.2	PUT

The HTTP PUT method enables an NF service consumer to request the update of an existing "Individual Energy Event Exposure Subscription" resource at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | Represents the updated "Individual Energy Event Exposure Subscription" resource representation. |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | 200 OK | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

* * * * Next changes * * * *

###### 6.1.3.3.3.3	PATCH

The HTTP PUT method enables an NF service consumer to request the modification of an existing "Individual Energy Event Exposure Subscription" resource at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.3-1.

Table 6.1.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.3.3.3-3.

Table 6.1.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EnergyEeSubscPatch | M | 1 | Contains the parameters to request the modification of the "Individual Energy Event Exposure Subscription" resource. |

Table 6.1.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | 200 OK | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

* * * * Next changes * * * *

###### 6.1.3.3.3.4	DELETE

The HTTP PUT method enables an NF service consumer to request the deletion of an existing "Individual Energy Event Exposure Subscription" resource at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.4-1.

Table 6.1.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.3.3.3.4-3.

Table 6.1.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

* * * * Next changes * * * *

##### 6.1.5.2.1	Description

The Energy Event Exposure Notification is used by the EIF to notify a previously subscribed NF service consumer on energy consumption information related event(s).

* * * * Next changes * * * *

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-2.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EnergyEeNotif | M | 1 | Represents the Energy Event Exposure Notification. |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Energy Event Exposure Notification is successfully received and acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF service consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF service consumer (service) instance towards which the notification request is redirected. |

Table 6.1.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF service consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF service consumer (service) instance towards which the notification request is redirected. |

* * * * Next changes * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Neif_EventExposure service-based interface protocol.

Table 6.1.6.1-1: Neif_EventExposure specific Data Type

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EnergyEeEvent | 6.1.6.3.3 | Represents the Energy related event. |  |
| EnergyEeNotif | 6.1.6.2.4 | Represents the Energy Event Exposure Notification. |  |
| EnergyEeReport | 6.1.6.2.6 | Represents the Energy Event Exposure report. |  |
| EnergyEeSubsc | 6.1.6.2.2 | Represents the Energy Event Exposure Subscription. |  |
| EnergyEeSubscPatch | 6.1.6.2.3 | Represents the requested modifications to an Energy Event Exposure Subscription. |  |
| EnergyEeSubscSet | 6.1.6.2.5 | Represents the Energy Event Exposure Subscription set. |  |

Table 6.1.6.1-2 specifies data types re-used by the Neif_EventExposure service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Neif_EventExposure service-based interface.

Table 6.1.6.1-2: Neif_EventExposure re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [14] | Represents the application identifier. |  |
| DateTime | 3GPP TS 29.571 [14] | Represents a date and a time. |  |
| Dnn | 3GPP TS 29.571 [14] | Represents a DNN. |  |
| DurationSec | 3GPP TS 29.571 [14] | Represents a time duration in units of seconds. |  |
| EnergyInfo | 3GPP TS 29.122 [17] | Represents the Energy consumption information. |  |
| FlowDescription | 3GPP TS 29.514 [16] | Represents an IP flow description. |  |
| Gpsi | 3GPP TS 29.571 [14] | Represents a GPSI. |  |
| RedirectResponse | 3GPP TS 29.571 [14] | Represents redirection related information. |  |
| ReportingInformation | 3GPP TS 29.523 [18] | Represents the reporting requirements. |  |
| Snssai | 3GPP TS 29.571 [14] | Represents an S-NSSAI. |  |
| Supi | 3GPP TS 29.571 [14] | Represents a SUPI. |  |
| SupportedFeatures | 3GPP TS 29.571 [14] | Represents the list of supported feature(s) and is used to negotiate the applicability of optional features. |  |
| TimeWindow | 3GPP TS 29.122 [17] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [14] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [14] | Represents a URI. |  |

* * * * Next changes * * * *

##### 6.1.6.2.2	Type: EnergyEeSubsc

Table 6.1.6.2.2-1: Definition of type EnergyEeSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Contains the URI via which Energy consumption information related event(s) reporting shall be delivered. |  |
| repReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| eventsSubscSets | map(EnergyEeSubscSet) | M | 1..N | Contains the set(s) of Energy Event Exposure subscription related details. The key of the map shall be set to the value of the "event" attribute of the corresponding map value encoded using the EnergyEeSubscSet data structure. |  |
| reports | array(EnergyEeReport) | C | 1..N | Contains the Energy Event Exposure report(s). This attribute shall be present only when immediate reporting was requested in the corresponding subscription request and there are available report(s). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features used as defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |

* * * * Next changes * * * *

##### 6.1.6.2.3	Type: EnergyEeSubscPatch

Table 6.1.6.2.3-1: Definition of type EnergyEeSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which Energy consumption information related event(s) reporting shall be delivered. |  |
| repReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| eventsSubscSets | maps(EnergyEeSubscSet) | O | 1..N | Contains the set(s) of Energy Event Exposure subscription related details. The key of the map shall be set to the value of the "event" attribute of the corresponding map value encoded using the EnergyEeSubscSet data structure. |  |

* * * * Next changes * * * *

##### 6.1.6.2.5	Type: EnergyEeSubscSet

Table 6.1.6.2.5-1: Definition of type  EnergyEeSubscSet

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | EnergyEeEvent | M | 1 | Contains the subscribed Energy related event. |  |
| subscSetId | string | M | 1 | Contains the identifier of the Energy event exposure subscription set. |  |
| supi | Supi | C | 0..1 | Contains the target SUPI. (NOTE 1) |  |
| gpsi | Gpsi | C | 0..1 | Contains the target GPSI. (NOTE 1) |  |
| dnn | Dnn | C | 0..1 | Contains the target DNN. (NOTE 2) |  |
| snssai | Snssai | C | 0..1 | Contains the target S-NSSAI. (NOTE 2, NOTE 4) |  |
| appId | ApplicationId | C | 0..1 | Contains the identifier of the target application. (NOTE 3) |  |
| flowDescs | array(FlowDescription) | C | 1..N | Contains the service flow information description for a target application. (NOTE 3) |  |
| repTimePeriod | TimeWindow | O | 0..1 | Contains the reporting time period. (NOTE 6) |  |
| enrgRepThres | EnergyInfo | O | 0..1 | Contains the reporting thresholds for the Energy consumption information event exposure. This attribute shall be present in case threshold-based reporting is requested. |  |
| repReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription set. The "repPeriod" within the ReportingInformation data structure shall be present in case periodic reporting is requested. (NOTE 5) |  |
| NOTE 1: These attributes are mutually exclusive and only one of them shall be present. NOTE 2: If the value of the "event" attribute is set to "PDU_SESSION_ENERGY" or "SERVICE_FLOW_ENERGY", then at least one of these attributes shall be present. NOTE 3: If the value of the "event" attribute is set to "SERVICE_FLOW_ENERGY", then at least one of these attributes shall be present. NOTE 4: if the value of the "event" attribute is set to "UE_SNSSAI_ENERGY", then this attribute shall be present. NOTE 5: When this attribute is present, the event reporting requirements provided within this attribute shall take precedence over the common events reporting requirements provided within the "repReqs" attribute of the parent EnergyEeSubsc data structure. NOTE 6: In this Release of the specification, the values of both the start time and end time within the "repTimePeriod" attribute shall be in the future. |  |  |  |  |  |

* * * * Next changes * * * *

##### 6.1.6.2.6	Type: EnergyEeReport

Table 6.1.6.2.6-1: Definition of type EnergyEeReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | EnergyEeEvent | M | 1 | Contains the reported Energy related event. |  |
| subscSetId | string | M | 1 | Contains the identifier of the Energy event exposure subscription set to which this report is related. |  |
| timeStamp | DateTime | M | 1 | Contains the time at which the event is observed. |  |
| timeWindow | TimeWindow | O | 0..1 | Contains the time window representing a start time and a stop time of the reported data collection period. |  |
| energyInfo | EnergyInfo | O | 0..1 | Contains the energy consumption information. |  |

* * * * Next changes * * * *

##### 6.1.6.3.3	Enumeration: EnergyEeEvent

The enumeration EnergyEeEvent represents the Energy event. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration EnergyEeEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE_ENERGY | Indicates that the Energy Exposure event is the total energy consumed by a UE. |  |
| PDU_SESSION_ENERGY | Indicates that the Energy Exposure event is the total energy consumed by a PDU session of a UE. |  |
| SERVICE_FLOW_ENERGY | Indicates that the Energy Exposure event is the total energy consumed by an application or service data flow of an application traffic of a UE. |  |
| UE_SNSSAI_ENERGY | Indicates that the Energy Exposure event is the total energy consumed by a UE in an S-NSSAI. |  |

* * * * Next changes * * * *

* * * * Next changes * * * *

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

* * * * Next changes * * * *

#### 6.1.6.5	Binary data

##### 6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

* * * * End of changes * * * *