3GPP TSG CT WG3 Meeting #143	C3-254046

# Sophia-Antipolis, FR, 13th – 17th October, 2025

Source:	Huawei

Title:	Pseudo-CR on defining the OpenAPI description

Spec:	3GPP TS 29.566

Agenda item:	19.60 (EnergySys)

Document for:	Agreement

1. Introduction

The OpenAPI description of the new Neif_EventExposure API defined in this new TS 29.566 has not yet been defined.

2. Reason for Change

Define the OpenAPI description of the new Neif_EventExposure API.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.566 V 1.1.0.

* * * * Start of changes * * * *

# A.2	Neif_EventExposure API

openapi: 3.0.0

info:

title: EIF Event Exposure Service

version: 1.0.0-alpha.1

description: |

API for the Neif_EventExposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.566 V1.2.0; 5G System; Energy Information Function Services; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.566/

servers:

- url: '{apiRoot}/neif-ee/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- neif-ee

paths:

/subscriptions:

get:

summary: Retrieve all the active Energy Event Exposure Subscription(s) managed by the EIF.

operationId: GetEnergyEESubscs

tags:

- Energy Event Exposure Subscriptions (Collection)

responses:

'200':

description: >

OK. All the active Individual Energy Event Exposure Subscription resource(s) managed by

the EIF are returned.

If there are no active Individual Energy Event Exposure Subscription resource(s) at the

EIF, an empty array is returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/EnergyEeSubsc'

minItems: 0

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of an Energy Event Exposure Subscription.

operationId: CreateEnergyEESubsc

tags:

- Energy Event Exposure Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

responses:

'201':

description: >

Created. The Energy Event Exposure Subscription is successfully created and a

representation of the created Individual Energy Event Exposure Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual Energy Event Exposure Subscription

resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

EnergyEENotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeNotif'

responses:

'204':

description: >

No Content. The Energy Event Exposure notification is successfully

received and acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subId}:

parameters:

- name: subId

in: path

description: >

Represents the identifier of the Individual Energy Event Exposure Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Energy Event Exposure Subscription resource.

operationId: GetIndEnergyEESubsc

tags:

- Individual Energy Event Exposure Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Energy Event Exposure Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Energy Event Exposure Subscription resource.

operationId: UpdateIndEnergyEESubsc

tags:

- Individual Energy Event Exposure Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

responses:

'200':

description: >

OK. The Individual Energy Event Exposure Subscription resource is successfully updated

and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

'204':

description: >

No Content. The Individual Energy Event Exposure Subscription resource is successfully

updated and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Energy Event Exposure Subscription resource.

operationId: ModifyIndEnergyEESubsc

tags:

- Individual Energy Event Exposure Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/EnergyEeSubscPatch'

responses:

'200':

description: >

OK. The Individual Energy Event Exposure Subscription resource is successfully modified

and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

'204':

description: >

No Content. The Individual Energy Event Exposure Subscription resource is successfully

modified and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Energy Event Exposure Subscription resource.

operationId: DeleteIndEnergyEESubsc

tags:

- Individual Energy Event Exposure Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Energy Event Exposure Subscription resource is

successfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

neif-ee: >

Enables to access all the resources and custom operations of the Neif_EventExposure

API.

schemas:

#

# STRUCTURED DATA TYPES

#

EnergyEeSubsc:

description: >

Represents an Energy Event Exposure Subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

repReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

eventsSubscSets:

type: object

additionalProperties:

$ref: '#/components/schemas/EnergyEeSubscSet'

minProperties: 1

description: >

Contains the set(s) of Energy Event Exposure subscription related details.

The key of the map shall be set to the value of the event attribute of the

corresponding map value encoded using the EnergyEeSubscSet data structure.

reports:

type: array

items:

$ref: '#/components/schemas/EnergyEeReport'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- eventsSubscSets

EnergyEeSubscPatch:

description: >

Represents the requested modifications to an Energy Event Exposure Subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

repReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

eventsSubscSets:

type: object

additionalProperties:

$ref: '#/components/schemas/EnergyEeSubscSet'

minProperties: 1

description: >

Contains the set(s) of Energy Event Exposure subscription related details.

The key of the map shall be set to the value of the event attribute of the

corresponding map value encoded using the EnergyEeSubscSet data structure.

EnergyEeNotif:

description: >

Represents the Energy Event Exposure Notification.

type: object

properties:

subId:

type: string

reports:

type: array

items:

$ref: '#/components/schemas/EnergyEeReport'

minItems: 1

required:

- subId

- reports

EnergyEeSubscSet:

description: >

Represents the Energy Event Exposure Subscription set.

type: object

properties:

event:

$ref: '#/components/schemas/EnergyEeEvent'

subscSetId:

type: string

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

flowDescs:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

repTimePeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

enrgRepThres:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/EnergyInfo'

repReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

required:

- event

- subscSetId

oneOf:

- required: [supi]

- required: [gpsi]

EnergyEeReport:

description: >

Represents the Energy Event Exposure report.

type: object

properties:

event:

$ref: '#/components/schemas/EnergyEeEvent'

subscSetId:

type: string

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

energyInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/EnergyInfo'

required:

- event

- subscSetId

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

EnergyEeEvent:

anyOf:

- type: string

enum:

- UE_ENERGY

- PDU_SESSION_ENERGY

- SERVICE_FLOW_ENERGY

- UE_SNSSAI_ENERGY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the Energy event.

Possible values are:

- UE_ENERGY: Indicates that the Energy Exposure event is the total energy consumed by a UE.

- PDU_SESSION_ENERGY: Indicates that the Energy Exposure event is the total energy consumed

by a PDU session of a UE.

- SERVICE_FLOW_ENERGY: Indicates that the Energy Exposure event is the total energy consumed

by an application or service data flow of an application traffic of a UE.

- UE_SNSSAI_ENERGY: Indicates that the Energy Exposure event is the total energy consumed by

a UE in an S-NSSAI.

# Data types describing alternative data types or combinations of data types:

#

* * * * End of changes * * * *