3GPP TSG CT WG3 Meeting #143	C3-254050

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.508 | CR | 0363 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates and corrections to Energy Consumption information exposure |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | The following issues were identified in the provisions related to the Energy Consumption information exposure: - Some provisions in the service description clauses were either not put in the right place or misplaced after the introduction of new provisions related to other Rel-19 features. - The "dataVolInfos" attribute needs to be removed from Table 5.6.2.5 as it was replaced by the "dataVolInfoData" attribute (wrong implementation of the CRs agreed during the last plenary cycle). The OpenAPI description is correct with regards to this. - NOTE 16 in clause 4.2.2.2 is changed to normative text to align with the latest updates in the related stage 2 requirements. - Various terminology alignments and text enhancements are needed to avoid confusion and enhance the clarity of the provisions. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed issues. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed issues are not addressed and the definition of the Energy management functionality related requirements/provisions is not corrected/completed. |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2, 4.2.3.2, 4.2.4.2, 5.3.3.3.3, 5.6.2.2, 5.6.2.5, 5.6.2.16, 5.6.3.3, A.2 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the OpenAPI descriptions of the following APIs: - TS29508_Npcf_EventExposure.yaml |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

#### 4.2.2.2	Notification about subscribed events

The present "notification about subscribed events" procedure is performed by the SMF when any of the subscribed events occur.

The following applies with respect to the detection of subscribed events:

-	If:

-	the SMF supports the "DownlinkDataDeliveryStatus" feature,

-	the event "DDDS" is subscribed,

-	the traffic descriptors of the downlink data source have been provided for that subscription, and

-	the SMF is informed that the UE corresponding to that subscription is unreachable,

-	if the data is buffered at the UPF, then the SMF shall interact with the UPF to notify that the UPF buffers the downlink packets. The SMF shall include the traffic descriptor of the subscriptions in the PDR with a higher priority if the PCC is not applied to the PDU session or derive the PDR from the PCC rule received from the PCF as defined in clause 4.2.4.27 of 3GPP TS 29.512 [14] if the PCC is applied to the PDU session and request the UPF to report when there are corresponding buffered downlink packets or discarded packets in the UPF as defined in clause 5.28.1 of 3GPP TS 29.244 [23]. When receiving the report from the UPF, the SMF shall determine whether that subscribed event with delivery status "DISCARDED" or "BUFFERED" occurred. The SMF shall determine that subscribed event with delivery status "TRANSMITTED" occurred by the fact that the related PDU session becomes ACTIVE.

-	if the data is buffered at the SMF, the SMF shall determine whether that subscribed event occurred by comparing the downlink packets with the traffic descriptors received in the corresponding event subscription. If the SMF decides to buffer the packets, the subscribed event with delivery status "BUFFERED" occurred. If the SMF decides to discard the packets, the subscribed event with delivery status "DISCARDED" occurred. The SMF shall determine that subscribed event with delivery status "TRANSMITTED" occurred by the fact that the related PDU session becomes ACTIVE.

Figure 4.2.2.2-1 illustrates the notification about subscribed events.

Figure 4.2.2.2-1: Notification about subscribed events

If the SMF observes PDU Session related event(s) for which an NF service consumer has subscribed, the SMF shall send an HTTP POST request with "{notifUri}", as previously provided by the NF service consumer within the corresponding subscription, as URI and NsmfEventExposureNotification data structure as request body that shall include:

-	Notification correlation ID provided by the NF service consumer during the subscription, or as provided by the PCF for implicit subscription of UP path change traffic correlation, and/or traffic routing requirement installation outcome, and/or simultaneous connectivity failure as defined in clause 4.2.6.2.6.2 of 3GPP TS 29.512 [14], or as provided by the PCF for implicit subscription of QoS Monitoring as defined in clause 4.2.3.25 of 3GPP TS 29.512 [14], or as provided by the V-NEF for implicit subscription of UP path change as defined in clause 4.4.2.4.2 of 3GPP TS 29.591 [28], as "notifId" attribute; and

-	information about the observed event(s) within the "eventNotifs" attribute that shall contain for each observed event an "EventNotification" data structure that shall include:

1.	the Event Trigger as "event" attribute, and if the "EnhEventMgmt" feature is supported and optionally, a reference identifier within the "referenceId" attribute;

2.	for a UP path change notification:

a)	type of notification ("EARLY" or "LATE") as "dnaiChgType" attribute;

b)	source DNAI and/or target DNAI as "sourceDnai" attribute and "targetDnai" attribute if DNAI is changed, respectively; and

c)	if the PDU Session type is IP, for the source DNAI IP address/prefix of the UE as "sourceUeIpv4Addr" attribute or "sourceUeIpv6Prefix" attribute; and

d)	if the PDU Session type is IP, for the target DNAI IP address/prefix of the UE as "targetUeIpv4Addr" attribute or "targetUeIpv6Prefix" attribute;

e)	if available, for the source DNAI, N6 traffic routing information related to the UE as "sourceTraRouting" attribute;

f)	if available, for the target DNAI, N6 traffic routing information related to the UE as "targetTraRouting" attribute;

g)	if the PDU Session type is Ethernet, the MAC address of the UE in the "ueMac" attribute;

h)	if the "CommonEASDNAI" feature is supported,

-	 the candidate DNAI(s) for the PDU Session in "candidateDnais" attribute, optionally together with the indication of their prioritization within the "candDnaisPrioInd" attribute, if the "candDnaiInd" attribute was set to "true" in the PCC rule(s); or

-	the indication of EAS re-discovery in "easRediscoverInd" attribute if EAS re-discovery took place.

NOTE 1:	In this release, when SMF acting as the I-SMF based on the Local Offloading Management does not cover common DNAI/EAS selection.

i)	if both the SMF and the NF service consumer support "ULBuffering" and/or "EASIPreplacement" features, these supported features within the "supportedFeatures" attribute.

NOTE 2:	The SMF gets the knowledge of the feature supported by the NF service consumer as described in clause 5.8.

j)	if the "EasRelocationEnh" feature is supported and the SMF determines that the target DNAI is supported by an AF different to the one that shall receive this notification, the identifier of the target AF that supports this DNAI in the "targetAfId" attribute.

NOTE 3:	The SMF can determine that the target DNAI is supported by an AF different to the one that shall receive this notification by comparing the AF ID of the EAS Deployment Information entry that contains the old DNAI with the AF ID of the EAS Deployment Information entry that contains the target DNAI. These EAS Deployment Information entries are received via the Nnef_EASDeployment API defined in 3GPP TS 29.591 [25].

k)	if the "HR-SBO" feature is supported and the SMF determines that the UE has moved to a serving PLMN in which local traffic offload is allowed, the identifier of this new serving PLMN within the "plmnId" attribute, as well as the DNN and S-SNSSAI of the HPLMN within the "dnn" and "snssai" attributes, respectively.

NOTE 4:	UP path change notification, i.e. DNAI change notification and/or N6 traffic routing information change notification, can be the result of an implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) via the Npcf_SMPolicyControl service (see clause 4.2.6.2.6.2 of 3GPP TS 29.512 [14]).

NOTE 5:	If the DNAI is not changed while the N6 traffic routing information change, the source DNAI and target DNAI are not provided.

NOTE 6:	The change from the UP path status where no DNAI applies to a status where a DNAI applies indicates the activation of the related AF request and therefore only the target DNAI and N6 traffic routing information is provided in the event notification; the change from the UP path status where a DNAI applies to a status where no DNAI applies indicates the de-activation of the related AF request and therefore only the source DNAI and N6 traffic routing information is provided in the event notification.

NOTE 7:	UP path change notification, i.e. DNAI change notification can be the result of an implicit subscription of the PCF on behalf of the AF as part of setting PCC rule(s) in case of UE-Satellite-UE communication in IMS via the Npcf_SMPolicyControl service (see clause 4.2.6.2.6.2 of 3GPP TS 29.512 [14]). The event that satellite identifier serving the UE has changed is represented by the event a DNAI has changed and the DNAI value can be derived from the satellite identifier as per operator policy and implementation.

3.	for a UE IP address change:

a)	added new UE IP address or prefix as "adIpv4Addr" attribute or "adIpv6Prefix" attribute, respectively; and/or

b)	released UE IP address or prefix as "reIpv4Addr" attribute or "reIpv6Prefix" attribute, respectively;

4.	for an access type change:

a)	new access type as "accType" attribute;

5.	for a PLMN Change:

a)	new PLMN as "plmnId" attribute;

6.	for a PDU Session Release:

a)	ID of the released PDU session as "pduSeId" attribute;

b)	DNN of the released PDU session as "dnn" attribute, if the "PduSessionStatus" feature is supported;

c)	the type of the released PDU session as "pduSessType" attribute, if the "PduSessionStatus" feature is supported;

d)	UE IPv4 address as "ipv4Addr" attribute and/or IPv6 information (IPv6 prefix(es) or IPv6 address(es)) as "ipv6Prefixes" or "ipv6Addrs" attributes, if the released PDU session type is IP and the "PduSessionStatus" feature is supported;

e)	S-NSSAI of the released PDU session as "snssai" attribute, if the "EneNA" feature is supported and "snssai" attribute is present in the subscribed "NsmfEventExposure" data type; and

f)	the current RAT Type of the released PDU Session, if available and the "EnPduSesRatType" feature is supported.

7.	the time at which the event was observed encoded as "timeStamp" attribute;

8.	the SUPI as the "supi" attribute if the subscription applies to a group of UE(s) or any UE. If the "WlanPerformanceExt_AIML " feature is supported, the "supi" attribute may also be included for a single UE when the subscription applies to the "WLAN_INFO" event;

9.	if available, the GPSI as the "gpsi" attribute if the subscription applies to a group of UE(s) or any UE;

10.	for a Downlink Data Delivery Status, if the "DownlinkDataDeliveryStatus" feature is supported:

a)	the downlink data delivery status as "dddStatus" attribute;

b)	the downlink data descriptors impacted by the downlink data delivery status change within the "dddTraDescriptor" attribute; and

c)	for downlink data delivery status "BUFFERED". the estimated maximum waiting time as "maxWaitTime" attribute;

11.	for a Communication Failure, if the "CommunicationFailure" feature is supported:

a)	the detailed communication failure information (e.g. 5G SM cause) as "commFailure" attribute; and

12.	for QoS Monitoring event, if the "QoSMonitoring" feature is supported:

a)	the uplink packet delays within the "ulDelays" attribute; and/or

b)	the downlink packet delays within the "dlDelays" attribute; and/or

c)	the round trip packet delays within the "rtDelays" attribute; or

NOTE 8: The UPF reports one UL, DL and/or round-trip packet delay measurement for each periodic and/or event-triggered report as described in 3GPP TS 29.244 [23]. i.e, the SMF can include only one element within the "ulDelays", "dlDelays", and/or "rtDelays" array(s), each one with the received report from the UPF for the UL, DL and/or round trip delay(s).

d)	if the feature "PacketDelayFailureReport" is supported, the packet delay measurement failure indicator within the "pdmf" attribute; and/or

e)	if the feature "EnQoSMon" is supported, UL and/or DL congestion information within the "ulCongInfo" attribute and "dlCongInfo" attribute; and/or

f)	if the feature "EnQoSMon" is supported, UL and/or DL data rate measurement within the "ulDataRate" attribute and/or "dlDataRate" attribute.

NOTE 9:	The SMF gets the knowledge of the NF service consumer support of "QoSMonitoring", "PacketDelayFailureReport" and "EnQoSMon" features as described in 3GPP TS 29.512 [14].

NOTE 10: QoS Monitoring notification can be the result of an implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) via the Npcf_SMPolicyControl service (see clause 4.2.3.25 of 3GPP TS 29.512 [14]).

13.	for a PDU Session Establishment, if the "PduSessionStatus" feature is supported:

a)	ID of the established PDU session as "pduSeId" attribute;

b)	DNN of the established PDU session as "dnn" attribute;

c)	the type of the established PDU session as "pduSessType" attribute;

d)	UE IPv4 address as "ipv4Addr" attribute and/or IPv6 information (IPv6 prefix(es) or IPv6 address(es)) as "ipv6Prefixes" or "ipv6Addrs" attributes if available at PDU session establishment; and

e)	S-NSSAI of the established PDU session as "snssai" attribute, if the "EneNA" feature is supported and "snssai" attribute is present in the subscribed "NsmfEventExposure" data type; and

f)	the current RAT Type of the established PDU Session, if available and the "EnPduSesRatType" feature is supported.

14.	for a QFI allocation, if the "QfiAllocation" feature is supported, or for a QFI deallocation or a QoS flow change if the "QoSAssistance" feature is supported:

a)	QFI of the allocated QoS Flow ID for the application as "qfi" attribute or, if the "EnQfiAllocation" feature is also supported, the 5QI of the allocated QoS Flow ID for the application as "5qi" attribute;

b)	DNN of the allocated PDU session as "dnn" attribute;

c)	Slice of the allocated PDU session as "snssai" attribute;

d)	The description of the application traffic as "appId", "fDescs" or "ethfDescs" attribute; and

e)	ID of the allocated PDU session as "pduSeId" attribute if the subscription was for a UE, a group of UEs, or any UE, and not for a specific PDU Session;

f)	To obtain the PDU Session information, if the "PduSessionInfo" feature is supported:

i)	the information about the UE access type provided as "accessType" attribute;

ii) the information about the PDU Session Type in the "pduSessType" attribute and/or the SSC mode in the "sscMode" attribute associated with the application provided as "appId" attribute; and/or

iii)	the information about the PDU Session associated list of access types as "pduAccTypes" attribute, if the "MultipleAccessTypes" feature is also supported.

g)	if the "QoSAssistance" feature is supported, the QoS parameters of the QoS flow in the "qosPara" attribute.

15.	for an RAT type change event, if the "EneNA" feature and/or "EnPduSesRatType" feature is supported:

a)	new RAT type as "ratType" attribute;

16.	for a SM congestion control experience for PDU Session, if the "SMCCE" feature is supported:

a)	DNN of the PDU session as "dnn" attribute if DNN based SMCC is applied

or Slice of the allocated PDU session as "snssai" attribute if S-NSSAI based SMCC is applied;

b)	Time window representing a start time and a stop time of the data collection period as "timeWindow" attribute;

c)	The information of the SM NAS requests from UE as "smNasFromUe" attribute; and

d)	The information of the SM NAS messages from SMF with backoff timer as "smNasFromSmf" attribute;

17. for transactions dispersion collection, if the Dispersion feature is supported:

a)	The transactions dispersion information collected as "transacInfos" attribute; and

b)	The UE IP address as "ueIpAddr" attribute if it is available and requested in the subscription;

18.	for redundant transmission experience of PDU Session, if the "RedundantTransmissionExp" feature is supported:

a)	DNN associated with URLLC service for the PDU session as "dnn" attribute; and

b)	UP with redundant transmission setup as "upRedTrans" attribute;

19.	for WLAN information on PDU Session, if the "WlanPerformance" feature is supported:

a)	SSID or BSSID that the PDU session is related to as "ssId" or "bssId" attribute; and

b)		start time or end time of the PDU Session for WLAN as "startWlan" or "endWlan" attribute;

20. for obtaining the UPF information, if the "ServiceExperience" and/or "DnPerformance" feature is supported:

a)	the information of the UPF serving the UE provided as "upfInfo" attribute.

21. for obtaining the User Plane status information, if the "UeCommunication" feature is supported:

a)	the information about the User Plane status provided as "pduSessInfos" attribute.

22.	for a satellite backhaul category change, if the "EnSatBackhaulCategoryChg" feature is supported:

a)	satellite backhaul category as "satBackhaulCat" attribute.

23.	for traffic correlation, if the "CommonEASDNAI" feature is supported:

a)	the traffic correlation information in the "trafCorreInfo" attribute, if the "notifUri" attribute, "notifCorrId" attribute and "tfcCorrId" attribute are provided in the PCC rule, and the common EAS is not provided in the PCC rule or the SMF decides to trigger EAS discovery for the set of UE(s).

NOTE 11:	Traffic correlation notification can be the result of an implicit subscription of the PCF on behalf of the NEF as part of setting PCC rule(s) via the Npcf_SMPolicyControl service (see clause 4.2.6.2.6.2 of 3GPP TS 29.512 [14]).

-	an URI for further AF acknowledgement in the "ackUri" attribute if the SMF determines to wait for the AF acknowledgement before activating the new UP path associated with the new DNAI.

NOTE 12:	Based on the indication of AF acknowledgment to be expected in the PCC rules received from the PCF and local configuration, the SMF may determine to wait for the AF acknowledgement before activating the new UP path associated with the new DNAI.

24.	for a traffic route requirement installation outcome notification, if the "TraffRouteReqOutcome" feature is supported:

a)	the information indicating the installation outcome of the traffic routing requirements within the "traffRouteReqOutcome" attribute.

b)	the source DNAI and/or target DNAI within the "sourceDnai" attribute and "targetDnai" attribute respectively, if the DNAI is changed;

c)	if available, for the source DNAI, the N6 traffic routing information related to the UE within the "sourceTraRouting" attribute; and

d)	if available, for the target DNAI, the N6 traffic routing information related to the UE within the "targetTraRouting" attribute;

NOTE 13: The Traffic Routing Requirement Installation Outcome Notification i.e. N6 traffic routing requirements installation outcome, can be the result of an implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) via the Npcf_SMPolicyControl service (see clause 4.2.6.2.6.2 of 3GPP TS 29.512 [14]).

25.	for a simultaneous connectivity failure notification, if the "SimConnFailure" feature is supported:

a)	the source DNAI and/or target DNAI within the "sourceDnai" attribute and "targetDnai" attribute respectively, if the DNAI is changed;

NOTE 14: The Simultaneous Connectivity Failure Notification can be the result of an implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) via the Npcf_SMPolicyControl service (see clause 4.2.6.2.6.2 of 3GPP TS 29.512 [14]).

26.	for the Energy Consumption Information event, if the "Energy" feature is supported:

a)	the list of the Data Volume information set(s) within the "dataVolInfoData" attribute, which shall include for each set the UL/DL Data Volume and the associated (I-)UPF ID(s), and may include the associated gNB ID;

b)	the SMF only reports the Energy Consumption Information of the UP resources for the PDU sessions of the UE that are over the 3GPP access;

c)	if the PDU session is over non-3GPP access, the data volume shall be set to 0 and the gNB ID information shall not be provided; and

d)	in the case of MA PDU sessions, the SMF does not include in the report the UL/DL Data volume related to the traffic over the Non-3GPP access leg, and the serving gNB ID, if provided, corresponds to the traffic over the 3GPP access leg.

27.	for signalling information, if the "SignallingInfo" feature is supported:

a)	the usage information of UE IP address resources as "usageInfo" attribute;

b)	the load information of connected UPFs as "loadInfos" attribute;

c)	the number of received Session Report from UPF as "numSessRep" attribute;

d)	the session-related state transitions as "stateTransitions" attribute; and/or

e)	the signalling information of the SMF as "nfSignalInfo" attribute.

Upon the reception of an HTTP POST request with "{notifUri}" as URI and an NsmfEventExposureNotification data structure as request body, the notified NF shall send an HTTP "204 No Content" response for a successful processing.

If errors occur when processing the HTTP POST request, the notified NF shall send the HTTP error response as specified in clause 5.7.

If the feature "ES3XX" is not supported and,

-	if the notified NF is not able to handle the Notification but another unknown NF could possibly handle the notification, it shall reply with an HTTP "404 Not found" error response.

NOTE 15:	An AMF as NF service consumer and/or notified NF can change.

-	if the SMF becomes aware that a new NF service consumer is requiring notifications (e.g. via the "404 Not found" response, or via Namf_Communication service AMFStatusChange Notifications, see 3GPP TS 29.518 [13], or via link level failures or via the Nnrf_NFDiscovery Service (using the service name and GUAMI obtained during the creation of the subscription) to discover the other AMFs within the AMF set) specified in 3GPP TS 29.510 [12]), and the SMF knows alternate or backup IPv4 Address(es), IPv6 Address(es) or FQDN(s) where to send Notifications (e.g. via "altNotifIpv4Addrs", "altNotifIpv6Addrs" or "altNotifFqdns" attributes received when the subscription was created), the SMF shall exchange the authority part of the Notification URI with one of those addresses and shall use that URI in any subsequent communication. If the SMF received a "404 Not found" response, the SMF should resend the failed notification to that URI.

If the feature "ES3XX" is supported, and the notified NF determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4] and,

-	if the SMF receives a "307 Temporary Redirect" response, the SMF shall resend the failed event notification request using the received URI in the Location header field as Notification URI. Subsequent event notifications, triggered after the failed one, shall be sent to the Notification URI provided by the NF service consumer during the corresponding subscription creation/update; or

-	if the SMF receives a "308 Permanent Redirect" response, the SMF shall resend the failed event notification request and send the subsequent event notification using the received URI in the Location header field as Notification URI.

If the SMF in the VPLMN needs to send an event notification to the NEF in the HPLMN, it may normalize the event based on roaming agreements when required before provisioning the event report to the NEF of the HPLMN.

* * * * Next changes * * * *

#### 4.2.3.2	Creating a new subscription

Figure 4.2.3.2-1 illustrates the creation of a subscription.

Figure 4.2.3.2-1: Creation of a subscription

To subscribe to event notifications, the NF service consumer shall send an HTTP POST request with: "{apiRoot}/nsmf-event-exposure/v1/subscriptions" as Resource URI and the NsmfEventExposure data structure as request body that shall include:

-	if the subscription applies to events related to a single PDU session for a UE, the PDU Session ID of that PDU session as "pduSeId" attribute and the UE identification as "supi" or "gpsi" attribute;

-	if the subscription applies to events not related to a single PDU session, the Network Function instance identity if "UPEAS" feature is supported and the "eventSubs" attribute contains an entry with the "event" set to the value "UPF_EVENT", and identification of UEs to which the subscription applies via:

a)	identification of a single UE by SUPI as "supi" attribute or GPSI as "gpsi" attribute;

b)	identification of a group of UE(s) via a "groupId" attribute; or

c)	identification of any UE via the "anyUeInd" attribute set to true;

NOTE 1:	The identification of any UE does not apply for local breakout roaming scenarios where the SMF is located in the VPLMN and the NF service consumer is located in the HPLMN.

-	an URI where to receive the requested notifications as "notifUri" attribute;

-	a Notification Correlation Identifier provided by the NF service consumer for the requested notifications as "notifId" attribute; and

-	if the NF service consumer is an AMF, the GUAMI encoded as "guami" attribute:

-	a description of the subscribed events as "eventSubs" attribute that for each event shall include:

a)	an event identifier as "event" attribute; and

b)	for event "UP_PATH_CH", whether the subscription is for early, late, or early and late notifications of UP path reconfiguration in the "dnaiChgType" attribute;

c)	for event "DDDS", the traffic descriptor(s) of the downlink data source in the "dddTraDescriptors" attribute;

and that may include:

a)	for event "DDDS", the subscribed delivery statuses in the "dddStati" attribute;

b)	for event "QFI_ALLOC" or "DISPERSION", the application identifiers in the "appIds" attribute;

c)	for event "SMCC_EXP", the data collection target period in the "targetPeriod" attribute;

d)	for event "DISPERSION", the UE IP Address in the "ueIpAddr" attribute, the indication of transaction dispersion collection in the "transacDispInd" attribute and the requested transaction metrics in the "transacMetrics" attribute;

e)	for event "WLAN_INFO", the data collection target period in the "targetPeriod" attribute;

f)	for event "RED_TRANS_EXP", the data collection target period in the "targetPeriod" attribute;

g)	for event "UPF_EVENT", the UPF event exposure information in the "upfEvents" attribute. If the "BERMS" feature is supported, it may also provide an indication that the bundling of event reports in UPF notifications is requested to be allowed by including the "bundlingAllowed" attribute set to "true", optionally together with a notification URI for bundled notifications within the "bundledEventNotifyUri" attribute, and a bundling identifier within the "bundleId" attribute;

NOTE 2:	The SMF uses the attributes related to notification bundling to set the equivalent attributes of Nupf_EventExposure when it later invokes this service, and the UPF handles these attributes to perform notification bundling as described in 3GPP TS 29.564 [26] clauses 5.2.2.2.2 and 5.2.2.3.4.

h)	for event "QOS_MON", the Application Identifier in the "appIds" of the application for which the QoS flows are to be monitored and an indication within the "defQosSupp" attribute to inform whether the NF service consumer supports to receive QoS Flow performance information for the QoS Flow associated with the default QoS rule if there are no measurements available for the provided Application Identifier included within the "appIds" attribute;

i)	for the "ENERGY_USAGE_DATA" event:

-	the UE Identity within the "supi" attribute;

-	the S-NSSAI within the "snssai" attribute;

-	the DNN within the "dnn" attribute; and/or

-	the identifier(s) of the application(s), within the "appIds", or the service data flow information, within the "flowDescs", for which the uplink/downlink data volume information is collected and notified;

NOTE 3:	The user-plane Energy Consumption Information reporting interval used by the involved SMF(s) is the network-wide configurable starting time and interval T.

j)	for the "SIGNALLING_INFO" event, the time windows for which the NF service consumer is requesting to receive signalling information in the "tws" attribute if the "SignallingInfo" feature is supported; and/or

k)	a reference identifier within the "referenceId" attribute, if the "EnhEventMgmt" feature is supported.

NOTE 4:	Explicit subscription to "UPF_EVENT" and "QOS_MON" events as described in this clause implies the direct notification from the UPF as specified in 3GPP TS 29.564 [26].

The NsmfEventExposure data structure as request body may also include:

-	if the NF service consumer is an AMF:

a) the name of a service produced by the AMF that expects to receive the notifications about subscribed events encoded as "serviceName" attribute;

b)	Alternate or backup IPv4 Address(es) where to send Notifications encoded as "altNotifIpv4Addrs" attribute;

c)	Alternate or backup IPv6 Address(es) where to send Notifications encoded as "altNotifIpv6Addrs" attribute;

d)	Alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute;

-	a Data Network Name as "dnn" attribute;

-	a single Network Slice Selection Assistance Information as "snssai" attribute;

-	an identification of network area by "networkArea" attribute, if the feature AreaFilter or the feature UPEAS is supported and the "anyUeInd" attribute is provided and set to true;

NOTE 5:	Care needs to be taken with regards to load and major signalling caused when requesting Any UE. This could be achieved via utilization of some event filters (e.g. Area of Interest), a specific DNN, S-NSSAI or sampling ratio as part of Event Reporting Information.

-	a Data Network Identifier as "dnai" attribute, if the feature UPEAS is supported;

-	the SSID that the PDU session is related to as "ssid" attribute, if the feature UPEAS is supported;

-	the BSSID that the PDU session is related to as "bssid" attribute, if the feature UPEAS is supported;

-	the UPF identifier as "upfId" attribute, if the feature UPEAS is supported;

-	immediate reporting flag as "ImmeRep" attribute;

NOTE 6:	For the "PDU_SES_EST" event subscription, the "ImmeRep" attribute needs to be included to enable the SMF to report the current available "PDU_SES_EST" event information for the subscribed PDU Session which is already established.

-	event notification method (periodic, one time, on event detection) as "notifMethod" attribute;

-	maximum Number of Reports as "maxReportNbr" attribute;

-	monitoring Duration as "expiry" attribute;

-	repetition Period for periodic reporting as "repPeriod" attribute;

-	sampling ratio as "sampRatio" attribute;

-	partitioning criteria for partitioning the UEs before performing sampling as "partitionCriteria" attribute if the EneNA feature is supported;

-	group reporting guard time as "grpRepTime" attribute;

-	a notification flag as "notifFlag" attribute if the EneNA feature is supported;

-	notification muting exception instructions within the "notifFlagInstruct" attribute, if the EnhDataMgmt feature is supported and the "notifFlag" attribute is provided and set to "DEACTIVATE"; and/or

-	UDR Restart Indication as "udrRestartInd" attribute; if the "UDRRestProc" feature is supported.

Upon the reception of an HTTP POST request with: "{apiRoot}/nsmf-event-exposure/v1/subscriptions" as Resource URI and NsmfEventExposure data structure as request body, the SMF shall:

-	create a new subscription;

-	assign a subscription correlation ID;

-	select an expiry time that is equal to or less than the expiry time potentially received in the request;

-	store the subscription;

-	if the feature "UPEAS" is supported, and if the NF service consumer subscribed to "QOS_MON" event, the SMF shall check if there is an active PCC rule that includes a Data Collection Application Identifier as described in 3GPP TS 29.512 [14] that matches the Application Identifier received within "appIds" attribute. If there is an active PCC rule, the SMF shall allow the NF service consumer to receive QoS monitoring reports enabled by that PCC rule. If no PCC rule is identified and the "defQosSupp" attribute was received and set to true, the SMF may instruct the UPF to perform QoS monitoring for the QoS Flow associated to the default QoS rule as described in 3GPP TS 29.244 [23]. If no PCC rule is identified and the "defQosSupp" attribute was received and set to false or not received, the SMF may, based on local configuration, reject the request by sending the NO_ACTIVE_PCC_RULE error described in clause 5.7 or include the "qosMonPending" indication set to true in the response to inform the NF service consumer that the reporting will be activated when the measurements are enabled by a PCC rule;

NOTE 7:	The reporting can be activated when a new PCC rule is installed or an existing one is modified with QoS monitoring information that includes the Data Collection Application Identifier related to the subscription. In this case the SMF will act as if the new subscription is received from the NF service consumer.

-	if the feature "UPEAS" is supported and the "upfEvents" attribute is provided, the SMF shall subscribe to the UPF for the respective UPF events as described in 3GPP TS 29.564 [26]. If the "upfEvents" attribute is provided together with the "networkArea" attribute in the EventSubscription data type, the SMF shall subscribe to the UPF for the respective UPF events only when the UE is located in the indicated area. When the UE leaves the indicated area, the SMF shall unsubscribe those events from the UPF as described in 3GPP TS 29.564 [26].

NOTE 8:	To know when a UE enters or leaves the indicated area, the SMF can subscribe to the respective AMF Event Exposure event.

-	if the feature "EnUPEAS" is supported, and the "remainingDataReports" attribute within the"upfEvents" attribute is provided, the SMF shall forward the remaining data reporting indication to the UPF for the respective UPF events as described in 3GPP TS 29.564 [26].

-	send an HTTP "201 Created" response with NsmfEventExposure data structure as response body and a Location header field containing the URI of the created individual subscription resource, i.e. "{apiRoot}/nsmf-event-exposure/v1/subscriptions/{subId}";

-	if the feature "ERIR" is not supported, and if the "ImmeRep" attribute is included and set to true in the request, the SMF shall immediately notify the recipient of notification(s) subscribed in the "notifUri" attribute of the current available value(s) using the Nsmf_EventExposure_Notify service operation, as defined in clause 4.2.2.1;

-	if the feature "ERIR" is supported, and if the "ImmeRep" attribute is included and set to true, the SMF may immediately notify the NF service consumer with the current available value(s) for the subscribed event(s) within the HTTP "201 Created" response as shown in figure 4.2.3.2-1, step 2. The "NsmfEventExposure" data type in the response may include the corresponding event(s) notification within the "eventNotifs" attribute.

-	if the sampling ratio attribute, as "sampRatio", is included in the subscription without a "partitionCriteria" attribute, the SMF shall select a random subset of UEs among the target UEs according to the sampling ratio and only report the event(s) related to the selected subset of UEs. If the "partitionCriteria" attribute is additionally included, then the SMF shall first partition the UEs according to the value of the "partitionCriteria" attribute and then select a random subset of UEs from each partition according to the sampling ratio and only report the event(s) related to the selected subsets of UEs;

-	when the group reporting guard time attribute, as "grpRepTime", is included in the subscription, the SMF shall accumulate all the event reports for the target UEs until the group reporting guard time expires. Then the SMF shall notify the NF service consumer using the Nsmf_EventExposure_Notify service operation, as described in clause 4.2.2.2; and

-	if the "notifFlag" attribute is included and set to "DEACTIVATE" in the request, the SMF shall mute the event notification and store the available events until the NF service consumer requests to retrieve them by setting the "notifFlag" attribute to "RETRIEVAL" or until a muting exception occurs (e.g. full buffer). When a muting exception occurs, the SMF may consider the contents of the "notifFlagInstruct" attribute (if provided) and/or local configuration to determine its actions. If the EnhDataMgmt feature is supported and the SMF accepts the muting instructions provided in the "notifFlag" and/or the "notifFlagInstruct" attributes, it may indicate the applied muting notification settings within the "mutingSetting" attribute in the response. If the SMF does not accept the muting instructions provided in the "notifFlag" and/or the "notifFlagInstruct" attributes, it shall send an HTTP "403 Forbidden" error response including the "cause" attribute set to "MUTING_INSTR_NOT_ACCEPTED"; and

-	if the "UDRRestProc" feature is supported, the "udrRestartInd" attribute was provided and set to "true" in the corresponding request and the SMF determines that there is an existing identical "Individual SMF Notification Subscription" resource, then the SMF shall not create a new subscripton and shall return within the response body of the HTTP "201 Created" status code the NsmfEventExposure data structure including the representation of this existing identical resource, and a Location header field containing the URI of this existing identical resource.

If the SMF received an GUAMI, the SMF may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service specified in 3GPP TS 29.518 [13], and it may use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [12] (using the obtained GUAMI and possibly service name) to query the other AMFs within the AMF set.

If errors occur when processing the HTTP POST request, the SMF shall send an HTTP error response as specified in clause 5.7.

* * * * Next changes * * * *

#### 4.2.4.2	Unsubscription from event notifications

Figure 4.2.4.2-1 illustrates the unsubscription from event notifications.

Figure 4.2.4.2-1: Unsubscription from event notifications

To unsubscribe from event notifications, the NF service consumer shall send an HTTP DELETE request with: "{apiRoot}/nsmf-event-exposure/v1/subscriptions/{subId}" as Resource URI, where "{subId}" is the subscription correlation ID of the existing subscription that is to be deleted.

Upon the reception of the HTTP DELETE request with: "{apiRoot}/nsmf-event-exposure/v1/subscriptions/{subId}" as Resource URI, if the received HTTP request is successfully processed and accepted, the SMF shall:

-	remove the corresponding subscription; and

-	return either:

-	an HTTP "204 No Content" status code; or

-	if the "Energy" feature is supported and the "ENERGY_USAGE_DATA" event was subscribed, an HTTP "200 OK" response with the response body containing the last notification of the Energy Consumption Information as specified in clause 4.2.2.2.

If errors occur when processing the HTTP DELETE request, the SMF shall send an HTTP error response as specified in clause 5.7.

If the feature "ES3XX" is supported, and the SMF determines the received HTTP DELETE request needs to be redirected, the SMF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

* * * * Next changes * * * *

##### 5.3.3.3.3	DELETE

This method shall support the URI query parameters specified in table 5.3.3.3.3-1.

Table 5.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.3.3.3-2 and the response data structures and response codes specified in table 5.3.3.3.3-3.

Table 5.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EventNotification | M | 1 | 200 OK | Successful case. The "Individual SMF Notification Subscription" resource was successfully deleted and the last event notification is returned in the response body. (NOTE 3) |
| n/a |  |  | 204 No Content | Successful case: The Individual SMF Notification Subscription resource matching the subId was deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual SMF Notification Subscription deletion. Applicable if the feature "ES3XX" is supported. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual SMF Notification Subscription deletion. Applicable if the feature "ES3XX" is supported. (NOTE 2) |
| NOTE 1: The manadatory HTTP error status code for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]). NOTE 3: This status code may be returned in the HTTP DELETE response only if the "Energy" feature is supported and the "ENERGY_USAGE_DATA" event was subscribed. |  |  |  |  |

Table 5.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative SMF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance towards which the request is redirected. |

Table 5.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative SMF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance towards which the request is redirected. |

* * * * Next changes * * * *

#### 5.6.2.2	Type NsmfEventExposure

Table 5.6.2.2-1: Definition of type NsmfEventExposure

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| supi |  | Supi |  | C |  | 0..1 |  | Subscription Permanent Identifier. (NOTE 1) (NOTE 8) (NOTE 9) |  |  |  |
| gpsi |  | Gpsi |  | C |  | 0..1 |  | Generic Public Subscription Identifier (NOTE 1) (NOTE 8) This IE is not applicable to "SMCC_EXP" event. |  |  |  |
| anyUeInd |  | boolean |  | C |  | 0..1 |  | This IE shall be present if the event subscription is applicable to any UE. It indicates whether the event subscription is applicable to any UE: - "true": the event subscription is applicable to any UE; - "false"(default): the event subscription is not applicable to any UE. (NOTE 1) (NOTE 4) (NOTE 7) |  |  |  |
| groupId |  | GroupId |  | C |  | 0..1 |  | Identifies a group of UEs. (NOTE 1) |  |  |  |
| pduSeId |  | PduSessionId |  | C |  | 0..1 |  | PDU session ID (NOTE 1) |  |  |  |
| dnn |  | Dnn |  | C |  | 0..1 |  | Data Network Name. (NOTE 9) |  |  |  |
| snssai |  | Snssai |  | C |  | 0..1 |  | A single Network Slice Selection Assistance Information. (NOTE 4) (NOTE 9) |  |  |  |
| dnai |  | Dnai |  | O |  | 0..1 |  | Data network access identifier. |  | UPEAS |  |
| ssId |  | string |  | O |  | 0..1 |  | SSID that the PDU session is related to. |  | UPEAS |  |
| bssId |  | string |  | O |  | 0..1 |  | BSSID that the PDU session is related to. |  | UPEAS |  |
| upfId |  | string |  | O |  | 0..1 |  | Identifies the UPF. |  | UPEAS |  |
| nfId |  | NfInstanceId |  | C |  | 0..1 |  | Indicates the instance identity of the NF creating the subscription. It shall be provided if the "eventSubs" attribute contains an entry with the "event" set to the value "UPF_EVENT". |  | UPEAS |  |
| subId |  | SubId |  | C |  | 0..1 |  | Subscription ID. This parameter shall be supplied by the SMF in HTTP responses that include an object of NsmfEventExposure type. |  |  |  |
| notifId |  | string |  | M |  | 1 |  | Notification Correlation ID provided by the NF service consumer. (NOTE 2) |  |  |  |
| notifUri |  | Uri |  | M |  | 1 |  | Identifies the recipient of Notifications sent by the SMF. |  |  |  |
| altNotifIpv4Addrs |  | array(Ipv4Addr) |  | O |  | 1..N |  | Alternate or backup IPv4 Address(es) where to send Notifications. |  |  |  |
| altNotifIpv6Addrs |  | array(Ipv6Addr) |  | O |  | 1..N |  | Alternate or backup IPv6 Address(es) where to send Notifications. |  |  |  |
| altNotifFqdns |  | array(Fqdn) |  | O |  | 1..N |  | Alternate or backup FQDN(s) where to send Notifications. |  |  |  |
| eventSubs |  | array(EventSubscription) |  | M |  | 1..N |  | Subscribed events. (NOTE 4) |  |  |  |
| eventNotifs |  | array(EventNotification) |  | O |  | 1..N |  | Represents the SMF Events to be reported in the Nsmf_EvenExposure_Subscribe response. May be present when the "ERIR" feature is supported and the "ImmeRep" attribute set to true is included in the subscription request. |  | ERIR |  |
| ImmeRep |  | boolean |  | O |  | 0..1 |  | Indicates whether immediate reporting of the current status of the subscribed event. Set to "true": it is requested that the current status of the subscribed event is immediately reported. - Set to "false": the current status of the subscribed event is not requested to be immediately reported. - Default value is "false" if omitted. (NOTE 6) |  |  |  |
| notifMethod |  | NotificationMethod |  | O |  | 0..1 |  | If "notifMethod" is not supplied, the default value "ON_EVENT_DETECTION" applies. (NOTE 4) (NOTE 5) (NOTE 10) |  |  |  |
| maxReportNbr |  | Uinteger |  | O |  | 0..1 |  | If omitted, there is no limit. (NOTE 4) (NOTE 5) |  |  |  |
| expiry |  | DateTime |  | C |  | 0..1 |  | This attribute indicates the expiry time of the subscription, after which the SMF shall not send any event notifications and the subscription becomes invalid. It may be included in an event subscription request and may be included in an event subscription response based on operator policies. If an expiry time was included in the request, then the expiry time returned in the response should be less than or equal to that value. If the expiry time is not included in the response, the NF service consumer shall not associate an expiry time for the subscription. (NOTE 4) |  |  |  |
| repPeriod |  | DurationSec |  | C |  | 0..1 |  | This attribute indicates the reporting period. Shall be provided if the notification method is set to "PERIODIC". (NOTE 10) |  |  |  |
| guami |  | Guami |  | C |  | 0..1 |  | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as NF service consumer. |  |  |  |
| serviceName |  | ServiceName |  | O |  | 0..1 |  | If the NF service consumer is an AMF, it should provide the name of a service produced by the AMF that makes use of the notification about subscribed events. |  |  |  |
| supportedFeatures |  | SupportedFeatures |  | C |  | 0..1 |  | Contains the list of supported feature(s) among the ones defined in clause 5.8. This attribute shall be present only when feature negotiation needs to take place. |  |  |  |
| sampRatio |  | SamplingRatio |  | O |  | 0..1 |  | Indicates the ratio of the random subset to target UEs, event reports only relates to the subset. |  |  |  |
| partitionCriteria |  | array(PartitioningCriteria) |  | O |  | 1..N |  | Defines criteria for partitioning the UEs in order to apply the sampling ratio for each partition. It may only be included in event subscription requests when the "sampRatio" attribute is also provided. (NOTE 3) |  | EneNA |  |
| grpRepTime |  | DurationSec |  | O |  | 0..1 |  | Indicates the time for which the SMF aggregates the event reports detected by the UEs in a group and report them together to the NF service consumer. |  |  |  |
| notifFlag |  | NotificationFlag |  | O |  | 0..1 |  | Indicates the notification flag, which is used to mute/unmute notifications and to retrieve events stored during a period of muted notifications. Default: "ACTIVATE" |  | EneNA |  |
| notifFlagInstruct |  | MutingExceptionInstructions |  | O |  | 0..1 |  | Contains instructions to be executed upon the occurrence of an event muting exception (e.g. full buffer). It may only be provided if the "notifFlag" is provided and set to "DEACTIVATE". |  | EnhDataMgmt |  |
| mutingSetting |  | MutingNotificationsSettings |  | O |  | 0..1 |  | Contains settings related to the muting of notifications. It may only be provided in the NF service producer response and only if the muting instructions provided in the "notifFlag" and/or the "notifFlagInstruct" attributes are accepted. |  | EnhDataMgmt |  |
| defQosSupp |  | boolean |  | O |  | 0..1 |  | Indicates whether the NF service consumer requests to receive QoS Flow performance information for the QoS Flow associated with the default QoS rule if there are no measurements available for the provided Application Identifier included within the "appIds" attribute. - Set to "true": NF service consumer requests to receive QoS Flow performance information for the QoS Flow associated with the default QoS rule. - Set to "false": NF service consumer does not request to receive QoS Flow performance information for the QoS Flow associated with the default QoS rule. - Default value is "false" if omitted. |  | UPEAS |  |
| qosMonPending |  | boolean |  | O |  | 0..1 |  | Indicates whether the reporting will be activated when the measurements are enabled by a PCC rule. - Set to "true": the reporting will be activated when the measurements are enabled by a PCC rule. It shall be always set to "true" when present. It may only be provided in the response. |  | UPEAS |  |
| udrRestartInd |  | boolean |  | O |  | 0..1 |  | Indicates whether the Event Exposure subscription was initiated by the UDM due to a UDR restart. - "true" indicates that the subscription creation request message is sent by the UDM due to UDR data restoration. Therefore, the Event Exposure Subscription may already exist at the SMF. - "false" indicates that this subscription message is not motivated by a UDR data restoration procedure. - The default value is "false" if this attribute is omitted. |  | UDRRestProc |  |
| NOTE 1: If the event subscription applies for a specific PDU session, the PDU session of a single UE (pduSeId, and gpsi/supi) shall be included; otherwise one and only one of a single UE (gpsi/supi), a group of UEs (groupId), or anyUeInd set to true shall be included. NOTE 2: If the UDM as NF service consumer subscribes to event (e.g. downlink data delivery status, PDU Session Establishment, PDU Session Release) on behalf of AF/NEF, the value of the "notifId" attribute shall be set to the same value as the value of the "notifyCorrelationId" attribute received from the AF/NEF as defined in subclause 6.4.6.2.2 of 3GPP TS 29.503 [30]. NOTE 3: For a given type of partitioning criteria, the UE shall belong to only one single partition as long as it is served by the NF service producer. NOTE 4: If EneNA feature is supported, when the "snssai" attribute is presented together with "anyUeInd" attribute and the "eventSubs" attribute contains "PDU_SES_EST" and "PDU_SES_REL", then only the "ON_EVENT_DETECTION" value is applicable in the "notifMethod" attribute together with "maxReportNbr" attribute and/or "expiry"attribute presence. NOTE 5: The attribute "maxReportNbr" is not applicable when the value of "notifMethod" is set to "ONE_TIME". NOTE 6: The attribute does not follow the related naming convention (i.e. "lowerCamel") defined in clause 5.1.4 of 3GPP TS 29.501 [7]. This attribute is however kept as currently defined in this specification for backward compatibility considerations. NOTE 7: When the "anyUeInd" attribute is present and set to "true", the SMF does not consider the PDU Session(s) for which it is acting as I-SMF when it selects the PDU Session(s). NOTE 8: Void. NOTE 9: If the "Energy" feature is supported and one of the subscribed events within the "eventSubs" attribute is the "ENERGY_USAGE_DATA" event, then the "supi" attribute shall be present, and the "snssai" attribute and/or the "dnn" attribute may be present. NOTE 10: When the "Energy" feature is supported, the only applicable value for the "notifMethod" attribute is "PERIODIC" and the "repPeriod" attribute's value shall be ignored as the reporting period is pre-configured at the SMF. |  |  |  |  |  |  |  |  |  |  |  |

* * * * Next changes * * * *

#### 5.6.2.5	Type EventNotification

Table 5.6.2.5-1: Definition of type EventNotification

| Attribute name |  | Data type |  |  | P | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| event |  | SmfEvent |  |  | M | 1 |  | Event that is notified. |  |  |  |
| referenceId |  | ReferenceId |  |  | O | 0..1 |  | Indicates the reference identifier of the event. |  | EnhEventMgmt |  |
| timeStamp |  | DateTime |  |  | M | 1 |  | Time at which the event is observed. |  |  |  |
| supi |  | Supi |  |  | C | 0..1 |  | Subscription Permanent Identifier. It is included when the subscription applies to a group of UE(s) or any UE. (NOTE 9) |  |  |  |
| gpsi |  | Gpsi |  |  | C | 0..1 |  | Identifies a GPSI. It shall contain an MSISDN. It is included when it is available and the subscription applies to a group of UE(s) or any UE. This IE is not applicable to "SMCC_EXP" event. |  |  |  |
| ueIpAddr |  | IpAddr |  |  | C | 0..1 |  | Indicates the UE IP address, It is included for event "DISPERSION" when it is available and requested in the subscription. |  | Dispersion |  |
| transacInfos |  | array(TransactionInfo) |  |  | C | 1..N |  | Transaction Information. Shall be included for event "DISPERSION". |  | Dispersion |  |
| sourceDnai |  | Dnai |  |  | C | 0..1 |  | Source DN Access Identifier. Shall be included for events "UP_PATH_CH", "TRAFF_ROUTE_REQ_OUTCOME", and "SIM_CONN_FAIL", if the DNAI changed. (NOTE 1, NOTE 2) |  |  |  |
| targetDnai |  | Dnai |  |  | C | 0..1 |  | Target DN Access Identifier. Shall be included for events "UP_PATH_CH", "TRAFF_ROUTE_REQ_OUTCOME", and "SIM_CONN_FAIL", if the DNAI changed. (NOTE 1, NOTE 2) |  |  |  |
| dnaiChgType |  | DnaiChangeType |  |  | C | 0..1 |  | DNAI Change Type. Shall be included for event "UP_PATH_CH". |  |  |  |
| traffRouteReqOutcome |  | TraffRouteReqOutcome |  |  | C | 0..1 |  | Contains the installation outcome of requested traffic routing requirements. This attribute shall be present when the event is set to "TRAFF_ROUTE_REQ_OUTCOME". |  | TraffRouteReqOutcome |  |
| candidateDnais |  | array(Dnai) |  |  | O | 1..N |  | The candidate DNAI(s) for the PDU Session. May be included for event "UP_PATH_CH". |  | CommonEASDNAI |  |
| easRediscoverInd |  | boolean |  |  | O | 0..1 |  | Indication of EAS re-discovery. If present and set to "true", it indicates the EAS re-discovery is performed, e.g. due to change of common EAS. Default value is "false" if omitted. |  | CommonEASDNAI |  |
| candDnaisPrioInd |  | boolean |  |  | O | 0..1 |  | If provided and set to "true", it indicates that the candidate DNAIs provided in the "candidateDnais" attribute are in descending priority order, i.e. the lower the array index the higher the priority of the respective DNAI. If omitted, the default value is "false". It may only be provided if the "candidateDnais" attribute is provided and the "dnaiChgType" attribute is set to the value "EARLY". |  | CommonEASDNAI |  |
| trafCorreInfo |  | TrafficCorrelationNotification |  |  | C | 0..1 |  | Contains traffic correlation information for notification. It shall be provided if the event attribute has the value "TRAFFIC_CORRELATION". |  | CommonEASDNAI |  |
| sourceUeIpv4Addr |  | Ipv4Addr |  |  | O | 0..1 |  | The IPv4 Address of the served UE for the source DNAI. May be included for event "UP_PATH_CH". |  |  |  |
| sourceUeIpv6Prefix |  | Ipv6Prefix |  |  | O | 0..1 |  | The Ipv6 Address Prefix of the served UE for the source DNAI. May be included for event "UP_PATH_CH". |  |  |  |
| targetUeIpv4Addr |  | Ipv4Addr |  |  | O | 0..1 |  | The IPv4 Address of the served UE for the target DNAI. May be included for event "UP_PATH_CH". |  |  |  |
| targetUeIpv6Prefix |  | Ipv6Prefix |  |  | O | 0..1 |  | The Ipv6 Address Prefix of the served UE for the target DNAI. May be included for event "UP_PATH_CH". |  |  |  |
| sourceTraRouting |  | RouteToLocation |  |  | C | 0..1 |  | N6 traffic routing information for the source DNAI. Shall be included for events "UP_PATH_CH" and "TRAFF_ROUTE_REQ_OUTCOME" if available (NOTE 2). |  |  |  |
| targetTraRouting |  | RouteToLocation |  |  | C | 0..1 |  | N6 traffic routing information for the target DNAI. Shall be included for events "UP_PATH_CH" and "TRAFF_ROUTE_REQ_OUTCOME" if available (NOTE 2). |  |  |  |
| ueMac |  | MacAddr48 |  |  | O | 0..1 |  | UE MAC address. May be included for event "UP_PATH_CH". |  |  |  |
| adIpv4Addr |  | Ipv4Addr |  |  | O | 0..1 |  | Added IPv4 Address(es). May be included for event "UE_IP_CH". |  |  |  |
| adIpv6Prefix |  | Ipv6Prefix |  |  | O | 0..1 |  | Added Ipv6 Address Prefix(es). May be included for event "UE_IP_CH". |  |  |  |
| reIpv4Addr |  | Ipv4Addr |  |  | O | 0..1 |  | Removed IPv4 Address(es). May be included for event "UE_IP_CH". |  |  |  |
| reIpv6Prefix |  | Ipv6Prefix |  |  | O | 0..1 |  | Removed Ipv6 Address Prefix(es). May be included for event "UE_IP_CH". |  |  |  |
| plmnId |  | PlmnIdNid |  |  | C | 0..1 |  | New PLMN Identifier or the SNPN Identifier. Shall be included for event "PLMN_CH". It shall be included for event "UP_PATH_CH" to contain the new serving PLMN identifier, if the "HR-SBO" feature is supported and the UE has moved to a serving PLMN where local traffic offloading is allowed. (NOTE 7) |  |  |  |
| accType |  | AccessType |  |  | C | 0..1 |  | New Access Type. Shall be included for event "AC_TY_CH" and may be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". |  |  |  |
| pduAccTypes |  | array(AccessType) |  |  | O | 1..N |  | The list of Access Types used for the PDU session. May be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". (NOTE 10) |  | MultipleAccessTypes |  |
| pduSeId |  | PduSessionId |  |  | C | 0..1 |  | PDU session ID. Shall be included for event "PDU_SES_REL" and "PDU_SES_EST". It shall also be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE" if the subscription was for a UE, a group of UEs, or any UE, and not for a specific PDU Session. |  |  |  |
| ratType |  | RatType |  |  | C | 0..1 |  | Contains the RAT Type. It shall contain either: - the new RAT Typeif the event is "RAT_TY_CH" and the "EneNA" feature and/or "EnPduSesRatType" feature is supported, or - the current RAT Type if the event is either "PDU_SES_REL" or "PDU_SES_EST" and the "EnPduSesRatType" feature is supported. |  | EneNA EnPduSesRatType |  |
| dddStatus |  | DlDataDeliveryStatus |  |  | C | 0..1 |  | Downlink data delivery status (discarded, transmitted, buffered). Shall be included for event "DDDS". |  | DownlinkDataDeliveryStatus |  |
| maxWaitTime |  | DateTime |  |  | C | 0..1 |  | The estimated maximum waiting time for downlink data delivery. Shall be included for event "DDDS" with status "BUFFERED". |  | DownlinkDataDeliveryStatus |  |
| dddTraDescriptor |  | DddTrafficDescriptor |  | C | 0..1 |  |  | The downlink data descriptor impacted by downlink data delivery status change. Shall be included for event "DDDS". |  | DownlinkDataDeliveryStatus |  |
| commFailure |  | CommunicationFailure |  |  | C | 0..1 |  | Describes the communication failure cause for the UE. Shall be included for event "COMM_FAIL". |  | CommunicationFailure |  |
| ipv4Addr |  | Ipv4Addr |  |  | O | 0..1 |  | IPv4 address. May be included for event "PDU_SES_REL" or "PDU_SES_EST". |  | PduSessionStatus |  |
| ipv6Prefixes |  | array(Ipv6Prefix) |  |  | O | 1..N |  | IPv6 prefixes. May be included for event "PDU_SES_REL" or "PDU_SES_EST". (NOTE 3) |  | PduSessionStatus |  |
| ipv6Addrs |  | array(Ipv6Addr) |  |  | O | 1..N |  | IPv6 addresses. May be included for event "PDU_SES_REL" or "PDU_SES_EST". (NOTE 3) |  | PduSessionStatus |  |
| pduSessType |  | PduSessionType |  |  | C | 0..1 |  | PDU session type. Shall be included if the PduSessionStatus or PduSessionInfo feature is supported. (NOTE 8) |  | PduSessionStatus PduSessionInfo |  |
| sscMode |  | SscMode |  |  | O | 0..1 |  | Represents the SSC mode of the PDU Session. It may be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". (NOTE 8) |  | PduSessionInfo |  |
| qfi |  | Qfi |  |  | C | 0..1 |  | QoS flow identifier. May be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". (NOTE 12) |  | QfiAllocation |  |
| appId |  | ApplicationId |  |  | O | 0..1 |  | Contains the application identifier. May be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". (NOTE 4) (NOTE 8) |  | QfiAllocation PduSessionInfo |  |
| ethFlowDescs |  | array(EthFlowDescription) |  |  | O | 1..N |  | Descriptor(s) for non-IP traffic in which only ethernet flow description is defined. It allows the encoding of multiple UL and/or DL flows. Each entry of the array describes a single Ethernet flow. May be included for events "QFI_ALLOC",, "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE" when the description of the Ethernet traffic requires multiple UL and/or DL flows. (NOTE 4) |  | MultipleFlowDescriptions |  |
| ethfDescs |  | array(EthFlowDescription) |  |  | O | 1..2 |  | Contains the flow description for the Uplink and/or Downlink Ethernet flows. May be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". (NOTE 4) |  | QfiAllocation |  |
| flowDescs |  | array(FlowDescription) |  |  | O | 1..N |  | Descriptor(s) of IP traffic. It allows the encoding of multiple UL and/or DL flows. Each entry of the array describes a single IP flow. May be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE", when the description of the IP traffic requires multiple UL and/or DL flows. (NOTE 4) |  | MultipleFlowDescriptions |  |
| fDescs |  | array(FlowDescription) |  |  | O | 1..2 |  | Contains the flow description for the Uplink and/or Downlink IP flows. May be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". (NOTE 4) |  | QfiAllocation |  |
| dnn |  | Dnn |  |  | C | 0..1 |  | Data network name. Shall be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". May be included for event "PDU_SES_REL" or "PDU_SES_EST". Shall be included to indicate the DNN associated with URLLC service for event "RED_TRANS_EXP". Shall be included if DNN based SMCC is applied. It shall be included for event "UP_PATH_CH" to contain the HPLMN DNN, if the "HR-SBO" feature is supported and the UE has moved to a serving PLMN where local traffic offloading is allowed. |  | QfiAllocation, PduSessionStatus RedundantTransmissionExp SMCCE HR-SBO |  |
| snssai |  | Snssai |  |  | C | 0..1 |  | Identifies the slice information. Shall be included for events "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". Shall be included if S-NSSAI based SMCC is applied. It shall be included for event "UP_PATH_CH" to contain the HPLMN S-NSSAI, if the "HR-SBO" feature is supported and the UE has moved to a serving PLMN where local traffic offloading is allowed. |  | QfiAllocation EneNA SMCCE HR-SBO |  |
| ulDelays |  | array(Uinteger) |  | O | 1..N |  |  | Uplink packet delay in units of milliseconds. May be included for event "QOS_MON". (NOTE 5) |  | QoSMonitoring |  |
| dlDelays |  | array(Uinteger) |  | O | 1..N |  |  | Downlink packet delay in units of milliseconds. May be included for event "QOS_MON". (NOTE 5) |  | QoSMonitoring |  |
| ulCongInfo |  | Uinteger |  | O | 0..1 |  |  | Uplink congestion information. Percentage of packets that UPF uses for ECN marking for L4S (without "%" sign). May be included for event "QOS_MON". |  | EnQoSMon |  |
| dlCongInfo |  | Uinteger |  | O | 0..1 |  |  | Downlink congestion information. Percentage of packets that UPF uses for ECN marking for L4S (without "%" sign). May be included for event "QOS_MON". |  | EnQoSMon |  |
| rtDelays |  | array(Uinteger) |  | O | 1..N |  |  | Round trip delay in units of milliseconds. May be included for event "QOS_MON". (NOTE 5) |  | QoSMonitoring |  |
| ulDataRate |  | BitRate |  | O | 0..1 |  |  | Uplink data rate. May be included for event "QOS_MON". |  | EnQoSMon |  |
| dlDataRate |  | BitRate |  | O | 0..1 |  |  | Downlink data rate. May be included for event "QOS_MON". |  | EnQoSMon |  |
| timeWindow |  | TimeWindow |  | C | 0..1 |  |  | Time window representing a start time and a stop time of the data collection period. Shall be included for event "SMCC_EXP". |  | SMCCE |  |
| smNasFromUe |  | array(SmNasFromUe) |  | C | 1..N |  |  | Information on the SM NAS messages that SMF receives from UE for PDU Session. Shall be included for event "SMCC_EXP". |  | SMCCE |  |
| smNasFromSmf |  | array(SmNasFromSmf) |  | C | 1..N |  |  | Information on the SM congestion control applied SM NAS messages that SMF sends to UE for PDU Session. Shall be included for event "SMCC_EXP". |  | SMCCE |  |
| upRedTrans |  | boolean |  | C | 0..1 |  |  | Indicates whether the redundant transmission is setup or terminated. Set to "true" if the redundant transmission is setup, otherwise set to "false" if the redundant transmission is terminated. Default value is "false" if omitted. Shall be included for event "RED_TRANS_EXP". |  | RedundantTransmissionExp |  |
| ssId |  | string |  | C | 0..1 |  |  | SSID that the PDU session is related to. (NOTE 6) |  | WlanPerformance |  |
| bssId |  | string |  | C | 0..1 |  |  | BSSID that the PDU session is related to. (NOTE 6) |  | WlanPerformance |  |
| startWlan |  | DateTime |  | C | 0..1 |  |  | The time stamp that indicates when the existing PDU Session's access type changes to WLAN or when the new PDU Session for WLAN is established. (NOTE 6) |  | WlanPerformance |  |
| endWlan |  | DateTime |  | C | 0..1 |  |  | The time stamp that indicates when the existing WLAN based PDU Session's access type is not WLAN any more or when the PDU Session for WLAN is released. (NOTE 6) |  | WlanPerformance |  |
| pduSessInfos |  | array(PduSessionInformation) |  | C | 1..N |  |  | The PDU session related information. It shall be included for event "UP_STATUS_INFO". |  | UeCommunication |  |
| upfInfo |  | UpfInformation |  | C | 0..1 |  |  | The information of the UPF serving the UE. Shall be included for event "UPF_INFO". |  | ServiceExperience DnPerformance |  |
| pdmf |  | boolean |  | O | 0..1 |  |  | Packet delay measurement failure indicator. When set to true, it indicates that a packet delay failure has occurred, i.e. no measurement result is available during the reporting period. Default value is false if omitted. May be included for event "QOS_MON". |  | PacketDelayFailureReport |  |
| satBackhaulCat |  | SatelliteBackhaulCategory |  | C | 0..1 |  |  | The satellite backhaul category or non-satellite backhaul used for the PDU session. Shall be included for event "SATB_CH". |  | EnSatBackhaulCategoryChg |  |
| supportedFeatures |  | SupportedFeatures |  | C | 0..1 |  |  | List of negotiated features supported by the SMF and NF service consumer as described in clause 5.8. This parameter shall be supplied by the SMF when the SMF detects that at least one feature related to an implicit subscription is supported by both the SMF and the NF service consumer. |  |  |  |
| targetAfId |  | string |  | O | 0..1 |  |  | Identifier of the Application Function responsible for the target DNAI. May be included for event "UP_PATH_CH" if the target DNAI is not known to the source AF. |  | EasRelocationEnh |  |
| 5qi |  | 5Qi |  | O | 0..1 |  |  | The 5G QoS Identifier. May be included for event "QFI_ALLOC", "QFI_DEALLOCATION", and "QOS_FLOW_CHANGE". |  | EnQfiAllocation |  |
| dataVolInfoData |  | array(DataVolumeInformation) |  | C | 1..N |  |  | Contains the list of data volume information set(s). This attribute shall be present only when the "event" attribute is set to "ENERGY_USAGE_DATA". |  | Energy |  |
| usageInfo |  | IpAddrUsageInfo |  | O | 0..1 |  |  | Indicates the usage information of UE IP address resources. May be included for event "SIGNALLING_INFO". |  | SignallingInfo |  |
| loadInfos |  | array(NfLoadLevelInformation) |  | O | 1..N |  |  | Each element indicates the load level information of connected UPFs. May be included for event "SIGNALLING_INFO". (NOTE 13) |  | SignallingInfo |  |
| numSessRep |  | Uinteger |  | O | 0..1 |  |  | Contains the number of received Session Report from UPF triggered by DL packet in case of PDU Session is in 5GCM-idle state. May be included for event "SIGNALLING_INFO". |  | SignallingInfo |  |
| stateTransitions |  | array(StateTransitionInfo) |  | O | 1..N |  |  | Contains information about session-related state transitions of specific UEs. May be included for event "SIGNALLING_INFO". |  | SignallingInfo |  |
| nfSignalInfo |  | NfSignallingInfo |  | O | 0..1 |  |  | Indicates the signalling information of the SMF per SMF service, e.g. the number of requests to each service and related statistics (e.g. successful vs failed responses, etc.). May be included for event "SIGNALLING_INFO". |  | SignallingInfo |  |
| NOTE 1: If the DNAI is not changed while the N6 traffic routing information is changed, the "sourceDnai" attribute and "targetDnai" attribute shall not be provided. NOTE 2: The change from the UP path status where no DNAI applies to a status where a DNAI applies indicates the activation of the related AF request and therefore only the target DNAI and N6 traffic routing information is provided in the event notification; the change from the UP path status where a DNAI applies to a status where no DNAI applies indicates the de-activation of the related AF request and therefore only the source DNAI and N6 traffic routing information is provided in the event notification. NOTE 3: If provided, either ipv6Prefixes or ipv6Addrs shall be present. NOTE 4: Only one of the "appId", "ethfDescs", "ethFlowDescs", "flowDescs" or "fDescs" attributes shall be provided. NOTE 5: In this release of the specification one element may be included in the array as specified in clause 4.2.2.2. NOTE 6: If notified event is "WLAN_INFO", then one of the "ssId" or "bssId" attribute and one of the "startWlan" or "endWlan" attribute shall be present. NOTE 7: The SNPN Identifier consists of the PLMN Identifier and the NID. NOTE 8: When the subscribed event is "QFI_ALLOC" and the PduSessionInfo feature is supported, if the "pduSessType" attribute and/or "sscMode" attribute is included, the associated "appId" attribute shall be provided. NOTE 9: If the "WlanPerformanceExt_AIML" feature is supported, the "supi" attribute may also be included for a single UE when the subscription applies to the "WLAN_INFO" event. NOTE 10: If multiple Access Types are used for the PDU Session and the "MultipleAccessTypes" feature is supported, the SMF shall include each PDU Session Access Type in the "pduAccTypes" attribute. NOTE 11: Void. NOTE 12: The "qfi" attribute and "5qi" attribute are mutually exclusive, either "qfi" attribute or "5qi" attribute shall be included for event "QFI_ALLOC". NOTE 13: The "nfLoadAvgInAoi", "snssai" and "confidence" attributes within the entries of the "loadInfos" attribute are not applicable in this release of the specification, and the other attributes are provided based on the values that are known to the SMF. |  |  |  |  |  |  |  |  |  |  |  |

Editor's note:	Whether the used QoS profile needs to be provided is FFS.

* * * * Next changes * * * *

#### 5.6.2.16	Type: DataVolumeInformation

Table 5.6.2.16-1: Definition of type DataVolumeInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataVol | VolumeTimedReport | M | 1 | Contains the UL/DL data usage information of a PDU Session. (NOTE 1) |  |
| upfIds | array(UpfInformation) | M | 1..N | Contains the identification information of the (I-)UPF(s) associated to the reported data volume information within the "dataVol" attribute. |  |
| gNBId | GNbId | C | 0..1 | Contains the identifier of the gNB serving the UE to which the reported data volume information within the "dataVol" attribute is related. (NOTE 2) |  |
| NOTE 1: The value of the "downlinkVolume" and "uplinkVolume" attributes within the VolumeTimedReport data structure shall be set to 0 when the PDU session is over a non-3GPP access type. NOTE 2: This attribute shall be present only when the PDU session is over a 3GPP access type. |  |  |  |  |  |

* * * * Next changes * * * *

#### 5.6.3.3	Enumeration: SmfEvent

Table 5.6.3.3-1: Enumeration SmfEvent

| Enumeration value |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- |
| AC_TY_CH |  | Access Type Change |  |  |  |
| UP_PATH_CH |  | UP Path Change |  |  |  |
| PDU_SES_REL |  | PDU Session Release |  |  |  |
| PLMN_CH |  | PLMN Change |  |  |  |
| UE_IP_CH |  | UE IP address change |  |  |  |
| RAT_TY_CH |  | RAT Type Change |  | EneNA EnPduSesRatType |  |
| DDDS |  | Downlink data delivery status |  | DownlinkDataDeliveryStatus |  |
| COMM_FAIL |  | Communication failure |  | CommunicationFailure |  |
| PDU_SES_EST |  | PDU Session Establishment |  | PduSessionStatus |  |
| QFI_ALLOC |  | QFI allocation |  | QfiAllocation |  |
| QOS_MON |  | QoS Monitoring |  | QoSMonitoring |  |
| SMCC_EXP |  | SM congestion control experience for PDU Session |  | SMCCE |  |
| DISPERSION |  | Session Management transaction dispersion |  | Dispersion |  |
| RED_TRANS_EXP |  | Redundant transmission experience for PDU Session |  | RedundantTransmissionExp |  |
| WLAN_INFO |  | WLAN information on PDU session for which Access Type is NON_3GPP_ACCESS and RAT Type is TRUSTED_WLAN |  | WlanPerformance |  |
| UPF_INFO |  | The UPF information, including the UPF ID/address/FQDN information. |  | ServiceExperience DnPerformance |  |
| UP_STATUS_INFO |  | User Plane status information |  | UeCommunication |  |
| UPF_EVENT |  | UPF event subscribed via SMF. (NOTE) |  | UPEAS |  |
| SATB_CH |  | Indicates that the SMF has detected a change between different satellite category, or non-satellite backhaul |  | EnSatBackhaulCategoryChg |  |
| TRAFFIC_CORRELATION |  | Indicates that the SMF provides 5GC determined traffic correlation information for a set of UEs identified by Traffic Correlation ID. |  | CommonEASDNAI |  |
| TRAFF_ROUTE_REQ_OUTCOME |  | Indicates the report of the installation outcome of the requested traffic routing requirements. |  | TraffRouteReqOutcome |  |
| SIM_CONN_FAIL |  | Indicates that simultaneous connecitivity over the source and target PDU Session Anchor failed to be established during a PDU Session Anchor change. If the simultaneous connectivity is successfully established, no notifications are sent. |  | SimConnFailure |  |
| QFI_DEALLOCATION |  | QFI deallocation. |  | QoSAssistance |  |
| QOS_FLOW_CHANGE |  | QoS flow change. |  | QoSAssistance |  |
| ENERGY_USAGE_DATA |  | Indicates that the SMF event is the Energy Consumption Information. |  | Energy |  |
| SIGNALLING_INFO |  | Indicates the report of Service Signalling characteristics. |  | SignallingInfo |  |
| NOTE: UPF_EVENT shall only be used for "USER_DATA_USAGE_MEASURES" and "USER_DATA_USAGE_TRENDS" event types in 3GPP TS 29.564 [26]. |  |  |  |  |  |

* * * * Next changes * * * *

# A.2	Nsmf_EventExposure API

openapi: 3.0.0

info:

version: 1.4.0-alpha.4

title: Nsmf_EventExposure

description: |

Session Management Event Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.508 V19.4.0; 5G System; Session Management Event Exposure Service.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.508/

servers:

- url: '{apiRoot}/nsmf-event-exposure/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nsmf-event-exposure

paths:

/subscriptions:

post:

operationId: CreateIndividualSubcription

summary: Create an individual subscription for event notifications from the SMF

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

responses:

'201':

description: Created.

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nsmf-event-exposure/v1/subscriptions/{subId}

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposureNotification'

responses:

'204':

description: No Content, Notification was successful.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

afAcknowledgement:

'{$request.body#/ackUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AckOfNotify'

responses:

'204':

description: No Content (successful acknowledgement)

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subId}:

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

$ref: '#/components/schemas/SubId'

get:

operationId: GetIndividualSubcription

summary: Read an individual subscription for event notifications from the SMF

tags:

- IndividualSubscription (Document)

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

operationId: ReplaceIndividualSubcription

summary: Replace an individual subscription for event notifications from the SMF

tags:

- IndividualSubscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

responses:

'200':

description: OK. Resource was successfully modified and representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

'204':

description: No Content. Resource was successfully modified

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

operationId: DeleteIndividualSubcription

summary: Delete an individual subscription for event notifications from the SMF

tags:

- IndividualSubscription (Document)

responses:

'200':

description: OK. Resource was successfully deleted and representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/EventNotification'

'204':

description: No Content. Resource was successfully deleted

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nsmf-event-exposure: Access to the Nsmf_EventExposure API

schemas:

NsmfEventExposure:

description: >

Represents an Individual SMF Notification Subscription resource. The serviveName property

corresponds to the serviceName in the main body of the specification.

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

anyUeInd:

type: boolean

description: >

Any UE indication. This IE shall be present if the event subscription is applicable to

any UE. Default value "false" is used, if not present.

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

pduSeId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

ssId:

type: string

description: SSID that the PDU session is related to.

bssId:

type: string

description: BSSID that the PDU session is related to.

upfId:

type: string

description: UPF identity.

nfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

subId:

$ref: '#/components/schemas/SubId'

notifId:

type: string

description: Notification Correlation ID assigned by the NF service consumer.

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

altNotifIpv4Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

description: Alternate or backup IPv4 address(es) where to send Notifications.

minItems: 1

altNotifIpv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

description: Alternate or backup IPv6 address(es) where to send Notifications.

minItems: 1

altNotifFqdns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

minItems: 1

description: Alternate or backup FQDN(s) where to send Notifications.

eventSubs:

type: array

items:

$ref: '#/components/schemas/EventSubscription'

minItems: 1

description: Subscribed events

eventNotifs:

type: array

items:

$ref: '#/components/schemas/EventNotification'

minItems: 1

ImmeRep:

type: boolean

notifMethod:

$ref: '#/components/schemas/NotificationMethod'

maxReportNbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

repPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

serviveName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

sampRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SamplingRatio'

partitionCriteria:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PartitioningCriteria'

minItems: 1

description: Criteria for partitioning the UEs before applying the sampling ratio.

grpRepTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifFlag:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NotificationFlag'

notifFlagInstruct:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MutingExceptionInstructions'

mutingSetting:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MutingNotificationsSettings'

defQosSupp:

type: boolean

description: >

Indicates whether the NF service consumer requests to receive QoS Flow performance

information for the QoS Flow associated with the default QoS rule if there are no

measurements available for the provided Application Identifier included in the appIds

attribute. Set to "true" if NF service consumer requests to receive QoS Flow

performance information for the QoS Flow associated with the default QoS rule, otherwise

Set to "false" to indicate if NF service consumer does not request to receive QoS Flow

Performance information for the QoS Flow associated with the default QoS rule. Default

value is "false" if omitted.

qosMonPending:

type: boolean

enum:

- true

description: >

Indicates whether the reporting will be activated when the measurements are

enabled by a PCC rule. Set to "true" indicates that the reporting will be activated.

It may only be provided in the response.

udrRestartInd:

type: boolean

default: false

description: >

Indicates whether the Event Exposure subscription was initiated by the UDM due to a UDR

restart.

"true" indicates that the subscription creation request message is sent by the UDM due to

UDR data restoration. Therefore, the Event Exposure Subscription may already exist at the

SMF.

"false" indicates that this subscription message is not motivated by a UDR data

restoration procedure.

The default value is "false" if this attribute is omitted

required:

- notifId

- notifUri

- eventSubs

NsmfEventExposureNotification:

description: Represents notifications on events that occurred.

type: object

properties:

notifId:

type: string

description: Notification correlation ID

eventNotifs:

type: array

items:

$ref: '#/components/schemas/EventNotification'

minItems: 1

description: Notifications about Individual Events

ackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

required:

- notifId

- eventNotifs

EventSubscription:

description: Represents a subscription to a single event.

type: object

properties:

event:

$ref: '#/components/schemas/SmfEvent'

referenceId:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/ReferenceId'

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

dddTraDescriptors:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DddTrafficDescriptor'

minItems: 1

dddStati:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DlDataDeliveryStatus'

minItems: 1

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

networkArea:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

targetPeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

tws:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

transacDispInd:

type: boolean

description: >

Indicates the subscription for UE transaction dispersion collectionon, if it is included

and set to "true". Default value is "false".

transacMetrics:

type: array

items:

$ref: '#/components/schemas/TransactionMetric'

description: Indicates Session Management Transaction metrics.

minItems: 1

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

upfEvents:

type: array

items:

$ref: 'TS29564_Nupf_EventExposure.yaml#/components/schemas/UpfEvent'

description: Indicates UPF event exposure information.

minItems: 1

bundlingAllowed:

type: boolean

enum:

- true

description: >

This attribute may be included for event "UPF_EVENT" and indicates whether it is

requested to allow the bundling of event reports in UPF notifications. Possible values:

-"true": it is requested to allow the bundling of event reports in UPF notifications.

The presence of this attribute with the value "false" shall be prohibited.

bundleId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

bundledEventNotifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

flowDescs:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

description: Descriptor(s) of IP traffic.

minItems: 1

required:

- event

EventNotification:

description: Represents a notification related to a single event that occurred.

type: object

properties:

event:

$ref: '#/components/schemas/SmfEvent'

referenceId:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/ReferenceId'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

transacInfos:

type: array

items:

$ref: '#/components/schemas/TransactionInfo'

description: Transaction Information.

minItems: 1

sourceDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

traffRouteReqOutcome:

$ref: '#/components/schemas/TraffRouteReqOutcome'

candidateDnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

description: The candidate DNAI(s) for the PDU Session.

candDnaisPrioInd:

type: boolean

description: >

If provided and set to true, it indicates that the candidate DNAIs provided

in the candidateDnais attribute are in descending priority order, i.e.,

the lower the array index the higher the priority of the respective DNAI.

If omitted, the default value is false.

easRediscoverInd:

type: boolean

description: >

Indication of EAS re-discovery. If present and set to "true", it indicates the EAS

re-discovery is performed, e.g. due to change of common EAS. Default value is "false" if

omitted.

trafCorreInfo:

$ref: '#/components/schemas/TrafficCorrelationNotification'

sourceUeIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

sourceUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

targetUeIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

targetUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

sourceTraRouting:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

targetTraRouting:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

ueMac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

adIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

adIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

reIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

reIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

accType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

pduAccTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

pduSeId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

dddStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DlDataDeliveryStatus'

dddTraDescriptor:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DddTrafficDescriptor'

maxWaitTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

commFailure:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/CommunicationFailure'

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Prefixes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

minItems: 1

ipv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

pduSessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

qfi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

ethFlowDescs:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

description: >

Descriptor(s) for non-IP traffic. It allows the encoding of multiple UL and/or DL flows.

Each entry of the array describes a single Ethernet flow.

ethfDescs:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

maxItems: 2

description: >

Contains the UL and/or DL Ethernet flows. Each entry of the array describes a single

Ethernet flow.

flowDescs:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

description: >

Descriptor(s) for IP traffic. It allows the encoding of multiple UL and/or DL flows.

Each entry of the array describes a single IP flow.

fDescs:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

maxItems: 2

description: >

Contains the UL and/or DL IP flows. Each entry of the array describes a single

IP flow.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

ulDelays:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

dlDelays:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

rtDelays:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

ulCongInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

dlCongInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

ulDataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

dlDataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

smNasFromUe:

$ref: '#/components/schemas/SmNasFromUe'

smNasFromSmf:

$ref: '#/components/schemas/SmNasFromSmf'

upRedTrans:

type: boolean

description: >

Indicates whether the redundant transmission is setup or terminated. Set to "true" if

the redundant transmission is setup, otherwise set to "false" if the redundant

transmission is terminated. Default value is "false" if omitted.

ssId:

type: string

bssId:

type: string

startWlan:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endWlan:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

pduSessInfos:

type: array

items:

$ref: '#/components/schemas/PduSessionInformation'

minItems: 1

upfInfo:

$ref: '#/components/schemas/UpfInformation'

pdmf:

type: boolean

description: >

Represents the packet delay measurement failure indicator. Default value is false

if omitted.

satBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

targetAfId:

type: string

description: Identifier of the Application Function responsible for the target DNAI.

5qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

qosPara:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/QosPara'

dataVolInfoData:

type: array

items:

$ref: '#/components/schemas/DataVolumeInformation'

minItems: 1

usageInfo:

$ref: '#/components/schemas/IpAddrUsageInfo'

loadInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NfLoadLevelInformation'

minItems: 1

numSessRep:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

stateTransitions:

type: array

items:

$ref: '#/components/schemas/StateTransitionInfo'

minItems: 1

nfSignalInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSignallingInfo'

required:

- event

- timeStamp

not:

required: [ipv6Prefixes,ipv6Addrs]

StateTransitionInfo:

description: Represents session-related state transition information.

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

transType:

$ref: '#/components/schemas/SmfEvent'

numOfTran:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

pctUes:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- transType

allOf:

- oneOf:

- required: [supi]

- required: [groupId]

- anyOf:

- required: [numOfTran]

- required: [pctUes]

SubId:

type: string

format: SubId

description: >

Identifies an Individual SMF Notification Subscription. To enable that the value is used as

part of a URI, the string shall only contain characters allowed according to the

"lower-with-hyphen" naming convention defined in 3GPP TS 29.501. In an OpenAPI schema, the

format shall be designated as "SubId".

AckOfNotify:

description: Represents an acknowledgement information of an event notification.

type: object

properties:

notifId:

type: string

ackResult:

$ref: 'TS29522_TrafficInfluence.yaml#/components/schemas/AfResultInfo'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

remoteImsAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteInformation'

required:

- notifId

- ackResult

SmNasFromUe:

description: >

Represents information on the SM NAS messages that SMF receives from UE for PDU Session.

type: object

properties:

smNasType:

type: string

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- smNasType

- timeStamp

SmNasFromSmf:

description: >

Represents information on the SM congestion control applied SM NAS messages that SMF sends

to UE for PDU Session.

type: object

properties:

smNasType:

type: string

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

backoffTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

appliedSmccType:

$ref: '#/components/schemas/AppliedSmccType'

required:

- smNasType

- timeStamp

- backoffTimer

- appliedSmccType

TransactionInfo:

description: Represents SMF Transaction Information.

type: object

properties:

transaction:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

transacMetrics:

type: array

items:

$ref: '#/components/schemas/TransactionMetric'

minItems: 1

required:

- transaction

PduSessionInformation:

description: Represents the PDU session related information.

type: object

properties:

pduSessId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

sessInfo:

$ref: '#/components/schemas/PduSessionInfo'

PduSessionInfo:

description: Represents session information.

type: object

properties:

n4SessId:

type: string

description: The identifier of the N4 session for the reported PDU Session.

sessInactiveTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

pduSessStatus:

$ref: '#/components/schemas/PduSessionStatus'

UpfInformation:

description: Represents the ID/address/FQDN of the UPF.

type: object

properties:

upfId:

type: string

upfAddr:

$ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AddrFqdn'

TrafficCorrelationNotification:

description: Represents notifications for 5GC determined Traffic Correlation Information.

type: object

properties:

smfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

tfcCorrId:

type: string

description: >

Identification of a set of UEs accessing the application identified by the

Application Identifier or traffic filtering information.

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

easFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

easIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

pduSessionNbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- smfId

- pduSessionNbr

- tfcCorrId

anyOf:

- required: [dnais]

- oneOf:

- required: [easFqdn]

- required: [easIpAddr]

TraffRouteReqOutcome:

description: >

Represents the installation outcome of the requested traffic routing requirements.

type: object

properties:

succTrafficFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

succEthTrafficFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

failedTrafficFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

failedEthTrafficFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

allOf:

- not:

required: [succTrafficFlows, succEthTrafficFlows]

- not:

required: [failedTrafficFlows, failedEthTrafficFlows]

DataVolumeInformation:

description: Represents the Data Volume information.

type: object

properties:

dataVol:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VolumeTimedReport'

upfIds:

type: array

items:

$ref: '#/components/schemas/UpfInformation'

minItems: 1

gNBId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GNbId'

required:

- dataVol

- upfIds

IpAddrUsageInfo:

description: Indicates the usage information of UE IP address resources.

type: object

properties:

ipv4AllocNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

Ipv6AllocNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

ipv4Usage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

Ipv6Usage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

UeIpNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

prohibitTimeWins:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

SmfEvent:

anyOf:

- type: string

enum:

- AC_TY_CH

- UP_PATH_CH

- PDU_SES_REL

- PLMN_CH

- UE_IP_CH

- RAT_TY_CH

- DDDS

- COMM_FAIL

- PDU_SES_EST

- QFI_ALLOC

- QOS_MON

- SMCC_EXP

- DISPERSION

- RED_TRANS_EXP

- WLAN_INFO

- UPF_INFO

- UP_STATUS_INFO

- UPF_EVENT

- SATB_CH

- TRAFFIC_CORRELATION

- TRAFF_ROUTE_REQ_OUTCOME

- SIM_CONN_FAIL

- QFI_DEALLOCATION

- QOS_FLOW_CHANGE

- ENERGY_USAGE_DATA

- SIGNALLING_INFO

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the types of events that can be subscribed.

Possible values are:

- AC_TY_CH: Access Type Change.

- UP_PATH_CH: UP Path Change.

- PDU_SES_REL: PDU Session Release.

- PLMN_CH: PLMN Change.

- UE_IP_CH: UE IP address change.

- RAT_TY_CH: RAT Type Change.

- DDDS: Downlink data delivery status.

- COMM_FAIL: Communication Failure.

- PDU_SES_EST: PDU Session Establishment.

- QFI_ALLOC: QFI allocation.

- QOS_MON: QoS Monitoring.

- SMCC_EXP: SM congestion control experience for PDU Session.

- DISPERSION: Session Management transaction dispersion.

- RED_TRANS_EXP: Redundant transmission experience for PDU Session.

- WLAN_INFO: WLAN information on PDU session for which Access Type is NON_3GPP_ACCESS and

RAT Type is TRUSTED_WLAN.

- UPF_INFO: The UPF information, including the UPF ID/address/FQDN information.

- UP_STATUS_INFO: The User Plane status information.

- UPF_EVENT: UPF event subscribed via SMF.

- SATB_CH: Satellite backhaul category change.

- TRAFFIC_CORRELATION: Indicates that the SMF provides 5GC determined traffic correlation

information for a set of UEs identified by Traffic Correlation ID.

- TRAFF_ROUTE_REQ_OUTCOME: Indicates the report of the installation outcome of the requested

traffic routing requirements.

- SIM_CONN_FAIL: Indicates that the simultaneous connectivity over the source and the target

PDU Session Anchor failed to be established during a PDU Session Anchor change.

- QFI_DEALLOCATION: QFI deallocation.

- QOS_FLOW_CHANGE: QoS flow change.

- ENERGY_USAGE_DATA: Indicates that the SMF event is the Energy Consumption Information.

- SIGNALLING_INFO: Indicates the report of Service Signalling characteristics.

NotificationMethod:

anyOf:

- type: string

enum:

- PERIODIC

- ONE_TIME

- ON_EVENT_DETECTION

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the notification methods that can be subscribed.

Possible values are:

- PERIODIC: The notification is periodically sent.

- ONE_TIME: The notification is only sent one time.

- ON_EVENT_DETECTION: The notification is sent each time the event is detected.

AppliedSmccType:

anyOf:

- type: string

enum:

- DNN_CC

- SNSSAI_CC

description: >

This string indicates the type of applied SM congestion control.

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the type of applied SM congestion control.

Possible values are:

- DNN_CC: Indicates the DNN based congestion control.

- SNSSAI_CC: Indicates the S-NSSAI based congestion control.

TransactionMetric:

anyOf:

- type: string

enum:

- PDU_SES_EST

- PDU_SES_AUTH

- PDU_SES_MODIF

- PDU_SES_REL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the metric on UE Session Management transactions.

Possible values are:

- PDU_SES_EST: PDU Session Establishment.

- PDU_SES_AUTH: PDU Session Authentication.

- PDU_SES_MODIF: PDU Session Modification.

- PDU_SES_REL: PDU Session Release

PduSessionStatus:

anyOf:

- type: string

enum:

- ACTIVATED

- DEACTIVATED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the status of the PDU Session.

Possible values are:

- ACTIVATED: PDU Session status is activated.

- DEACTIVATED: PDU Session status is deactivated.

* * * * End of changes * * * *