3GPP TSG CT WG3 Meeting #143	C3-254087

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.214 | CR | 1703 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of AF trigger the PCEF failure checking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RES-CT |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in clause 7.2.2 in TS 23.380, it requires the AF to request the PCRF to detect PCEF failure via Specific-Action AVP. And when a failure detected, it shall inform P-CSCF and trigger the P-CSCF restoration procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The AF request the PCRF to detect the PCEF failure via Specific-Action AVP. 2. Add a new value of the Specific-Action AVP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 requirement will not be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.6.2, 5.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.4.6.2	Service Data Flow Deactivation and Resource Allocation Failure

It may happen that one or more PCC/QoS Rules (i.e. Service Data Flows) are deactivated at the PCEF/BBERF at a certain time, either permanently or temporarily or resource allocation for one or more PCC rules is unsuccessful at the PCEF/BBERF when the PCC rule(s) are installed. When the PCRF gets the knowledge that one or more SDFs have been deactivated, (e.g. due to a bearer release or loss of bearer or out of credit condition), or the resource allocation failed the PCRF shall inform the AF accordingly if the AF has previously subscribed using the Specific-Action AVP in the AAR command.

When not all the service data flows within the AF session are affected, the PCRF shall inform the AF by sending an RAR (re-authorization request) command. The RAR command shall include the affected IP Flows encoded in the Flows AVP, the cause encoded in the Specific-Action AVP and the content version of a media component within the Content-Version AVP if it was included when the media component was provisioned.

If the RAN-NAS-Cause feature is supported and the PCRF received the access network information from the PCEF/BBERF due to bearer termination or unsuccessful bearer establishment/modification, the PCRF shall include in the RAR command the access network information within the 3GPP-User-Location-Info AVP (if available), TWAN-Identifier (if available and Netloc-Trusted-WLAN feature is supported) User-Location-Info-Time AVP (if available), 3GPP-SGSN-MCC-MNC AVP (if location info is not available) and/or 3GPP-MS-TimeZone AVP (if available). Additionally, if the PCRF received from the PCEF the RAN cause and/or NAS cause, TWAN cause or untrusted WLAN cause due to bearer termination, the PCRF shall provide the received cause(s) in the RAN-NAS-Release-Cause AVP in the RAR command.

When the AF receives the RAR command, it shall acknowledge the command by sending an RAA (re-authorization answer) command to the PCRF. The AF may also update the session information by sending an AAR (AA-request) command to the PCRF.

If the PCRF receives the AAR command, it shall acknowledge the command by sending an AAA (AA-answer) command to the AF.

When all the service data flows within the AF session are affected, the PCRF shall inform the AF by sending an ASR command on the Rx Diameter session related to the AF session. When the AF receives the ASR command, it shall acknowledge the command by sending an ASA (abort session answer) command to the PCRF. After that the AF shall initiate an AF session termination procedure as defined in clause 4.4.4.

If the PCRF supports PcefFailureDetection feature and if the PCRF has detected due to operator configuration that the PCEF has failed, the PCRF may inform the AF by sending an ASR command on the Rx Diameter session related to the AF session with the Abort-Cause AVP set to PCEF-FAILURE (5) to trigger the HSS based P-CSCF restoration procedure as defined in 3GPP TS 23.380 [28].

If the PCRF supports PcefFailureDetection feature is supported and the AF indicates that it requests the PCRF to check the PCEF failure via the Specific-Action AVP set to value PCEF_FAILURE_CHECK (21), the PCRF shall return an AAA with Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065) when the PCRF checks any failure in the PCEF.

Signalling flows for Service Data Flow Deactivation and Resource Allocation Failure cases are presented in 3GPP TS 29.213 [9].

* * * Next Change * * * *

5.3.13	Specific-Action AVP

The Specific-Action AVP (AVP code 513) is of type Enumerated.

Within a PCRF initiated Re-Authorization Request, the Specific-Action AVP determines the type of the action.

Within an initial AA request the AF may use the Specific-Action AVP to request any specific actions from the server at the bearer events and to limit the contact to such bearer events where specific action is required. If the Specific-Action AVP is omitted within the initial AA request, no notification of any of the events defined below is requested at this time.

For one time specific actions, as identified in the value descriptions below, the AF may also provide the Specific-Action AVP with the applicable one-time-specific-action value(s) in subsequent AA-Requests. Non-one-time-specific-action value(s) may only be provided in the initial AA-Request and shall then be applicable for the entire lifetime of the Rx session.

NOTE 1:	One time specific actions are reported once the required action is fulfilled and are not reported again unless the AF sends a new request.

NOTE 2:	Unless otherwise stated in the definition of the specific action value, when the AF requests specific actions in the initial AA-Request, the PCRF reports that action whenever new related information is available during the lifetime of the Rx session.

NOTE 2a:	Whether the PCRF decides to report INDICATION_OF_RELEASE_OF_BEARER (4) or INDICATION_OF_FAILED_RESOURCES_ALLOCATION (9) upon receipt of a bearer failure from the PCEF is left to the implementation.

The following values are defined:

Void (0)

CHARGING_CORRELATION_EXCHANGE (1)

Within a RAR, this value shall be used when the server reports the access network charging identifier to the AF. The Access-Network-Charging-Identifier AVP shall be included within the request. In the AAR, this value indicates that the AF requests the server to provide the access network charging identifier to the AF for each authorized flow, when the access network charging identifier becomes known at the PCRF.

INDICATION_OF_LOSS_OF_BEARER (2)

Within a RAR, this value shall be used when the server reports a loss of a bearer (in the case of GPRS PDP context bandwidth modification to 0 kbit for GBR bearers) to the AF. The SDFs that are deactivated as a consequence of this loss of bearer shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the server to provide a notification at the loss of a bearer.

INDICATION_OF_RECOVERY_OF_BEARER (3)

Within a RAR, this value shall be used when the server reports a recovery of a bearer (in the case of 3GPP-GPRS or 3GPP-EPS when PGW interoperates with a Gn/Gp SGSN, PDP context bandwidth modification from 0 kbit to another value for GBR bearers) to the AF. The SDFs that are re-activated as a consequence of the recovery of bearer shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the server to provide a notification at the recovery of a bearer.

INDICATION_OF_RELEASE_OF_BEARER (4)

Within a RAR, this value shall be used when the server reports the release of a bearer (e.g. PDP context removal for 3GPP-GPRS or bearer/PDP context removal for 3GPP-EPS) to the AF. The SDFs that are deactivated as a consequence of this release of bearer shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the server to provide a notification at the removal of a bearer. The content version corresponding to the affected media component may be provided in the Content-Version AVP included within the Flows AVP.

Void (5)

IP-CAN_CHANGE (6)

This value shall be used in RAR command by the PCRF to indicate a change in the IP-CAN type or RAT type (if applicable). When used in an AAR command, this value indicates that the AF is requesting subscription to IP-CAN change and RAT change notification. When used in RAR it indicates that the PCRF generated the request because of an IP-CAN or RAT change. IP-CAN-Type AVP, RAT-Type AVP (if applicable) ,AN-Trusted AVP (if applicable) and AN-GW-Address AVP (if applicable) shall be provided in the same request with the new/valid value(s).

If an IP-CAN type or RAT type change is due to IP flow mobility and a subset of the flows within the AF session is affected, the affected service data flows shall be provided in the same request.

If ATSSS feature is supported, and the PDU session is a MA PDU session, the PCF may include the MA-Information AVP in the RAR command with the additional/released IP-CAN type and RAT type (if applicable), with the new/valid values as described in clause E.4.

INDICATION_OF_OUT_OF_CREDIT (7)

Within a RAR, this value shall be used when the PCRF reports to the AF that SDFs have run out of credit, and that the termination action indicated by the corresponding Final-Unit-Action AVP applies (3GPP TS 32.240 [23] and 3GPP TS 32.299 [24]. The SDFs that are impacted as a consequence of the out of credit condition shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the PCRF to provide a notification of SDFs for which credit is no longer available. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

INDICATION_OF_SUCCESSFUL_RESOURCES_ALLOCATION (8)

Within a RAR, this value shall be used by the PCRF to indicate that the resources requested for particular service information have been successfully allocated. The SDFs corresponding to the resources successfully allocated shall be provided within the Flows AVP and the content version within the Content-Version AVP as included when the corresponding media component was provisioned.

In the AAR, this value indicates that the AF requests the PCRF to provide a notification when the resources associated to the corresponding service information have been allocated.

Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

NOTE 3:	This value applies to applications for which the successful resource allocation notification is required for their operation since subscription to this value impacts the resource allocation signalling overhead towards the PCEF/BBERF.

INDICATION_OF_FAILED_RESOURCES_ALLOCATION (9)

Within a RAR, this value shall be used by the PCRF to indicate that the resources requested for a particular service information cannot be successfully allocated. The SDFs corresponding to the resources that could not be allocated shall be provided within the Flows AVP. In case of session modification failure, the status of the related service information may be reported in the Media-Component-Status AVP included within the Flows AVP and the content version within the Content-Version AVP as included when the corresponding media component was provisioned.

In the AAR, this value indicates that the AF requests the PCRF to provide a notification when the resources associated to the corresponding service information cannot be allocated. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

NOTE 4:	This value applies to applications for which the unsuccessful resource allocation notification is required for their operation since subscription to this value impacts the resource allocation signalling overhead towards the PCEF/BBERF.

INDICATION_OF_LIMITED_PCC_DEPLOYMENT (10)

Within a RAR, this value shall be used when the PCRF reports the limited PCC deployment (i.e. dynamically allocated resources are not applicable) as specified at Annex K and Annex L in 3GPP TS 23.203 [2] to the AF. In the AAR, this value indicates that the AF requests the PCRF to provide a notification for the limited PCC deployment. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

USAGE_REPORT (11)

In the RA-Request (RAR), this value shall be used by the PCRF to report accumulated usage volume and/or time of usage when the usage threshold provided by the AF has been reached.

In the AA-Request (AAR), this value indicates that the AF requests PCRF to report accumulated usage volume and /or time of usage when it reaches the threshold.

Applicable to functionality introduced with the SponsoredConnectivity feature for volume usage reporting and with SCTimeBased UM feature for time usage reporting as described in clause 5.4.1.

ACCESS_NETWORK_INFO_REPORT (12)

In the RA-Request (RAR), this value shall be used by the PCRF to report access network information (i.e.user location, user timezone information and/or UE serving satellite identifier information) when the PCRF receiving an Access Network Information report corresponding to the AF session from the PCEF/BBERF.

In the AA-Request (AAR), this value indicates that the AF requests PCRF to report one time access network information when the PCRF receives the first Access Network Information report corresponding to the AF session from the PCEF/BBERF after the AF request for the access network information. The required access information is provided within the Required-Access-Info AVP. Applicable to functionality introduced with the NetLoc feature as described in clause 5.4.1.

The Specific-Action AVP with this value indicates a one time specific action.

INDICATION_OF_RECOVERY_FROM_LIMITED_PCC_DEPLOYMENT (13)

Within a RAR, this value shall be used when the PCRF reports the recovery from limited PCC deployment (i.e. the UE moves from the VPLMN to the HPLMN as specified at Annex K in 3GPP TS 23.203 [2]) to the AF. In the AAR, this value indicates that the AF requests the PCRF to provide a notification for the recovery from limited PCC deployment. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

NOTE 5:	This value is optional and only applicable to the scenario where PCC is deployed in the HPLMN but	not in the VPLMN and dynamic policy provisioning only occurs in the home routed roaming cases if no BBERF is employed.

INDICATION_OF_ACCESS_NETWORK_INFO_REPORTING_FAILURE (14)

In the RAR, this value shall be used when the PCRF reports the access network information reporting failure. When applicable, the NetLoc-Access-Support AVP may be provided as well to indicate the reason for the access network information reporting failure. This specific action does not require to be provisioned by the AF. Applicable to functionality introduced with the NetLoc feature as described in clause 5.4.1.

INDICATION_OF_TRANSFER_POLICY_EXPIRED (15)

In the RAR, this value shall be used when the PCRF determines that the transfer policy has expired. This specific action does not require to be provisioned by the AF.

PLMN_CHANGE (16)

In the AA-Request (AAR), this value indicates that the AF requests PCRF to report changes of PLMN. In the RA-Request (RAR), this value shall be used by the PCRF to indicate that there was a change of PLMN. 3GPP-SGSN-MCC-MNC AVP shall be provided in the same RAR command with the new value and, if available, the NID AVP. Applicable to functionality introduced with the PLMNInfo feature as described in clause 5.4.1.

The NID AVP is only applicable in 5GS when the serving network is an SNPN, as described in Annex E.

EPS_FALLBACK (17)

In the RA-Request (RAR), this value shall be used to report EPS Fallback for the resources requested for a particular service information (media type voice).

In the AA-Request (AAR), this value indicates that the AF requests to provide a notification when the access network initiates EPS Fallback for the requested resources associated to service information for voice media type.

Applicable to functionality introduced with the EPSFallbackReport feature as described in clause 5.4.1.

This value is only applicable to 5GS as described in Annex E.

INDICATION_OF_REALLOCATION_OF_CREDIT (18)

Within a RAR, this value shall be used to report to the AF the SDFs for which credit has been reallocated after the former out of credit indication (3GPP TS 32.240 [23] and 3GPP TS 32.299 [24]). The SDFs that are impacted as a consequence of the reallocation of credit condition shall be provided within the Flows AVP. In the AAR, this value indicates the AF requests to provide a notification of SDFs for which credit has been reallocated after the former out of credit indication. Applicable to functionality introduced with the ReallocationOfCredit feature as described in clause 5.4.1.

This value is only applicable to 5GS as described in Annex E.

SUCCESSFUL_QOS_UPDATE (19)

Within a RAR, this value shall be used by the PCRF to indicate that the QoS of the default bearer has been successfully updated.

In the AAR, this value indicates that the AF requests the PCRF to provide a notification when the QoS of the default bearer has been successfully updated.

Applicable to functionality introduced with the MPSforDTS feature as described in clause 5.4.1.

FAILED_QOS_UPDATE (20)

Within a RAR, this value shall be used by the PCRF to indicate that the QoS of the default bearer has failed to update.

In the AAR, this value indicates that the AF requests the PCRF to provide a notification when the QoS of the default bearer has failed to update.

Applicable to functionality introduced with the MPSforDTS feature as described in clause 5.4.1.

PCEF_FAILURE_CHECK (21)

In the AAR, this value indicates that the AF requests PCRF start health check. This action serves as a test to determine whether a failure exists in the EPC network.

Applicable to functionality introduced with the PcefFailureDetection feature as described in clause 5.4.1.

* * * End of Change * * * *