3GPP TSG-CT WG3 Meeting #143	C3-254102

# SophiaAntipolis, FR, 13 - 17 October 2025

Source:	Nokia

Title:	MRI packet transforms naming

Document for:	Discussion

# Agenda Item:	19.48

# Work Item / Release:	XRM_Ph2 / Rel-19

Abstract of the contribution:

The need for the common MRI packet transformation naming across the features and security mechanisms is elaborated.

# Introduction:

As part of TS 29.561 clause 22.3.4 defines 3gpp:media-related-info-transform for sending Media Related Information using QUIC-aware proxying using HTTP as per draft-ietf-masque-quic-proxy.

During the HTTP CONNECT method,

- the UPF inserts 3gpp:media-related-info-transform in the accept-transform parameter of the proxy-quic-forwarding header field in the HTTP CONNECT request.

- the AS shall indicate 3gpp:media-related-info-transform in the transform parameter of the proxy-quic-forwarding header field in the HTTP CONNECT successful response.

3gpp:media-related-info-transform has to registered in IANA as per draft-ietf-masque-quic-proxy requirements.

When the Media Related Information are transformed in the QUIC short header as per IETF RFC 9000 [68], then the 3gpp:media-related-info-transform shall be used to encode the transformed QUIC packet as shown in Figure 22.3.4-1 (using the notational conventions defined in section 1.3 of IETF RFC 9000 [68]).

Transformed QUIC Packet {

Header Form (1) = 0,

Fixed Bit (1) = 1,

Spin Bit (1),

Reserved Bits (2),

Key Phase (1),

Packet Number Length (2),

Destination Connection ID (0..160),

Length of Protected Media Related Information(8)

Protected Media Related Information (..),

Packet Number (8..32),

Packet Payload (8..),

}

Figure 22.3.4-1: Transformed QUIC Packet with Media Related Information Container as per 3gpp:media-related-info-transform

As per TS 33.501 clause 18.2.3,

The defined packet transform below is named 3GPP_XRM_AESCCM_8 in the extended connect HTTP request negotiating the Forward Mode.

Editor’s note: The transformation label 3GPP_XRM_AESCCM_8 needs to be registered in IANA.

# CT3 protocol considerations:

From CT3 perspective, UPF indicates its support for 3gpp:media-related-info-transform in the HTTP CONNECT Request method and AS response with its support 3gpp:media-related-info-transform in the response method.

3gpp:media-related-info-transform covers the functional aspects for VCID, Packet number and Media Related Information which covers PDU set handling information, expedited transfer indication, time to next burst, burst size apart from the security aspects.

In terms of Rel-19, CCM is the only mechanism proposed in clause 18.2.4 of 3GPP TS 33.501. There is no need of security negotiation requirement for this release. There is no need of security mechanism specific information in the transformed QUIC packet. This is similar to the approach for the version handling of HTTP based QUIC aware proxy handling.

Even in case of the future 3GPP release with the alternative security mechanism like GCM is introduced, the security mechanism can be negotiated in HTTP CONNECT message.

# SA3 Naming issue:

In case of the alternative security mechanism like GCM is proposed in the future release of TS 33.501, the transform packet entries are not going to be modified. With these requirements, there is no need to IANA registration per security mechanism.

3GPP_XRM_AESCCM_8 naming does not distinguish media related information. In case any other transform service is added for XRM feature, this naming will not scale. The byte size (8) as the transform name leads to multiple transform names for the algorithm like GCM with varying length.

In CT3, it is concluded in Rel-18 that the feature name XRM was not accepted, renamed and the corresponding granular feature names are specified. The same approach has to be followed for the transform name as well.

# Conclusion:

CT3 solution of the single MRI packet transformation is the best solution which caters to the different functional and security requirements.

3gpp:media-related-info-transform has to be registered in IANA once draft-ietf-masque-quic-proxy is published as RFC.