3GPP TSG CT WG3 Meeting #143	C3-254147

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0431 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates and corrections to the Initiate_Authentication service operation of the AEF_Security_API |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | NBI19 |  |  |  |  |  | Date: |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | The following issues were identified in the provisions related to the authentication initiation procedure/service operation: - The was where the authentication initiation fails (e.g., the AEF is not able to get the authentication data from the CCF) is not covered as the "200 OK" status code response does not contain any attribute indicating the outcome/result of the authentication initiation. - This cannot be treated as an error case as it is neither a client-side error nor a server-side error, but rather one possible normal/successful outcome of this service operation, and it should hence be treated like that. - If this issue is not addressed, the definition of this service operation would remain incomplete and thus misaligned with the related stage 2 requirements. - There are some additional editorial issues, misalignments with the drafting rules, missing clauses in the data model section and various text enhancements that are needed. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed issues. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed issues and misalignments with stage 2 remain in the specification. |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.2.2.1, 6.2.2.2.1, 6.2.2.2.2, 9.1.2A.1, 9.1.2A.2.1, 9.1.2A.2.2, 9.1.4.1, 9.1.4.2.3, 9.1.4.3, 9.1.4.3.1 (new clause), 9.1.4.3.2 (new clause) , 9.1.4.3.2 (new clause) , 9.1.4.4 (new clause) , 9.1.4.5 (new clause), A.10 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces a backwards compatible new feature to the OpenAPI descriptions of the following APIs: - TS29222_AEF_Security_API.yaml |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

## 6.1	Introduction of Services

The table 6.1-1 lists the AEF APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 6.1-1: List of AEF Services

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| AEF_Security_API | Initiate_Authentication | Request/ Response | e.g., API Invoker |
|  | Revoke_Authorization | Request/ Response | e.g., CCF |

Table 6.1-2 summarizes the corresponding APIs defined in this specification.

Table 6.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| AEF_Security_API | 9.1 | AEF Security API Service | TS29222_AEF_Security_API.yaml | aef-security | A.10 |

* * * * Next changes * * * *

#### 6.2.2.1	Introduction

The service operation defined for AEF_Security_API is shown in table 6.2.2.1-1.

Table 6.2.2.1-1: Operations of the AEF_Security_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Initiate_Authentication | This service operation is used by a service consumer to request the AEF to confirm that the necessary authentication data is available to authenticate the API Invoker | e.g., API Invoker |
| Revoke_Authorization | This service operation is used by a service consumer to request the AEF to revoke the authorization of service API(s) for an API Invoker. | e.g., CCF |

* * * * Next changes * * * *

##### 6.2.2.2.1	General

This service operation is used by a service consumer to initiate authentication with the AEF. On receiving the Initiate_Authentication, the AEF fetches the authentication information of the API Invoker from the CCF, if required.

* * * * Next changes * * * *

##### 6.2.2.2.2	API invoker initiating authentication using Initiate_Authentication service operation

To initiate authentication with the AEF, the service consumer shall send an HTTP POST request message to the AEF targeting the URI "{apiRoot}/aef-security/v1/check-authentication", with the request body including the "CheckAuthenticationReq" data structure.

Upon receiving the above described HTTP POST request message, the AEF shall check if the credentials of the API invoker for authentication are available with the AEF. If the credentials of the API invoker for authentication are not available, the AEF shall use the service defined in clause 5.6.2.4.2 to fetch and store the credentials from the CCF.

The AEF shall then respond to the service consumer with an HTTP "200 OK" status code with the response body including the "CheckAuthenticationRsp" data structure indicating the result of the authentication initiation.

* * * * Next changes * * * *

#### 9.1.2A.1	Overview

Custom operations used for this API are summarized in table 9.1.2A.1-1. "{apiRoot}" and "<apiVersion>" are set as described in clause 7.5 and clause 9.1.1 respectively.

Table 9.1.2A.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| check-authentication | /check-authentication | POST | Enables a service consumer to initiate authentication. |
| revoke-authorization | /revoke-authorization | POST | Enables a service consumer to revoke authorization for service APIs. |

* * * * Next changes * * * *

##### 9.1.2A.2.1	Description

This custom operation allows the service consumer to initiate authentication at the AEF and receive an indication from the AEF on whether the necessary authentication data is available to authenticate the service consumer for API invocations.

* * * * Next changes * * * *

##### 9.1.2A.2.2	Operation Definition

This method shall support the URI query parameters specified in table 9.1.2A.2.2-1.

Table 9.1.2A.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This operation shall support the request and response data structures, and response codes specified in tables 9.1.2A.2.2-2 and 9.1.2A.2.2-3.

Table 9.1.2A.2.2-2: Data structures supported by the POST Request Body on this operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CheckAuthenticationReq | M | 1 | Contains the authentication initiation request data. |

Table 9.1.2A.2.2-3: Data structures supported by the POST Response Body on this operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CheckAuthenticationRsp | M | 1 | 200 OK | Successful case. The authentication initiation request was successfully processed and the result of the authentication initiation is returned in the response. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] shall also apply. |  |  |  |  |

Table 9.1.2A.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AEF. |

Table 9.1.2A.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AEF. |

* * * * Next changes * * * *

#### 9.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API.

Table 9.1.4.1-1 specifies the data types defined specifically for the AEF_Security_API service.

Table 9.1.4.1-1: AEF_Security_API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AuthResult | Clause 9.1.4.3.3 | Represents the result of the authentication initiation. |  |
| CheckAuthenticationReq | Clause 9.1.4.2.2 | Represents authentication initiation request data. |  |
| CheckAuthenticationRsp | Clause 9.1.4.2.3 | Represents authentication initiation response data. |  |
| RevokeAuthorizationReq | Clause 9.1.4.2.4 | Represents authorization revocation request data. |  |
| RevokeAuthorizationRsp | Clause 9.1.4.2.5 | Represents authorization revocation response data. |  |

Table 9.1.4.1-2 specifies data types re-used by the AEF_Security_API service.

Table 9.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SecurityNotification | Clause 8.5.4.2.5 | Represents information about the revoked APIs. |  |
| SupportedFeatures | 3GPP TS 29.571 [19] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features defined in Table 9.1.6-1. |  |

* * * * Next changes * * * *

##### 9.1.4.2.3	Type: CheckAuthenticationRsp

Table 9.1.4.2.3-1: Definition of type CheckAuthenticationRsp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | AuthResult | C | 0..1 | Contains the result of the authentication initiation. This attribute shall be present to indicate the result of the authentication initiation. |  |
| supportedFeatures | SupportedFeatures | M | 1 | Used to negotiate the supported optional features of the API as described in clause 7.8. |  |

* * * * Next changes * * * *

#### 9.1.4.3	Simple data types and enumerations

##### 9.1.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 9.1.4.3.2	Simple data types

The simple data types defined in table 9.1.4.3.2-1 shall be supported.

Table 9.1.4.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

* * * * Next changes * * * *

##### 9.1.4.3.3	Enumeration: AuthResult

The enumeration AuthResult represents the result of the authentication initiation. It shall comply with the provisions defined in table 9.1.4.3.3-1.

Table 9.1.4.3.3-1: Enumeration AuthResult

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESS | Indicates that the authentication initiation was successful. |  |
| FAILURE | Indicates that the authentication initiation failed. |  |

* * * * Next changes * * * *

#### 9.1.4.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

* * * * Next changes * * * *

#### 9.1.4.5	Binary data

##### 9.1.4.5.1	Binary Data Types

Table 9.1.4.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

* * * * Next changes * * * *

# A.10	AEF_Security_API

openapi: 3.0.0

info:

title: AEF_Security_API

description: |

API for AEF security management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.1"

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/aef-security/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/check-authentication:

post:

summary: Check authentication.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CheckAuthenticationReq'

responses:

'200':

description: The request was successful.

content:

application/json:

schema:

$ref: '#/components/schemas/CheckAuthenticationRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/revoke-authorization:

post:

summary: Revoke authorization.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/RevokeAuthorizationReq'

responses:

'200':

description: The request was successful.

content:

application/json:

schema:

$ref: '#/components/schemas/RevokeAuthorizationRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

#

# STRUCTURED DATA TYPES

#

schemas:

CheckAuthenticationReq:

type: object

description: Represents authentication check request data.

properties:

apiInvokerId:

type: string

description: >

API invoker ID assigned by the CAPIF core function to the API invoker

while on-boarding the API invoker.

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- apiInvokerId

- supportedFeatures

CheckAuthenticationRsp:

type: object

description: Represents authentication check response data.

properties:

result:

$ref: '#/components/schemas/AuthResult'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supportedFeatures

RevokeAuthorizationReq:

type: object

description: Represents authorization revocation request data.

properties:

revokeInfo:

$ref: 'TS29222_CAPIF_Security_API.yaml#/components/schemas/SecurityNotification'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- revokeInfo

- supportedFeatures

RevokeAuthorizationRsp:

type: object

description: Represents authorization revocation response data.

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supportedFeatures

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AuthResult:

anyOf:

- type: string

enum:

- SUCCESS

- FAILURE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the result of the authentication initiation.

Possible values are:

- SUCCESS: Indicates that the authentication initiation was successful.

- FAILURE: Indicates that the authentication initiation failed.

# Data types describing alternative data types or combinations of data types:

#

* * * * End of changes * * * *