3GPP TSG CT WG3 Meeting #143	C3-254153

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.486 | CR | 0142 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to support CAPIF-based resource and operation level access control |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | With the support of resource and operation level access control and RNAA use cases in CAPIF, the provisions of clause 7.2 needs to be updated to: - Remove the restriction of only having API level access control. - Remove the restriction on the support of only the "Client Credentials" grant type for OAuth2. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed necessary enhancements. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed necessary enhancements are not addressed and the definition of resource and operation level access control and RNAA use cases in CAPIF is not completed is not completed in stage 3. |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions of the APIs. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

## 8.2	Security

When CAPIF is used for managing the exposure of the VAE Server APIs, before invoking an API exposed by the VAE Server, the service consumer (e.g., VASS), acting as a CAPIF API Invoker, shall negotiate the security method (PKI, TLS-PSK or OAuth2.02) with the CAPIF Core Function and ensure that the VAE Server has enough credentials to authenticate the service consumer (e.g., VASS) as defined in clauses 5.6.2.2 and 6.2.2.2 of 3GPP TS 29.222 [26].

If the PKI or TLS-PSK is used as the selected security method between the service consumer and the VAE Server, upon API invocation, the VAE Server shall retrieve the authorization information from the CAPIF Core Function as described in clause 5.6.2.4 of 3GPP TS 29.222 [26].

As indicated in 3GPP TS 33.122 [27], the access to the VAE Server APIs may be authorized by means of the OAuth2.0 protocol (see IETF RFC 6749 [23]), where the CAPIF Core Function (see 3GPP TS 29.222 [26]) plays the role of the authorization server.

If OAuth2.0 is used as the selected security method between the service consumer and the VAE Server:

-	the service consumer shall, prior to consuming the services offered by the VAE Server, obtain a "token" from the authorization server, by invoking the Obtain_Authorization service as described in clause 5.6.2.3.2 of 3GPP TS 29.222 [26];

-	it is the VAE Server responsibility to check whether the service consumer is authorized to use an API (or particular resource(s) or operation(s) defined within an API) based on the OAuth2.0 "token";

-	once the VAE Server verifies the "token", it shall check whether the VAE Server identifier in the "token" matches its own published identifier, whether the API name in the "token" matches its own published API name and whether the granted scope (e.g., API level, resource level, operation level) in the "token" is authorized; and

-	if those checks are passed, the service consumer has full authority to access any resource(s) or operation(s) of the invoked service API and that are within the boundaries of the granted scope in the "token".

NOTE:	For the aforementioned security requirements, the VAE Server needs to apply admission control according to access control policies after performing the authorization checks.

* * * * End of changes * * * *