3GPP TSG CT WG3 Meeting #143	C3-254183

# Sophia-Antipolis, France, 13 – 17 October, 2025

Source:	Samsung

Title:	Pseudo-CR on defining the OpenAPI description of the SS_SmDataSourceDiscovery API

Spec:	3GPP TS 29.437 (V1.1.0)

Agenda item:	19.42

Document for:	Approval

1. Introduction

The stage 2 requirements for the new SS_SmDataSourceDiscovery API have been defined in clauses 9.5 and 9.7.6 of TS 23.437. The API data model and the related descriptions of the service operations of the API are specified in TS 29.437.

2. Reason for Change

Define the OpenAPI description of this new API in the corresponding new Metaverse applications Services TS 29.437.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.437 V 1.1.0.

* * * First Change * * * *

## 5.1	Introduction

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| <service name> | <ref clause> | <short description as included in the OpenAPI file> | <file name> | <apiName in the URI> | <ref Annex> |
| SS_SmDataSourceDiscovery | 6.2.4 | SM Data Source Discovery | TS29437_SS_SmDataSourceDiscovery.yaml | ssm-dsdisc | A.X |
| SS_SmSmasRegistration | 6.2.5 | SMAS Registration Management | TS29437_SS_SmSmasRegistration.yaml | ssm-smasr | A.8 |

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e., SAn Server, SM Server) takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

* * * Next Change * * * *

# A.X	SS_SmDataSourceDiscovery API

openapi: 3.0.0

info:

title: SM data source discovery

version: 1.0.0-alpha.1

description: |

SM Data Source Discovery.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.437 V1.2.0; Service Enabler Architecture Layer for Verticals (SEAL);

Metaverse Enablement Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.437/

servers:

- url: '{apiRoot}/ssm-dsdisc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/ datasource-profiles:

get:

summary: Retrieve Spatial Map data source profiles information.

operationId: GetSpatialMapDataSourceDIscovery

tags:

- Spatial Maps Data Source Profiles

parameters:

- name: area-int

in: query

description: Contains the three-dimensional area or localization information.

content:

application/json:

schema:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

- name: end-point

in: query

description: >

Contains the end point information where the Spatial Map data source sends the SM data.

content:

application/json:

schema:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

- name: ds-disc-fltr

in: query

description: Contains the set of characteristics related to Spatial Map data sources.

content:

application/json:

schema:

$ref: '#/components/schemas/SMDataSourceDiscFilter'

- name: supp-feats

in: query

description: >

Contains the list of supported feature(s) among the ones defined in clause 6.1.2.8.

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The requested SM data source information shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SMDataSourceDiscResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SMDataSourceDiscFilter:

description: Represents the SM data source discovery filters.

type: object

properties:

smInfoDets:

type: array

items:

$ref: 'TS24550_SS_SmDataSourceRegistration.yaml#/components/schemas/SpatialMapInfoDetails'

minItems: 1

required:

- smInfoDets

SMDataSourceDiscResp:

description: >

Represents the SM data source discovery response. Represents the SM data source discovery response.

type: object

properties:

results:

type: array

items:

$ref: 'TS24550_SS_SmDataSourceRegistration.yaml#/components/schemas/DataSourceProfile'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- results

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

#

# Data types describing alternative data types or combinations of data types:

#

* * * * End of Changes * * * *