3GPP TSG-CT WG3 Meeting #143	C3-254192

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.569 | CR | 0008 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of failure report in AIOT notify |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 23.369, 6.2.3 Command Procedure If none of successful Inventory response is received, step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in step 12. 7.4.4 Nnef_AIoT_Notify service operation Inputs, Optional: 1) List of AIoT Device ID or Failure Cause in case of Failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To support of failure report in AIOT notify: - add failCause attribute, of type FailureCause, to AIoTNotif data structure - define Enumeration FailureCause |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with SA2 requiement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.4, 6.1.6.2.6. 6.1.6.3.3, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following API: - TS29569_Naiotf_AIoT.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Naiotf_AIoT service-based interface protocol.

Table 6.1.6.1-1: Naiotf_AIoT specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AIoTDevices | 6.1.6.2.7 | Represents the AIoT device(s) related information. |  |
| AIoTNotif | 6.1.6.2.6 | Represents the AIoT Operations Notification. |  |
| CommandReq | 6.1.6.2.4 | Represents the AIoT Command request. |  |
| CommandResp | 6.1.6.2.5 | Represents the AIoT Command response. |  |
| DevicesRepInfo | 6.1.6.2.8 | Represents the AIoT device(s) related reporting information. |  |
| FailureCause | 6.1.6.3.3 | Represents the failure cause of the enforcement of the AIoT Inventory/Command request. |  |
| InventoryReq | 6.1.6.2.2 | Represents the AIoT Inventory request. |  |
| InventoryResp | 6.1.6.2.3 | Represents the AIoT Inventory response. |  |

Table 6.1.6.1-2 specifies data types re-used by the Naiotf_AIoT service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naiotf_AIoT service-based interface.

Table 6.1.6.1-2: Naiotf_AIoT re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AiotArea | 3GPP TS 29.571 [16] | Represents the AIoT Service Area. |  |
| AiotDevPermId | 3GPP TS 29.571 [16] | Represents the permanent identifier of the AIoT Device. |  |
| AiotFilteringInformation | 3GPP TS 29.571 [16] | Represents the the filtering information used for identifying the target AIoT device(s). |  |
| Bytes | 3GPP TS 29.571 [16] | Represents a sequence of bytes. |  |
| CommandType | 3GPP TS 29.522 [15] | Represents the type of AIoT Command. |  |
| DurationSec | 3GPP TS 29.571 [16] | Represents a time duration in units of seconds. |  |
| ProblemDetails | 3GPP TS 29.571 [16] | Represents error related information. |  |
| RedirectResponse | 3GPP TS 29.571 [16] | Contains redirection related information. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported feature(s) and is used to negotiate the applicability of optional features. |  |
| Uinteger | 3GPP TS 29.571 [16] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |

# *** 2nd Change ***

6.1.6.2.4	Type: CommandReq

Table 6.1.6.2.4-1: Definition of type CommandReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that triggered the request. |  |
| commandType | CommandType | M | 1 | Contains the type of the requested command. |  |
| targetArea | AiotArea | C | 0..1 | Contains the target area within which the requested Command operation shall apply. (NOTE) |  |
| targetDevices | AIoTDevices | C | 0..1 | Contains the target AIoT device(s) related information. (NOTE) |  |
| numDevices | Uinteger | O | 0..1 | Contains the approximative number of the targeted AIoT device(s). |  |
| msgSize | Uinteger | O | 0..1 | Contains the approximative message size in units of Bytes. This attribute may be present only if the "commandType" attribute is set to "READ". |  |
| offset | Uinteger | C | 0..1 | Contains the offset, expressed in units of bytes. This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the offset from which to read the application data. - If the "commandType" attribute is set to "WRITE", this attribute contains the offset from which to write the application data. |  |
| length | Uinteger | C | 0..1 | Contains the length of application data, expressed in units of bytes (i.e., byte length). This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the length of application data to read. - If the "commandType" attribute is set to "WRITE", this attribute contains the length of application data to write. |  |
| data | Bytes | C | 0..1 | Contains the application data to write. This attribute shall be present only if the "commandType" attribute is set to "WRITE". |  |
| notifUri | Uri | M | 1 | Contains the URI via which the AIoT Command operation related notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** 3rd Change ***

6.1.6.2.6	Type: AIoTNotif

Table 6.1.6.2.6-1: Definition of type AIoTNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| transId | string | M | 1 | Contains the identifier of the AIoT transaction to which the notification is related. |  |
| devicesRepData | array(DevicesRepInfo) | C | 1..N | Contains the AIoT device(s) related reporting information. (NOTE) |  |
| lastRepInd | boolean | C | 0..1 | Indicates that this is the last reporting from the AIOTF for the AIoT service operation identified by the "transId" attribute. - "true" indicates that this is the last report. This attribute shall be present only when this is the last reporting from the AIOTF for the AIoT service operation identified by the "transId" attribute. When present, this attribute shall be set to "true". The presence of this attribute set to the value "false" is forbidden. (NOTE) |  |
| failCause | FailureCause | C | 0..1 | Contains the failure cause of the enforcement of the AIoT Inventory/Command request. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. The "failCause" attribute and "devicesRepData" attribute are mutually exclusive. The "failCause"" attribute and "lastRepInd" attribute are mutually exclusive. |  |  |  |  |  |

# *** 4th Change ***

6.1.6.3.3	Enumeration: FailureCause

The enumeration "FailureCause" represents the failure cause of the enforcement of the AIoT Inventory/Command request. It shall comply with the provisions of table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration FailureCause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| NO_AIOT_DEVICE | Indicates that none of successful Inventory response is received. |  |

# *** 5th Change ***

A.2	Naiotf_AIoT API

openapi: 3.0.0

info:

title: AIOTF Ambient IoT Service

version: 1.0.0-alpha.5

description: |

API for the Naiotf_AIoT Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.569 V19.0.0; Ambient IoT Function (AIOTF) Services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.569/

servers:

- url: '{apiRoot}/naiotf-aiot/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

paths:

/request-inv:

post:

summary: Request to perform an AIoT Inventory operation.

operationId: InventoryRequest

tags:

- AIoT Inventory Request (custom operation without associated resources)

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

- oAuth2ClientCredentials:

- naiotf-aiot

- naiotf-aiot:inventory

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryReq'

responses:

'200':

description: >

OK. The AIoT Inventory request is successfully received and processed, and

AIoT Inventory related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/request-cmd:

post:

summary: Request to perform an AIoT Command operation.

operationId: CommandRequest

tags:

- AIoT Command Request (custom operation without associated resources)

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

- oAuth2ClientCredentials:

- naiotf-aiot

- naiotf-aiot:command

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CommandReq'

responses:

'200':

description: >

OK. The AIoT Command request is successfully received and processed, and

AIoT Command related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CommandResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naiotf-aiot: >

Enables to access all the resources and custom operations of the Naiotf_AIoT API.

naiotf-aiot:inventory: >

Enables to access only the InventoryRequest custom operation (Naiotf_AIoT_Inventory

service operation) of the Naiotf_AIoT API.

naiotf-aiot:command: >

Enables to access only the CommandRequest custom operation (Naiotf_AIoT_Command

service operation) of the Naiotf_AIoT API.

schemas:

#

# STRUCTURED DATA TYPES

#

InventoryReq:

description: Represents the AIoT Inventory request.

type: object

properties:

afId:

type: string

targetArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

targetDevices:

$ref: '#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeInterval:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

InventoryResp:

description: Represents the AIoT Inventory response.

type: object

properties:

transId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- transId

CommandReq:

description: Represents the AIoT Command request.

type: object

properties:

afId:

type: string

commandType:

$ref: 'TS29522_AIoT.yaml#/components/schemas/CommandType'

targetArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

targetDevices:

$ref: '#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

msgSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

offset:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

length:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

data:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- commandType

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

CommandResp:

description: Represents the AIoT Command response.

type: object

properties:

transId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- transId

AIoTNotif:

description: Represents the AIoT Operations Notification.

type: object

properties:

transId:

type: string

devicesRepData:

type: array

items:

$ref: '#/components/schemas/DevicesRepInfo'

minItems: 1

lastRepInd:

type: boolean

enum:

- true

description: >

Indicates that this is the last reporting from the AIOTF for the AIoT service

operation identified by the "transId" attribute.

true indicates that this is the last report.

This attribute shall be present only when this is the last reporting from the AIOTF

for the AIoT service operation identified by the transId attribute.

When present, this attribute shall be set to true. The presence of this attribute set

to the value false is forbidden.

failCause:

$ref: '#/components/schemas/FailureCause'

required:

- transId

oneOf:

- required: [devicesRepData]

- required: [lastRepInd]

- required: [devicesRepData, lastRepInd]

- required: [failCause]

AIoTDevices:

description: Represents the AIoT device(s) related information.

type: object

properties:

devices:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

minItems: 1

filteringInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotFilteringInformation'

oneOf:

- required: [devices]

- required: [filteringInfo]

DevicesRepInfo:

description: Represents the AIoT device(s) related reporting information.

type: object

properties:

deviceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

readCmdRep:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

required:

- deviceId

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

FailureCause:

anyOf:

- type: string

enum:

- NO_AIOT_DEVICE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the failure cause of the enforcement of the AIoT Inventory/Command request.

Possible values are:

- NO_AIOT_DEVICE: Indicates that none of successful Inventory response is received.

#

# Data types describing alternative data types or combinations of data types

#

*** End of Changes ***