3GPP TSG-CT WG3 Meeting #143	C3-254231

Sophia-Antipolis, France, 13 – 17 October 2025	(Revision of C3-25xxxx))

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.591 | CR | 0250 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Limited alignment for VFL Inference with VFL Training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Nnef_VFLInference service is not currently on 3GPP TS 29.581 but when introduced it is expected to be aligned with Nnwdaf_VFLTraining service on TS 29.520 and with Nnef_VFLTraining service on TS 29.522. Nnef_VFLInference service in TS 29.591 is not aligned with VFL Training services above in following issues: - Support of GET operation for individual subscriptions. - Possibility of updating VFL inference subscriptions per analytics event in Patch operation. - The name of the attribute for reporting requirements in the VflInferSub and vflInferSubPatch data types ("vflReportInfo" attribute) does not reflect that it is a requirement. - Different naming on the attribute that indicates the external identity of the VFL Client (in Nnef_VFLTraining service on TS 29.522 it is called extNwdafId since the VFL Client there is a NWDAF, so in 29.591 needs to be adapted since the VFL Client is an AF). - Different names are used for the attributes corresponding to the notification correlation id where applicable. - The parameters section in on the TS29591_Nnef_VFLInference.yaml is specific per operation. It is considered convenient that Nnef_VFLInference service is updated to be aligned with VFL Training services in all these issues. Moreover, the VflInferSubPatch data type has the "notifUri" attribute repeated on clause 5.7.6.2.3. One of them shall be the "notifCorrId" attribute. |  |  |  |  |  |  |  |  |
| Summary of change: |  | GET operation is added to the "Individual VFL Inference Subscription" resource. "vflInferAnaSubs" attribute is added to the VflInferSubPatch data type. "vflReportInfo" attribute is replaced by "reportingReqs" attribute in VflInferSub and VflInferSubPatch data types. "vflClientId" attribute is replaced by "afId" attribute in VflInferSub data type. "notifCorreId" attribute is replaced by "notifCorrId" attribute in VflInferSub and VflInferSubPatch data types. The parameters section is moved on the TS29591_Nnef_VFLInference.yaml API from the PUT, PATCH and DELETE operations to make it general under the /subscriptions/{subscriptionId} resource. One of the "notifUri" attributes on clause 5.7.6.2.3 is replaced by the "notifCorrId" attribute. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No alignment of VFL Training and VFL Inference services can make their handling more complex making it prone to errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible feature in the OpenAPI file of TS29591_Nnef_VFLInference.yaml API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GMS	5G Media Streaming

AF	Application Function

AnLF	Analytics Logical Function

API	Application Programming Interface

ASP	Application Service Provider

DCCF	Data Collection Coordination Function

EAS	Edge Application Server

EHE	Edge Hosting Environment

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

I-SMF	Intermediate SMF

LCS	LoCation Services

LMF	Location Management Function

MFAF	Messaging Framework Adaptor Function

NEF	Network Exposure Function

NF	Network Function

NWDAF	Network Data Analytics Function

SMF	Session Management Function

SUPI	Subscription Permanent Identifier

URI	Uniform Resource Identifier

VFL	Vertical Federated Learning

# *** Next Change ***

## 4.1	Introduction

The NEF offers to other NFs the following southbound services:

-	Nnef_EventExposure

-	Nnef_PFDManagement

-	Nnef_SMContext

-	Nnef_SMService

-	Nnef_Authentication

-	Nnef_EASDeployment

-	Nnef_TrafficInfluenceData

-	Nnef_ECSAddress

-	Nnef_UEId

-	Nnef_Inference

-	Nnef_VFLInference

NOTE 1:	The northbound services offered by the NEF are defined in 3GPP TS 29.522 [15], e.g. the northbound requirement of Nnef_EventExposure service or Nnef_EASDeployment.

NOTE 2:	The services offered by the NEF (e.g. Nnef_EventExposure service) as specified in the present specification are only applicable for Nnef southbound services.

NOTE 3:	The Nnef_PFDManagement service offered by the NEF southbound is defined in 3GPP TS 29.551 [19].

NOTE 4:	The Nnef_SMContext service and the Nnef_SMService offered by the NEF southbound is defined in 3GPP TS 29.541 [20].

NOTE 5:	The Nnef_Authentication service offered by the NEF southbound is defined in 3GPP TS 29.256 [23].

# *** Next Change ***

#### 5.7.3.1	Resource Structure

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 5.7.3.1-1 depicts the resource URIs structure for the Nnef_VFLInference API.

Figure 5.7.3.1-1: Resource URI structure of the Nnef_VFLInference API

Table 5.7.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.7.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| NEF VFL Inference Subscriptions | /subscriptions | POST | Creates a new Individual NEF VFL Inference Subscription resource. |
| Individual NEF VFL Inference Subscription | /subscriptions/{subscriptionId} | GET | Retrieves an existing Individual VFL Inference Subscription identified by subresource {subscriptionId}. |
|  |  | PUT | Updates an existing Individual NEF VFL Inference Subscription identified by subresource {subscriptionId}. |
|  |  | PATCH | Modifies an existing Individual NEF VFL Inference Subscription identified by subresource {subscriptionId}. |
|  |  | DELETE | Deletes an Individual NEF VFL Inference Subscription identified by subresource {subscriptionId}. |

# *** Next Change ***

###### 5.7.3.3.3.1	GET

This method shall support the URI query parameters specified in table 5.7.3.3.3.1-1.

Table 5.7.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ###### supp-feat | ###### SupportedFeatures | ###### O | ###### 0..1 | ###### The features supported by the NF service consumer. |

This method shall support the request data structures specified in table 5.7.3.3.3.1-2 and the response data structures and response codes specified in table 5.7.3.3.3.1-3.

Table 5.7.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.7.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 200 OK | Contains the representation of the Individual VFL Inference Subscription resource. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. Applicable if the feature "ES3XX" is supported. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. Applicable if the feature "ES3XX" is supported. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 5.7.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. |

Table 5.7.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. |

# *** Next Change ***

###### 5.7.3.3.3.2	PUT

This method shall support the URI query parameters specified in table 5.7.3.3.3.2-1.

Table 5.7.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.7.3.3.3.2-2 and the response data structures and response codes specified in table 5.7.3.3.3.2-3.

Table 5.7.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferSub | M | 1 | Parameters to replace a subscription to NEF VFL Subscription resource. |

Table 5.7.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 200 OK | Successful case. The Individual VFL Inference Subscription resource was modified and a representation is returned. "exterGroupIds" and "gpsis" target VFL identities on " vflInferAnaSub" attribute under this structure are not applicable to this API. |
| n/a |  |  | 204 No Content | Successful case. The Individual VFL Inference Subscription resource was modified. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during subscription modification. (NOTE 3) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during subscription modification. (NOTE 3) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in clause 5.7.7. NOTE 3: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 5.7.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

Table 5.7.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

# *** Next Change ***

###### 5.7.3.3.3.3	PATCH

This method shall support the URI query parameters specified in table 5.7.3.3.3.3-1.

Table 5.7.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.7.3.3.3.3-2 and the response data structures and response codes specified in table 5.7.3.3.3.3-3.

Table 5.7.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferSubPatch | M | 1 | Partially modify an existing subscription to VFL Inference events. |

Table 5.7.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 200 OK | The Individual NEF VFL Inference Subscription resource was partial modified successfully and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | The Individual NEF VFL Inference Subscription resource was partial modified successfully.and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 5.7.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |

Table 5.7.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. |

# *** Next Change ***

###### 5.7.3.3.3.4	DELETE

This method shall support the URI query parameters specified in table 5.7.3.3.3.4-1.

Table 5.7.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.7.3.3.3.4-2 and the response data structures and response codes specified in table 5.7.3.3.3.4-3.

Table 5.7.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.7.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Individual VFL Inference Subscription resource matching the subscriptionId was deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during subscription termination. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during subscription termination. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 5.7.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. |

Table 5.7.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. |

# *** Next Change ***

##### 5.7.6.2.2	Type VflInferSub

Table 5.7.6.2.2-1: Definition of type VflInferSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorrId | string | M | 1 | The value of Notification Correlation ID in the corresponding notification. |  |
| notifUri | Uri | M | 1 | URI at which the NF service consumer requests to receive notifications. |  |
| afId | string | O | 0..1 | Identity of the AF acting as VFL Client. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Reporting requirements of the VFL inference subscription. |  |
| suppFeats | SupportedFeatures | C | 0..1 | List of Supported features used as described in clause 5.7.8. It shall be supplied by NF service consumer in the POST requests that request the creation of an NEF VFL Subscriptions resource and shall be supplied by the NEF in the reply of corresponding request. |  |
| vflInferAnaSubs | array(VflInferAnaSub) | M | 1..N | Contains the VFL inference subscription information for the subscribed analytics ID(s). |  |
| vflInferResults | array(VflInferResult) | O | 1..N | Represents intermediate VFL inference results |  |

# *** Next Change ***

##### 5.7.6.2.3	Type VflInferSubPatch

Table 5.7.6.2.3-1: Definition of type VflInferSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorrId | string | O | 0..1 | The value of Notification Correlation ID in the corresponding notification. |  |
| notifUri | Uri | O | 0..1 | URI at which the NF service consumer requests to receive notifications. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Reporting requirements of the VFL inference subscription. |  |
| vflInferAnaSubs | array(VflInferAnaSub) | O | 1..N | Contains the updated VFL inference subscription information for the subscribed analytics ID(s). |  |
| vflInferReq | VflInferReq | O | 0..1 | Represents required conditions to apply VFL inference. |  |

# *** Next Change ***

# A.8	Nnef_VFLInference API

openapi: 3.0.0

info:

title: Nnef_VFLInference

version: 1.0.0-alpha.1

description: |

API for Nnef_VFLInference Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.591 V19.4.0; 5G System; Network Exposure Function Southbound Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.591/

servers:

- url: '{apiRoot}/nnef-vfl-inference/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-vfl-inference

paths:

/subscriptions:

post:

summary: Create a new Individual VFL Inference Subscription resource.

operationId: CreateVFLInferenceSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

responses:

'201':

description: Create a new Individual VFL Inference Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the

structure

{apiRoot}/nnef-vfl-inference/v1/subscriptions/{subscriptionId}.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferNotif'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: String identifying an Individual VFL Inference Subscription.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual VFL Inference Subscription resource.

operationId: GetVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual VFL Inference Subscription resource is

successfully returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual VFL Inference Subscription

operationId: UpdateVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

responses:

'200':

description: >

The Individual VFL Inference Subscription resource was modified

successfully and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

'204':

description: >

The Individual VFL Inference Subscription resource was modified

successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Partial update an existing Individual VFL Inference Subscription

operationId: PartialUpdateVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/VflInferSubPatch'

responses:

'200':

description: >

The Individual VFL Inference Subscription resource was partial

modified successfully and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

'204':

description: >

The Individual VFL Inference Subscription resource was partial

modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual VFL Inference Subscription.

operationId: DeleteVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

responses:

'204':

description: >

No Content. The Individual VFL Inference Subscription matching the

subscriptionId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-vfl-inference: Access to the Nnef_VFLInference API

schemas:

VflInferSub:

description: Represents a VFL Inference subscription.

type: object

properties:

notifCorrId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding

notification.

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

afId:

type: string

description: Identitty of the AF acting as VFL client.

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

vflInferAnaSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferAnaSub'

minItems: 1

description: Represents inference subscription per analytics Id.

vflInferReq:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferReq'

vflInferResults:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferResult'

minItems: 1

description: Represents intermediate VFL Inference result.

required:

- notifUri

- notifCorrId

- vflInferAnaSubs

VflInferSubPatch:

description: >

Represents parameters to request the modification of a VFL Inference

subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding

notification.

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

vflInferAnaSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferAnaSub'

minItems: 1

description: Represents inference subscription per analytics Id.

vflInferReq:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferReq'

# *** End of Changes ***