# 3GPP TSG CT WG3 Meeting #143	C3-254255

# Sophia-Antipolis, France, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.514 | CR | 0804 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the congestReports |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, XRM |  |  |  |  |  | Date: |  |  | 2025-9-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In table 5.6.2.37 of TS 29.514, it specified the congestion information should only contain one element as speficied in clause 4.2.5.14: However in clause 4.2.5.14, the procedure description still indicates multiple congestion information could be provided in the "ulConInfo" and "dlConInfo" attributes. The fact is from the SMF, the PCF can only receive one report for congestion during each periodic or event-triggered report, as specified in TS 29.512 clause 5.6.2.42. And since the OpenAPI does not put the limits on maximum number of congestion information within the array in TS 29.514. The current procedure is not aligned with the data table and can cause incorrect implementation. Checking the current implementation for the similar attributes "ulDelays", "dlDelays", "rtDelays", a Note is added for clarify the usage of these attributes is limited to 1 element. A similar Note is needed for the congestion reporting as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the procedure with the data type, and also clearly indicate to the OpenAPI that the maximum number should be 1, even it was missing in the OpenAPI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect procedure description can lead to implementation errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has no impact on OpenAPI |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.2.5.14	Notification about Service Data Flow QoS Monitoring control

When the PCF gets the information about real-time measurements of QoS monitoring parameters for one or more SDFs from the SMF (e.g. for QoS monitoring for packet delay, uplink packet delay(s), downlink packet delay(s) and/or round trip delay(s) or if the feature "PacketDelayFailureReport" is supported, indicator of packet delay measurement failure) the PCF shall inform the NF service consumer accordingly if the NF service consumer has previously subscribed as described in clauses 4.2.2.23 and 4.2.3.23 and 4.2.6.8.

The PCF shall notify the NF service consumer of the QoS monitoring events by including the "EventsNotification" data type in the body of the HTTP POST request as described in clause 4.2.5.2.

The PCF shall include:

-	within the "evNotifs" attribute an event entry of the "AfEventNotification" data type with the matched event "QOS_MONITORING" in the "event" attribute; and

-	for QoS monitoring for packet delay, one or more entries of the "qosMonReports" array, where each entry shall contain:

1.	the identification of the affected service flows (if not all the flows are affected) encoded in the "flows" attribute if applicable; and:

2.	the received packet delay measurement:

a)	the uplink packet delays within the "ulDelays" attribute; and/or

b)	the downlink packet delays within the "dlDelays" attribute; and/or

c)	the round trip packet delays within the "rtDelays" attribute; or

d)	if the feature "PacketDelayFailureReport" is supported, the packet delay measurement failure indicator within the"pdmf" attribute; and/or

NOTE 1:	The SMF reports one UL, DL and/or round-trip packet delay measurement for each periodic and/or event-triggered report as described in 3GPP TS 29.512 [8]. I.e, the PCF can include only one element within the "ulDelays", "dlDelays", and/or "rtDelays" array(s) respectively, each one with the received report from the SMF for the UL, DL and/or round trip delay(s).

-	if the feature "EnQoSMon" is supported, to report data rate measurements, one or more entries of the "qosMonDatRateReps" array, where each entry shall contain:

1.	the identification of the affected service flows (if not all the flows are affected) encoded in the "flows" attribute if applicable; and

2.	the received data rate measurement:

a)	one data rate measurement for the UL within the "ulDataRate" attribute; and/or

b)	one data rate measurement for the DL within the "dlDataRate" attribute.

-	if the feature "EnQoSMon" is supported, for QoS monitoring report for congestion information, one or more entries of the "congestReports" array, where each entry shall contain:

1.	the identification of the affected service flows (if not all the flows are affected) encoded in the "flows" attribute if applicable;

2.	the received congestion measurement:

a)	the uplink congestion information measurement(s) within the "ulConInfo" attribute; and/or

b)	the downlink congestion information measurement(s) within the "dlConInfo" attribute.

NOTE 2:	The SMF reports one UL, DL congestion measurement for each periodic and/or event-triggered report as described in 3GPP TS 29.512 [8]. I.e, the PCF can include only one element within the "ulConInfo", "dlConInfo", each one with the received report from the SMF for the UL, DL congestion.

# *** End of Changes ***