3GPP TSG-CT3 Meeting #143	C3-254290

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

Title:	Reply on LS to 3GPP CT1 and CT3 on Reserved QoS Rule Precedence Values

Response to:	LS to 3GPP CT1 and CT3 on Reserved QoS Rule Precedence Values (C3-253030, C1-255326)

Release:	Rel-19

Work Item:	-

Source:	CT3

To:	GSMA Terminal Steering Group, CT1

Cc:

Contact person:	Bhaskar Paul

bhaskar.paul@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR 1426 (C3-254289) for 3GPP TS 29.512 v19.4.0

# 1	Overall description

CT3 thanks GSMA’s Terminal Steering Group (TSG) for their LS on Reserved QoS Rule Precedence Values (C3-253030).

CT3 also thanks CT1 for their LS on Reserved QoS Rule Precedence Values (C1-255326).

CT3 discussed the following issues from the respective received LS(s) and has agreed the attached CR to address the concerns of GSMA:

- LS C3-253030

## Observation on 3GPP TS 29.512: The Precedence value for PccRule QoS Flows is defined in 3GPP TS 29.512 v18.9.0 Table 5.6.2.6-1 as: Attribute name; Data type; P; Cardinality; Description; Applicability. ⁞; ⁞; ⁞; ⁞; ⁞; ⁞. precedence; Uinteger; O; 0..1; Determines the order in which this PCC rule is applied relative to other PCC rules within the same PDU session. It shall be included if the "flowInfos" attribute is included or may be included if the "appId" attribute is included when the PCF initially provisions the PCC rule. (NOTE 2) (NOTE 4). ⁞; ⁞; ⁞; ⁞; ⁞; ⁞. ⁞ NOTE 2: For a PCC rule with the "appId" attribute, the precedence can be preconfigured in SMF or provided in the PCC rule from PCF. The precedence provided by the PCF shall take precedence. ⁞ NOTE 4: The "precedence" attribute is used to specify the precedence of the PCC rule among all PCC rules associated with the PDU session. It includes an integer value in the range from 0 to 255 (decimal). The higher the value of the "precedence" attribute, the lower the precedence of that PCC rule is. The precedence value range from 70 to 99 (decimal) shall be used for the PCC rules subject to Reflective QoS. ⁞. Based on definition in 3GPP TS 29.512, there does not appear to be any reserved ranges/values for the Precedence attribute. ## Observation on 3GPP TS 24.501: This additional requirement of “The value 80 (decimal) is reserved”, which is not present in 3GPP TS 29.512 or the previous reference is the source of the perceived ambiguity as some device vendors will not accept QoS Flow Rules with this value (80) because it’s defined as reserved.

- LS C1-255326

From CT1 perspective, - Precedence value 80 (decimal) is only used for a derived QoS rule. Derived QoS rule is used only when UE and network support reflective QoS. The derived QoS rule is only used locally in the UE and will not be included in any NAS signalling exchanged between UE and network. - For signalled QoS rules, CT1 decided to define the codepoint for precedence value 80 (decimal) as 'reserved' since Rel-15, to prevent the collision cases where the UE has a derived QoS rule with precedence value 80 (decimal) and the network sending a signalled QoS rule with the precedence value 80 (decimal). This means the precedence value 80 (decimal) cannot be used for any signalled QoS rule, regardless whether reflective QoS is used or not. - Upon receipt of a 'reserved' value, the receiving entity (UE or network) will immediately trigger the error handling. Hence the observed UE behaviour is as intended, and there is no need for CT1 to update TS 24.501. CT1 kindly asks GSMA’s Terminal Steering Group (TSG) and CT3 to take above information into account, and kindly asks CT3 to update their specifications if necessary.

CT3 would like to inform GSMA’s Terminal Steering Group (TSG) and CT1 that CT3 has agreed on the attached CR 1426 (C3-254289) to 3GPP TS 29.512 v 19.4.0 in order to clarify the handling of precedence value 80 (decimal).

Specifically, the agreed CR clarifies that:

- For derived QoS rules, the UE shall use the standardized value 80 when setting the precedence. The PCF shall also take this value 80 into account when setting the precedence in the range 70 to 99 for reflective QoS, as specified in clause 4.2.6.5.7.

- For signalled QoS rules, the value 80 shall not be used when setting the precedence attribute in the PCC rule, as specified in clause 5.6.2.6.

# 2	Actions

To GSMA’s Terminal Steering Group (TSG).

ACTION: 	CT3 kindly asks GSMA’s Terminal Steering Group (TSG) to take the attached agreed CR into account.

To CT1.

ACTION: 	CT3 kindly asks CT1 to take the attached agreed CR into account.

# 3	Dates of next meetings

CT3#144                           17th November- 21st November 2025............Dallas, US

CT3#145                           9th February- 13th February 2026..................India

CT3 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C3.htm