3GPP TSG-CT WG3 Meeting #143	C3-254341

# Sophia-Antipolis, France, 13 - 17 October 2025

Source:	Ericsson

Title:	Discussion on SID for Interworking between 6G Network and Data Networks

Document for:	Discussion & Agreement

# Agenda Item:	20.1

# Work Item / Release:	6G/Rel-20

Abstract of the contribution:

# 1	Decision/action requested

Discuss the proposals for 6G SID on Interworking between 6G Network and Data Networks.

# 2	References

[1]	3GPP TR 22.870 " SA1 6G study"

[2]	3GPP TR 23.801-01 "Study on Architecture for 6G System"

[3]	3GPP TR 33.801-01 "SA3 6G study"

# 3	Rationale

Upon Stage 2 has started SID on architecture for 6G System in SA2 with the SID SP-250806 approved in SA#108, SID on Security for 6G System in SA3 with the SID SP-251233 approved in SA#109, and CT3#143 meeting guideline "DP(s) & SID(s) related to 6G will be treated", considering the corresponding Stage 1 and Stage 2 studies and requirements need to be closely followed up to see whether and how the related interworking between 6G Network and Data Networks are impacted to support 6G new services and architectures. Moreover, the related studies are needed in detail from protocol level and also some areas not covered in Stage 2 in history e.g., Accounting between Core Network and DN-AAA server, Resilience between Core Network and Data Networks etc. All these aspects need CT3 early study in scope of SID on Interworking between 6G Network and Data Networks to effectively fulfil the Stage 1 and industry requirements.

# 4	Discussion

## 4.1	Current status of 6G related studies in Stage 1 & Stage 2

Current status of 6G SIDs in Stage 1 and Stage 2 working groups is summarised in table below:

| Working Group | References | Observations |
| --- | --- | --- |
| SA1 | SA1 6G study; TR 22.870 | TR 22.870 contains high-level Stage 1 use-cases that need to be analysed by Stage 2 WG (SA2, SA3, SA6) |
| SA2 | Study on Architecture for 6G System; TR 23.801-01 | TR 23.801-01 contains the skeleton without content. |
| SA3 | SA3 6G study; TR 33.801-01 | SA3 6G study SID was approved in September SA plenary. TR 33.801-01 is not available and SA3 will discuss the structure, scope and potential content in October 2025 meeting. |

Observation 1: Stage 1 in TR 22.870 has defined different use cases and requirements in 6G which include new capabilities/applications and also system level enhancements like resilience and security requirements. Stage 2 SIDs following the 6G use cases requirements while still some aspects are not covered for interworking between 6G network and Data Networks.

Considering current status of Stage 2 working groups on 6G development (Observation 1) and the CT3#143 meeting guideline "DP(s) & SID(s) related to 6G will be treated", the generic principles for study on Interworking between 6G Network and Data Networks are proposed as below:

Generic principles:

- CT3 check Stage 1 use cases and requirements vs. Stage 2 SIDs to identify whether Stage 2 SIDs Work Tasks covering the related interworking between 6G Network and Data Networks required in Stage 1:

  - for Stage 2 Work Tasks that covering Stage 1 requirements, CT3 SID justification and objective should follow the Stage 2 study conclusion and related normative requirements; and

  - for Stage 1 related requirements not covered in Stage 2, check whether belongs to Stage 2 scope in history, if yes could issue LS to indicate Stage 2 to add them early in time and waiting reply from stage 2 on whether and how to include them in CT3 SID scope, otherwise for the contents not covered in Stage 2 scope in history e.g., Accounting with DN-AAA sever defined in CT3 directly in history, could be included in CT3 SID scope which can be studied early to better align with Stage 1 requirements.

- CT3 further study the protocol level requirements for Interworking between 6G Network and Data Networks to be concluded in CT3 SID scope.

Ericsson is interested in leading the study for Interworking between 6G Network and Data Networks in CT3 remit and would like to discuss below possible working areas in the SID.

## 4.2	Stage 2 SIDs covering 6G Network interworking with DNs

### 4.2.1	Authentication/Authorization of Interworking with DN-AAA server

Stage 2 dependency analysis: the implementation of authentication/authorization between Core Network and DN-AAA server in 3GPP TS 29.061 and 3GPP TS 29.561 is based on the corresponding definitions in Stage 2 TSes, hence needs to continue such dependency to ensure Stage 3 implementation follows the Stage 2 normative requirements.

Proposal 1: All the authentication/Authorization studies for 6G Network interworking with DN-AAA server should be included in SID while need to wait and align with the corresponding Stage 2 study conclusions and requirements.

### 4.2.2	User plane Interworking with Data Networks

Stage 2 dependency analysis: the implementation of user plane for Core Network interworking with Data Networks in 3GPP TS 29.061 and 3GPP TS 29.561 in history is based on the corresponding Stage 2 (SA2 & SA4 and possible SA6) TS requirements while some detail protocol level implementation is based on study of the applicability and detail definitions of corresponding protocols (e.g., IETF protocols etc.) to meet the stage 2 normative requirement, hence needs to continue such dependency to ensure Stage 3 implementation follows the Stage 2 normative requirements.

Proposal 2: All the user plane studies for 6G Network interworking with Data Networks should be included in SID while should wait and align with the corresponding Stage 2 studies and requirements, meanwhile CT3 can early study the applicability and detail contents of the related protocols to better fulfil the stage 2 requirement and avoid negative impacts.

### 4.2.3	Security Interworking with Data Networks

Stage 2 dependency analysis: the implementation of Security for Core Network interworking with Data Networks in 3GPP TS 29.061 and 3GPP TS 29.561 is based on the corresponding SA3 TS requirements while some detail protocol level definition is implemented with combination of both study the corresponding protocols e.g., IETF protocol capabilities and specify the corresponding 3GPP definition to meet SA3 normative requirement, hence needs to continue such dependency to ensure Stage 3 implementation follows the SA3 normative requirements.

Proposal 3: All the Security studies for 6G Network interworking with Data Networks can be included in SID while should wait and align with the corresponding SA3 studies and requirements, meanwhile CT3 can early study the applicability and detail contents of related protocols to better fulfil the stage 2 requirement and avoid negative impacts.

## 4.3	Stage 2 SIDs not covering 6G Network interworking with DNs

### 4.3.1	Accounting for Interworking with DN-AAA server

Stage 2 dependency analysis: the implementation of Accounting for Core Network interworking with DN-AAA server in 3GPP TS 29.061 and 3GPP TS 29.561 in history is implemented directly by CT3 and no specific Accounting requirements in Stage 2 related TS, hence needs to continue to early start the related study in CT3 on effective Accounting for 6G Network interworking with DN-AAA server.

Proposal 4: All the Accounting related studies for 6G Network interworking with DN-AAA server should be included in SID scope and start related study early in CT3.

### 4.3.2	Resilience for Interworking with Data Networks

Stage 2 dependency analysis: the Resilience scope is included in Stage 1 TR  22.870 and CT4 responsible TS 23.527 5G System; Restoration Procedures only covering the restoration procedures within 5GC, not covering the scope of interworking with Data Networks, also in history CT3 has conducted similar SID within CT3 remit e.g., Study for PCRF failure and restoration in Rel-10, hence needs to early start the resilience related study in CT3 remit for 6G Network interworking with Data Networks.

Proposal 5: All the Resilience related studies for 6G Network interworking with Data Networks should be included in SID and can start related study early in CT3.

## 4.4	Protocol level further studies

SA1 defined a lot of use-cases regarding novel 6G services (e.g., AI Agents, ISAC, etc.) with the related ongoing Stage 2 studies related to potential new protocols matching the requirements, whether and how the existing protocols to be enhanced for effective interworking with Data Networks to commonly carry the new 6G services together with the existing services and whether some legacy protocols should be obsoleted for 6G Network interworking with Data Networks etc., could also be studied in CT3 with stage 3 experiences and may be indicated by LS with some input to stage 2 related studies.

Proposal 6: Include study on potential new protocols or enhancement on existing protocols for new 6G services based on SA2 requirements in SID. The work task study will start when Stage 2 architecture requirements are stable and should be based on SA2, SA3 and SA6 requirements for the new 6G services related Interworking between 6G Network and Data Networks.

3GPP TS 29.061 and 3GPP TS 29.561 including many legacy protocols which maybe not used for Interworking between 6G Network and Data Networks, and some legacy protocols (e.g., some early dated IETF protocols) are obsoleted and/or replaced by new commonly applied protocols, is also worth to be studied.

Proposal 7: Study whether some legacy protocols should be obsoleted for 6G Network interworking with DNs.

# 5	Summary on SID contents for Interworking between 6G Network and Data Networks

Based on the above analysis and proposals, Ericsson would like to propose the following contents in SID for Interworking between 6G Network and Data Networks.

| Work Area Description | Interworking between 6G Network and Data Networks |
| --- | --- |
| Candidate Work Tasks | 1. Design Authentication/Authorization related with new 6G services for Interworking between 6G Network and DN-AAA server (based on SA2 requirements). (NOTE) 2. Design user plane Interworking between 6G Network and Data Networks (based on SA2 & SA4 and possible SA6 requirements). (NOTE) 3. Design Security for Interworking between 6G Network and Data Networks (based on SA3 requirements). (NOTE) 4. Study Accounting of new 6G services for Interworking between 6G Network and DN-AAA server. 5. Study Resilience for Interworking between 6G Network and Data Networks. 6. Study on potential new protocols for new 6G services based on SA2 requirements, enhancement for existing protocols to support new 6G services. 7. Study whether some legacy protocols to be obsoleted for Interworking between 6G Network and Data Networks. |
| NOTE: The work task study will start when Stage 2 architecture, functionalities and/or security requirements are stable and should be based on Stage 2 requirements on the Interworking between 6G Network and Data Networks. |  |

# 6	Proposal

Ericsson would like to lead the study for Interworking between 6G Network and Data Networks in CT3 remit and submitted the SID for Interworking between 6G Network and Data Networks for discussion, support and agreement.