3GPP TSG CT WG3 Meeting #143	C3-254351

Sophia-Antipolis, France, 13 – 17 October 2025	(Revision of C3-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.569 | CR | 0010 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support Device Location in Ambient IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Upon SA#109 approved TS 23.369 CR 0007 (S2-2507755) added indication of location information requested in the Naiotf_AIoT_Inventory and Naiotf_AIoT_Command service operations, also added location information of each target AIoT Device in the Naiotf_AIoT_Notify service operation, hence the corresponding changes are needed in this TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added location information requested indicator in the InventoryReq and CommandReq data types and added location information of each target AIoT Device in the DevicesRepInfo data type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not support Device Location in Naiotf_AIoT API required by SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2, 6.1.6.2.4, 6.1.6.2.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible feature to the OpenAPI file of the following APIs: TS29569_Naiotf_AIoT.yaml TS29522_AIoT.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

##### 6.1.6.2.2	Type: InventoryReq

Table 6.1.6.2.2-1: Definition of type InventoryReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that triggered the request. |  |
| locInfoReqInd | boolean | O | 0..1 | Indicates the location information of each target AIoT Device is requested. - "true" indicates that the location information of each target AIoT Device is requested. The presence of this attribute with the value "false" shall be prohibited. |  |
| targetArea | AiotArea | C | 0..1 | Contains the target AIoT Service Area within which the requested Inventory operation shall apply. (NOTE) |  |
| targetDevices | AIoTDevices | C | 0..1 | Contains the target AIoT device(s) related information. (NOTE) |  |
| numDevices | Uinteger | O | 0..1 | Contains the approximative number of the targeted AIoT device(s). |  |
| timeInterval | DurationSec | O | 0..1 | Contains the time interval to be used for results aggregation. |  |
| notifUri | Uri | M | 1 | Contains the URI via which the AIoT Inventory operation related notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** 2nd Change ***

##### 6.1.6.2.4	CommandReq

Table 6.1.6.2.4-1: Definition of type CommandReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that triggered the request. |  |
| commandType | CommandType | M | 1 | Contains the type of the requested command. |  |
| locInfoReqInd | boolean | O | 0..1 | Indicates the location information of each target AIoT Device is requested. - "true" indicates that the location information of each target AIoT Device is requested. The presence of this attribute with the value "false" shall be prohibited. |  |
| targetArea | AiotArea | C | 0..1 | Contains the target area within which the requested Command operation shall apply. (NOTE) |  |
| targetDevices | AIoTDevices | C | 0..1 | Contains the target AIoT device(s) related information. (NOTE) |  |
| numDevices | Uinteger | O | 0..1 | Contains the approximative number of the targeted AIoT device(s). |  |
| msgSize | Uinteger | O | 0..1 | Contains the approximative message size in units of Bytes. This attribute may be present only if the "commandType" attribute is set to "READ". |  |
| offset | Uinteger | C | 0..1 | Contains the offset, expressed in units of bytes. This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the offset from which to read the application data. - If the "commandType" attribute is set to "WRITE", this attribute contains the offset from which to write the application data. |  |
| length | Uinteger | C | 0..1 | Contains the length of application data, expressed in units of bytes (i.e., byte length). This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the length of application data to read. - If the "commandType" attribute is set to "WRITE", this attribute contains the length of application data to write. |  |
| data | Bytes | C | 0..1 | Contains the application data to write. This attribute shall be present only if the "commandType" attribute is set to "WRITE". |  |
| notifUri | Uri | M | 1 | Contains the URI via which the AIoT Command operation related notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** 3rd Change ***

##### 6.1.6.2.8	Type: DevicesRepInfo

Table 6.1.6.2.8-1: Definition of type DevicesRepInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| deviceId | AiotDevPermId | M | 1 | Contains the identifier of the AIoT device to which the reporting information is related. |  |
| locInfo | string | C | 0..1 | Contains the location information of the AIoT Device. Based on operator policy, if the "locInfoReqInd" attribute set to "true" is included in the request and if the location of the reader is configured, the AIOTF shall provide this attribute use the Reader ID reported from NG-RAN to determine the AIoT Device Location. The location format is defined based on the SLA between the operator and the AF, which is out of 3GPP scope. |  |
| readCmdRep | Bytes | O | 0..1 | Contains the Read command specific report information for the AIoT device identified by the "deviceId" attribute. This attribute may be present only if the reporting information is related to a Read command operation, i.e., the "commandType" attribute is set to "READ" in the corresponding AIoT Command service operation. |  |

# *** 4th Change ***

# A.2	Naiotf_AIoT API

openapi: 3.0.0

info:

title: AIOTF Ambient IoT Service

version: 1.0.0-alpha.5

description: |

API for the Naiotf_AIoT Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.569 V2.0.0; Ambient IoT Function (AIOTF) Services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.569/

servers:

- url: '{apiRoot}/naiotf-aiot/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

paths:

/request-inv:

post:

summary: Request to perform an AIoT Inventory operation.

operationId: InventoryRequest

tags:

- AIoT Inventory Request (custom operation without associated resources)

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

- oAuth2ClientCredentials:

- naiotf-aiot

- naiotf-aiot:inventory

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryReq'

responses:

'200':

description: >

OK. The AIoT Inventory request is successfully received and processed, and

AIoT Inventory related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/request-cmd:

post:

summary: Request to perform an AIoT Command operation.

operationId: CommandRequest

tags:

- AIoT Command Request (custom operation without associated resources)

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

- oAuth2ClientCredentials:

- naiotf-aiot

- naiotf-aiot:command

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CommandReq'

responses:

'200':

description: >

OK. The AIoT Command request is successfully received and processed, and

AIoT Command related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CommandResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naiotf-aiot: >

Enables to access all the resources and custom operations of the Naiotf_AIoT API.

naiotf-aiot:inventory: >

Enables to access only the InventoryRequest custom operation (Naiotf_AIoT_Inventory

service operation) of the Naiotf_AIoT API.

naiotf-aiot:command: >

Enables to access only the CommandRequest custom operation (Naiotf_AIoT_Command

service operation) of the Naiotf_AIoT API.

schemas:

#

# STRUCTURED DATA TYPES

#

InventoryReq:

description: Represents the AIoT Inventory request.

type: object

properties:

afId:

type: string

locInfoReqInd:

type: boolean

enum:

- true

description: >

Indicates the location information of each target AIoT Device is requested.

"true" indicates that the location information of each target AIoT Device is requested.

The presence of this attribute with the value "false" shall be prohibited.

targetArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

targetDevices:

$ref: '#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeInterval:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

InventoryResp:

description: Represents the AIoT Inventory response.

type: object

properties:

transId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- transId

CommandReq:

description: Represents the AIoT Command request.

type: object

properties:

afId:

type: string

commandType:

$ref: 'TS29522_AIoT.yaml#/components/schemas/CommandType'

locInfoReqInd:

type: boolean

enum:

- true

description: >

Indicates the location information of each target AIoT Device is requested.

"true" indicates that the location information of each target AIoT Device is requested.

The presence of this attribute with the value "false" shall be prohibited.

targetArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

targetDevices:

$ref: '#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

msgSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

offset:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

length:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

data:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- commandType

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

CommandResp:

description: Represents the AIoT Command response.

type: object

properties:

transId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- transId

AIoTNotif:

description: Represents the AIoT Operations Notification.

type: object

properties:

transId:

type: string

devicesRepData:

type: array

items:

$ref: '#/components/schemas/DevicesRepInfo'

minItems: 1

lastRepInd:

type: boolean

enum:

- true

description: >

Indicates that this is the last reporting from the AIOTF for the AIoT service

operation identified by the "transId" attribute.

true indicates that this is the last report.

This attribute shall be present only when this is the last reporting from the NF service

consumer for the AIoT service operation identified by the transId attribute.

When present, this attribute shall be set to true. The presence of this attribute set

to the value false is forbidden.

required:

- transId

anyOf:

- required: [devicesRepData]

- required: [lastRepInd]

AIoTDevices:

description: Represents the AIoT device(s) related information.

type: object

properties:

devices:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

minItems: 1

filteringInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotFilteringInformation'

oneOf:

- required: [devices]

- required: [filteringInfo]

DevicesRepInfo:

description: Represents the AIoT device(s) related reporting information.

type: object

properties:

deviceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

locInfo:

type: string

description: >

Contains the location information of the AIoT Device.

Based on operator policy, if the "locInfoReqInd" attribute set to "true" is included in

the request and if the location of the reader is configured, the AIOTF shall provide

this attribute use the Reader ID reported from NG-RAN to determine the AIoT Device

Location. The location format is defined based on the SLA between the operator and the

AF, which is out of 3GPP scope.

readCmdRep:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

required:

- deviceId

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

#

# Data types describing alternative data types or combinations of data types

#

*** End of Changes ***