3GPP TSG-CT WG3 Meeting # 145	C3-260079

# Goa, India, 09 – 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.437 | CR | 0004 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SS_SmManagement API correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following issues related to SS_SmManagement API are identified: - In 6.2.1.6.2.19, mapIds cardinality is wrongly mentioned as 1 even though its array datatype. - In 6.2.1.6.2.16, the typo in the eventFilter name as CHANGE_OF_OBJECT - In 6.2.1.6.3.5, MapObjectChangeEvent typo in the enum namely CHANGED_POSTION - In A.5, 202 response details are missing - In A.5, svc-id, map-id query parameters mandatory is missing in the OpenAPI. - In A.5, mapInf attribute is not mandated in SpatialMapRetResp datatype. - In A.5, mapIds attribute in MapsReport datatype is not mandated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The above issues are fixed appropriately by - In 6.2.1.6.2.16, the typo in the eventFilter name is fixed as CHANGE_OF_OBJECTS - In 6.2.1.6.2.19, mapIds cardinality is updated as 1..N. Typo in the event name is updated. - In 6.2.1.6.3.5, MapObjectChangeEvent enum typo is fixed as CHANGED_POSITION - In A.5, 202 response code handling is add with the corresponding SpatialMapTempResp datatype addtition. - In A.5, svc-id, map-id query parameters are mandated. - In A.5, mapInf is mandated in SpatialMapRetResp |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The quality of the SS_SmManagement API is not up to the mark. Also, it creates the confusion for the implementors due to the inconsistency between the data model and the OpenAPI. Implementations may misinterpret mandatory parameters or schema structure, causing interoperability issues during REST API integration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.6.2.16, 6.2.1.6.2.19, 6.2.1.6.3.5, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible correction to the OpenAPI descriptions of the following APIs: TS29437_SS_SmManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

6.2.1.6.2.16	Type: EventFilter

Table 6.2.1.6.2.16-1: Definition of type EventFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| objList | array(string) | C | 1..N | Contains the list of spatial map objects identifiers, to trigger notification when added, removed, updated etc. This attribute may be present only for "CHANGE_OF_OBJECTS" event. When absent, then the "CHANGE_OF_OBJECTS" event is applicable for any map objects. |  |
| layers | array(string) | C | 1..N | Contains the list of spatial map layers identifiers, to trigger notification when the spatial map layers change is detected. This attribute may be present only for "LAYERS_MODIFICATION" event. When absent, then the "LAYERS_MODIFICATION" event is applicable for any spatial map layer. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# * * * *2nd change * * * *

6.2.1.6.2.19	Type: MapsReport

Table 6.2.1.6.2.19-1: Definition of type MapsReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mapIds | array(SpatialMapId) | M | 1..N | Contains the identifiers of the spatial maps that the report applies to. |  |
| event | MapEvent | M | 1 | Contains the Spatial Map event detected. |  |
| objDet | array(MapObjectChangeReport) | C | 1..N | Contains the information about the changed Spatial Map objects. This attribute shall be present when the "event" value is "CHANGE_OF_OBJECTS". |  |
| lyrDet | SpatialMapLayers | C | 0..1 | Contains the information about the layers of the Spatial Map. This attribute shall be present when the "event" value is "LAYERS_MODIFICATION". |  |

# * * * *3rd change * * * *

6.2.1.6.3.5	Enumeration: MapObjectChangeEvent

The enumeration MapObjectChangeEvent represents the Spatial Map object change event. It shall comply with the provisions defined in table 6.2.1.6.3.3-1.

Table 6.2.1.6.3.5-1: Enumeration MapObjectChangeEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ADDED | Indicates that the spatial map object is added. |  |
| REMOVED | Indicates that the spatial map object is removed. |  |
| CHANGED_POSITION | Indicates that the spatial map object is changed position. |  |
| CHANGED_DIRECTION | Indicates that the spatial map object is changed direction. |  |
| ANY_OTHER | Indicates that the spatial map object is any other. |  |

# * * * *4th change * * * *

# A.5	SS_SmManagement API

openapi: 3.0.0

info:

title: SM Server Spatial Maps Management Service

version: 1.0.0

description: |

API for Spatial Map Management Service.

©2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.437 V19.0.0; Service Enabler Architecture Layer for Verticals (SEAL);

Metaverse Enablement Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.437/

servers:

- url: "{apiRoot}/ssm-smm/v1"

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/spatial-maps:

post:

summary: Create a new Spatial Maps.

operationId: CreateSpatialMaps

tags:

- Spatial Maps (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapCreateReq'

responses:

'201':

description: >

Created. The Spatial Map is successfully created and a representation of the created

Individual Spatial Map resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMap'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'202':

description: >

Accepted. The SM server has accepted the request and processing it.

The assigned Spatial Map Identifier shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapTempResp'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve spatial map information.

operationId: GetSpatialMap

tags:

- Spatial Maps (Collection)

parameters:

- name: svc-id

in: query

description: >

Contains the VAL application service identifier.

schema:

type: string

required: true

- name: map-id

in: query

description: >

Contains the identifier of the spatial map.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapId'

- name: map-layer-list

in: query

description: >

Contains a list of spatial map layers.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapLayers'

- name: aug-with

in: query

description: >

Contains the information augmented with the spatial map layers.

content:

application/json:

schema:

$ref: '#/components/schemas/AugmentLayerList'

responses:

'200':

description: >

OK. The requested spatial map information is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapRetResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/spatial-maps/{spatialMapId}:

parameters:

- name: spatialMapId

in: path

description: >

Represents the identifier of the Individual Spatial Map.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Spatial Map Resource.

operationId: GetIndSpatialMap

tags:

- Individual Spatial Map (Document)

responses:

'200':

description: >

OK. The requested Individual Spatial Map resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMap'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual Spatial Map resource.

operationId: UpdateIndSpatialMap

tags:

- Individual Spatial Map (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMap'

responses:

'200':

description: >

OK. The Individual Spatial Map resource is successfully updated and

a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMap'

'204':

description: >

No Content. The Individual Spatial Map resource is successfully updated and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual Spatial Map resource.

operationId: ModifyIndSpatialMap

tags:

- Individual Spatial Map (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapPatch'

responses:

'200':

description: >

OK. The Individual Spatial Map resource is successfully modified and

a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMap'

'204':

description: >

No Content. The Individual Spatial Map resource is successfully modified and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Spatial Map resource.

operationId: DeleteIndSpatialMap

tags:

- Individual Spatial Map (Document)

responses:

'204':

description: >

No Content. The Individual Spatial Map resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Create a new Spatial Maps Subscription.

operationId: CreateSpatialMapsSubs

tags:

- Spatial Maps Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapsSub'

responses:

'201':

description: >

Created. The Spatial Maps Subscription is successfully created and a

representation of the created Individual Spatial Maps Subscription

resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapsSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Spatial Maps Subscription.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Spatial Maps Subscription resource.

operationId: GetIndSpatialMapsSubs

tags:

- Individual Spatial Maps Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Spatial Maps Subscription

resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapsSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Spatial Maps Subscription resource.

operationId: UpdateIndSpatialMapsSubs

tags:

- Individual Spatial Maps Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapsSub'

responses:

'200':

description: >

OK. The Individual Spatial Maps Subscription resource is successfully

updated and a representation of the updated resource shall be returned in the

response

body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapsSub'

'204':

description: >

No Content. The Individual Spatial Maps Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Spatial Maps Subscription resource.

operationId: ModifyIndSpatialMapsSubs

tags:

- Individual Spatial Maps Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SpatialMapsSubPatch'

responses:

'200':

description: >

OK. The Individual Spatial Maps Subscription resource is successfully

modified and a representation of the updated resource shall be returned in the

response

body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapsSub'

'204':

description: >

No Content. The Individual Spatial Maps Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Spatial Maps Subscription resource.

operationId: DeleteIndSpatialMapsSubs

tags:

- Individual Spatial Maps Subscription (Document)

responses:

'204':

description: >

The Individual Spatial Maps Subscription resource

is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

SpatialMapsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapsNotif'

responses:

'204':

description: >

No Content. The Spatial Maps Notification is successfully

received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: "{tokenUrl}"

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SpatialMapCreateReq:

description: >

Represents the spatial map creation request.

type: object

properties:

svcId:

type: string

areaInt:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

notiUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

mapInf:

$ref: '#/components/schemas/SpatialMapCreateInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- svcId

- areaInt

SpatialMapCreateInfo:

description: >

Represents the spatial map information to create.

type: object

properties:

allwEnt:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/AllowedRules'

lyrInf:

type: array

items:

$ref: '#/components/schemas/MapLayerInfo'

minItems: 1

augLyrInf:

type: array

items:

$ref: '#/components/schemas/AugmentLayerInfo'

minItems: 1

anyOf:

- required: [allwEnt]

- required: [lyrInf]

- required: [augLyrInf]

AugmentLayerInfo:

description: >

Represents the augmented spatial map layer information.

type: object

properties:

augWith:

$ref: '#/components/schemas/AugmentLayer'

augLyrDet:

type: array

items:

type: string

minItems: 1

required:

- augWith

SpatialMap:

description: >

Represents the spatial map resource representation.

type: object

properties:

svcId:

type: string

smProf:

$ref: '#/components/schemas/SpatialMapProfile'

augLyrInf:

$ref: '#/components/schemas/AugmentLayerInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- smProf

SpatialMapProfile:

description: >

Represents the spatial map profile information.

type: object

properties:

mapId:

$ref: '#/components/schemas/SpatialMapId'

mapCov:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

allwEnt:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/AllowedRules'

mapLyrList:

$ref: '#/components/schemas/SpatialMapLayers'

objList:

type: array

items:

$ref: '#/components/schemas/MapObjectInfo'

minItems: 1

required:

- mapId

- mapCov

- mapLyrList

SpatialMapLayers:

description: >

Represents the spatial map layers information.

type: object

properties:

lyrList:

type: array

items:

$ref: '#/components/schemas/SpatialMapLayerInfo'

minItems: 1

required:

- lyrList

SpatialMapLayerInfo:

description: >

Represents the individual spatial map layer.

type: object

properties:

mapLyrId:

type: string

mapLyrInf:

$ref: '#/components/schemas/MapLayerInfo'

required:

- mapLyrId

MapLayerInfo:

description: >

Represents the layer specific information of an spatial map layer.

type: object

properties:

area:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

mapAsc:

type: string

anyOf:

- required: [area]

- required: [mapAsc]

MapObjectInfo:

description: >

Represents the spatial map object.

type: object

properties:

objId:

type: string

objType:

type: string

objPos:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Local3dPointUncertaintyEllipsoid'

objInf:

type: string

required:

- objId

- objType

SpatialMapPatch:

description: >

Represents the partial spatial map resource representation.

type: object

properties:

svcId:

type: string

smProf:

$ref: '#/components/schemas/SpatialMapProfile'

augLyrInf:

type: array

items:

$ref: '#/components/schemas/AugmentLayerInfo'

minItems: 1

SpatialMapRetResp:

description: >

Represents the spatial map information returned in response.

type: object

properties:

svcId:

type: string

mapInf:

$ref: '#/components/schemas/SpatialMapProfile'

augLyrInf:

$ref: '#/components/schemas/AugmentLayerInfo'

required:

- mapInf

SpatialMapsSub:

description: >

Represents the spatial map events subscription request.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/MapEventSub'

minItems: 1

svcId:

type: string

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

report:

$ref: '#/components/schemas/SpatialMapsNotif'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventSubs

- svcId

- notifUri

MapEventSub:

description: >

Represents the spatial map event subscription details.

type: object

properties:

event:

$ref: '#/components/schemas/MapEvent'

mapId:

$ref: '#/components/schemas/TargetSpatialMaps'

evntFltr:

$ref: '#/components/schemas/EventFilter'

required:

- event

- mapId

TargetSpatialMaps:

description: >

Represents the target spatial maps for events.

type: object

properties:

mapIds:

type: array

items:

$ref: '#/components/schemas/SpatialMapId'

minItems: 1

area:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

pose:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/Pose'

anyOf:

- required: [mapIds]

- required: [locInfo]

- required: [area]

- required: [pose]

EventFilter:

description: >

Represents the event filters for spatial map events subscription.

type: object

properties:

objList:

type: array

items:

type: string

minItems: 1

layers:

type: array

items:

type: string

minItems: 1

anyOf:

- required: [objList]

- required: [layers]

SpatialMapsSubPatch:

description: >

Represents the spatial map event subscription modification.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/MapEventSub'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

SpatialMapsNotif:

description: >

Represents the spatial maps event notification.

type: object

properties:

subId:

type: string

reports:

type: array

items:

$ref: '#/components/schemas/MapsReport'

minItems: 1

required:

- subId

- reports

MapsReport:

description: >

Represents the spatial map event report details.

type: object

properties:

mapIds:

type: array

items:

$ref: '#/components/schemas/SpatialMapId'

minItems: 1

event:

$ref: '#/components/schemas/MapEvent'

objDet:

type: array

items:

$ref: '#/components/schemas/MapObjectChangeReport'

minItems: 1

lyrDet:

$ref: '#/components/schemas/SpatialMapLayers'

required:

- mapIds

- event

MapObjectChangeReport:

description: >

Represents the map object change details.

type: object

properties:

objInf:

$ref: '#/components/schemas/MapObjectInfo'

objChgEvnt:

$ref: '#/components/schemas/MapObjectChangeEvent'

required:

- objInf

SpatialMapTempResp:

description: >

Represents the SM server has accepted the request and processing it. The assigned

Spatial Map Identifier shall be returned.

type: object

properties:

mapId:

$ref: '#/components/schemas/SpatialMapId'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mapId

AugmentLayerList:

description: >

Represents the list of augment layer values.

type: object

properties:

augLyrs:

type: array

items:

$ref: '#/components/schemas/AugmentLayer'

minItems: 1

required:

- augLyrs

# SIMPLE DATA TYPES

#

SpatialMapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

#

# ENUMERATIONS

#

AugmentLayer:

anyOf:

- type: string

enum:

- VAL_USERS

- SPATIAL_ANCHORS

- type: string

description: >

This string provides forward-compatibility with future extensions to the

enumeration but is not used to encode content defined in the present version of

this API.

description: |

Represents the type of entity.

Possible values are:

- VAL_SERVER: Indicates that the spatial map layer augment information is VAL users.

- VAL_CLIENT: Indicates that the spatial map layer augment information is Spatial

Anchors.

MapEvent:

anyOf:

- type: string

enum:

- CHANGE_OF_OBJECTS

- LAYERS_MODIFICATION

- SPATIAL_MAP_READY

- type: string

description: >

This string provides forward-compatibility with future extensions to the

enumeration but is not used to encode content defined in the present version of

this API.

description: |

Represents the type of entity.

Possible values are:

- CHANGE_OF_OBJECTS: Indicates that the spatial map event is change of objects.

- LAYERS_MODIFICATION: Indicates that the spatial map event is modification of spatial

map layers.

- SPATIAL_MAP_READY: Indicates that the spatial map event is spatial map ready.

MapObjectChangeEvent:

anyOf:

- type: string

enum:

- ADDED

- REMOVED

- CHANGED_POSITION

- CHANGED_DIRECTION

- ANY_OTHER

- type: string

description: >

This string provides forward-compatibility with future extensions to the

enumeration but is not used to encode content defined in the present version of

this API.

description: |

Represents the type of entity.

Possible values are:

- ADDED: Indicates that the spatial map object is added.

- REMOVED: Indicates that the spatial map object is removed.

- CHANGED_POSITION: Indicates that the spatial map object is changed position.

- CHANGED_DIRECTION: Indicates that the spatial map object is changed direction.

- ANY_OTHER: Indicates that the spatial map object is any other.

# Data types describing alternative data types or combinations of data types:

#

# *** End of Changes ***