3GPP TSG CT WG3 Meeting #145	C3-260152

Goa, India, 9 - 13 February, 2026

Source:	Ericsson

Title:	Modification of BDT Energy Indicator

Document for:	DISCUSSION

Agenda item:	19.60

# Work Item / Release:	EnergySys / Release 19

## 1	Introduction

The Background Data Transfer (BDT) framework enables Application Functions (AFs) to request network-optimized time windows for deferred data transmission. The Energy Indicator (energyInd) was introduced to express whether the AF prefers transfers in low-energy consumption periods. Although SA2 included the Energy Indicator as an input to BDT policy determination (TS 23.501, 23.502, 23.503), no Stage-2 requirement allows its modification once a BDT policy is established.

In Release 19, CT3 approved several CRs in TS 29.122, 29.513, 29.519, 29.522, and 29.554 that introduced the capability for AF-triggered modification of the Energy Indicator. However, recent analysis highlights two major concerns:

- Stage-2 and Stage-3 misalignment

- The actual usability of the modification is very limited, since modification is only considered after a BDT warning notification, which only occurs if:

  - The network sends a network performance degradation notification, and

  - The AF had previously requested warning notifications.

## 2	Discussions

From the Stage-2 specifications (TS 23.501 / 23.502 / 23.503), the Energy Indicator is:

- Provided during initial BDT negotiation as an optional input.

- Stored in UDR when the BDT policy is created.

- Used by PCF to determine candidate BDT policies.

However:

- No modification procedure exists in Stage-2 for changing the Energy Indicator after BDT policy creation.

- No re-evaluation logic describes how the PCF should behave if the Energy Indicator changes after initial negotiation is performed.

- No Stage-2 dependency exists stating that Energy Indicator changes should be tied to network performance-based re-negotiation.

Therefore, Stage-3's modifiability of energyInd is unsupported by the Stage-2 architecture.

On the other hand, even though Stage-3 specifications allow modification of the Energy Indicator, this modification is only effective under very restrictive conditions:

- Modification is only considered upon reception of a BDT warning notification. A re-negotiation of BDT policies is triggered only when the PCF sends a BDT warning notification to the AF, and in case the AF had requested warning notifications.

- BDT warning notification itself only occurs if the NWDAF or PCF detects a network performance deterioration, and not upon Energy Consumption changes which is under release 20 evaluation at stage 2 level. If no network degradation is detected:

  - No network performance notification is sent.

  - No warning notification is triggered.

  - No re-negotiation occurs.

  - Therefore, Energy Indicator modification is not applied.

Hence, somehow we could say that Energy Indicator modification in Release 19 is effectively unusable in normal operation, because it only takes effect during a network-initiated warning notification scenario.

This raises the question of why this parameter is modifiable when other conceptually similar parameters (e.g., desired time window, number of UEs) are not.

Hence, Ericsson has found the following observations:

- Observation 1: Only one BDT related requirement (energyInd) is modifiable after BDT policy initial negotiation. Other BDT requirements remain static.

- Observation 2: The modification is only usable in specific circumstances (network degradation–driven renegotiation) not directly related to the energy consumption of the network.

- Observation 3: No Stage-2 support currently exists for energyInd modification and SA2 EnergySys work item analysis within release 20 is not covering this possibility.

Which are introducing the following problems:

- For Observation 1, there is an inconsistency in stage 3 when it comes to considering different BDT related requirements, since other BDT related requirements (e.g. number of UEs or desired time window) cannot be modified.

- For Observation 2, there is no meaningful benefit, due to the use case is constrained to network degradation and not directly to energy consumption variability or applied immediately after the modification.

- For Observation 3, there is an inconsistency between stage 3 and stage 2, and with the handling of other BDT requirements which so far does not seem to be resolved in release 20.

Taking into account all the observations above, the following options can be considered:

- Option 1 — Add a normative note in TS 29.122, 29.519 and 29.554

Solving Observation 2, this option provides strong guidance to implementers by clarifying in a note in the tables where this attribute is defined, that Energy Indicator modification is only considered upon:

- Reception of a BDT warning notification,

- Triggered by a network performance notification,

- And only during the resulting BDT policy re-negotiation cycle .

Constraints:

- Release 19 is frozen.

- Does not solve Observations 1 and 3

Pros:

- Minimal change; preserves backwards compatibility.

- Helps avoid misinterpretation by external implementers.

- Acknowledges the limited usability of the feature.

Cons:

- Does not address Stage-2 misalignment (Observation 3) although we might consider.

- Still leaves an inconsistent architecture where only Energy Indicator can be modified (Observation 1)

Option 2 — Remove the Energy Indicator Modification Capability

Covering Observation 1, 2 and 3 and considering that modification is only possible upon network degradation warning-notification which is not directly related to energy consumption variability, that no similar parameters are modifiable and that Stage-2 provides no such requirement, thus, the cleanest solution is to remove the modification capability entirely from Stage-3 specifications in release 19 and wait for release 20 to clarify its behavior.

This can be done via CRs removing:

- energyInd attribute from PATCH related data structures in at least TS 29.554 and 29.519.

- PCF PATCH processing logic for energyInd

Pros:

- Full alignment across Stage-2 and Stage-3

- Removes ambiguity and inconsistent behaviour.

- Eliminates feature that currently has little practical value.

Cons:

- Requires a normative CR on a frozen release removing an already agreed feature.

## 3	Conclusion and Proposal

Given that Ericsson does not have a strong preference on the options above, it is proposing any of these options for CT3 consideration being opened for discussion.

However, in case option 1 is chosen (in order to cover Observation 3) or no decision is taken about which option to go for, Ericsson suggests to require a Liaison Statement to SA2 to determine whether Energy Indicator modification should be standardized at Stage-2. The LS should request SA2 guidance on:

- Whether modification of Energy Indicator (and maybe other BDT related requirements) is architecturally desired

- How Energy Indicator updates (and maybe other BDT related requirements) should take place.

- How Energy Indicator updates (and maybe other BDT related requirements) should interact with warning notifications.

- Under what conditions (if any) Stage-2 would allow re-evaluation of BDT policies based on updated BDT requirement preferences

SA2 feedback will define whether:

- Energy Indicator modification becomes a proper feature,

- Or whether Stage-3 should be simplified by removing it.