3GPP TSG-CT WG3 Meeting #145	C3-260266

# Goa, India; 09th – 13th February 2026												(revision of C3-260abc)

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.482 | CR | 0018 | rev | - | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the AIMLES_SplitOpNodeRegistration API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2026-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | In cl. 5.2.12.2.2.2: the resource is incorrect. In Figure 5.2.12.2.3.2-1 and Figure 5.2.12.2.4.2-1, the resource URI is incorrect. In cl. 6.1.14.6.3.3, title for the data type is incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the indiated errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.12.2.2.2; 5.2.12.2.3.2; 5.2.12.2.4.2; 6.1.14.6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

5.2.12.2.2.2	AIMLE split operation node registration request

Figure 5.2.12.2.2.2-1 depicts a scenario where a VAL server sends a request to the AIMLE Server for AIMLE Split Operation Node Registration Request (see also clause 8.14.2.4.2 of 3GPP TS 23.482 [9]).

Figure 5.2.1.2.2.2-1: Procedure for AIMLE Split Operation Node Registration Request

1.	In order to register for AIMLE Split Operation Node Registration, the VAL server shall send an HTTP POST request to the AIMLE Server targeting the URI of the corresponding custom operation, with the request body including the SplitOpNodeReg data structure.

2a.	Upon reception of the HTTP POST registration request, the AIMLE server shall perform an authentication and authorization check to determine if the service consumer is permitted to register at the AIMLE server and participate in AIML operations. If the VAL server is authorized to register at the AIMLE server, the AIMLE server shall:

a)	create a new "Individual AIMLE split operation node register Configuration" resource with the received registration information; and

b)	respond with an HTTP "201 Created" status code with the response body including the SplitOpNodeReg data structure and an HTTP "Location" header field containing the URI of the created resource.

2b. If the VAL server is not authorized to register at the AIMLE server, the AIMLE server shall take proper error handling actions, as specified in clause 6.1.14.7, and respond with an appropriate error status code.

# *** Next Change ***

5.2.12.2.3.2	AIMLE split operation node registration update

Figure 5.2.12.2.3.2-1 depicts a scenario where a VAL server sends a request to the AIMLE Server for AIMLE Split Operation Node Registration Update (see also clause 8.14.2.4.3 of 3GPP TS 23.482 [9]).

Figure 5.2.12.2.3.2-1: Procedure for AIMLE Split Operation Node Registration Update

1.	In order to update an Individual Registered AIMLE Split Operation Node Register, the service consumer shall send an HTTP PUT/PATCH request to the AIMLE Server targeting the URI of the corresponding resource (i.e., "Individual Registered AIMLE Split Operation Node Register Configuration"), with the request body including either:

-	the updated representation of the resource within the SplitOpNodeReg data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the SplitOpNodeRegPatch data structure, in case the HTTP PATCH method is used.

2a. Upon reception of the HTTP PUT/PATCH request, the AIMLE server shall perform an authentication and authorization check to determine if the VAL server is permitted to update the targeted registration. If the VAL server is authorized to update the targeted registration at the AIMLE server, the AIMLE server shall:

a)	accordingly update the targeted "Individual AIMLE split operation node register Configuration" resource; and

b)	respond with either:

-	an HTTP "204 No Content" status code; or

-	an HTTP "200 OK" status code with the response body including a representation of the updated resource within the SplitOpNodeReg data structure.

2b. If the VAL server is not authorized to update the targeted registration at the AIMLE server, the AIMLE server shall take proper error handling actions, as specified in clause 6.1.14.7, and respond with an appropriate error status code.

# *** Next Change ***

5.2.12.2.4.2	AIML split operation node deregistration

Figure 5.2.12.2.4.2-1 depicts a scenario where a VAL server sends a request to the AIMLE Server for AIMLE split operation node deregistration (see also clause 8.14.2.4.4 of 3GPP TS 23.482 [9]).

Figure 5.2.12.2.4.2-1: Procedure for AIMLE Split Operation Node Deregistration

1. To deregister itself at the AIMLE server, the VAL server shall send an HTTP DELETE request to the AIMLE server targeting the "Individual AIMLE split operation register Configuration" resource, as specified in clause 6.1.12.3.3.3.2.

2a. Upon reception of the HTTP DELETE request, the AIMLE server shall perform an authentication and authorization check to determine if the VAL server is permitted to deregister at the AIMLE server. If the VAL server	is authorized to deregister at the AIMLE server, the AIMLE server shall:

a)	delete the corresponding "Individual AIMLE Split Operation Node register Configuration" resource; and

b)	respond with an HTTP "204 Not Content" status code.

2b. If the VAL server is not authorized to deregister at the AIMLE server, the AIMLE server shall take proper error handling actions, as specified in clause 6.1.14.7, and respond with an appropriate error status code.

# *** Next Change ***

6.1.14.6.3.3	Enumeration: SuppMlTaskType

The enumeration SuppAimlRoleType represents information regarding the supported AIML role identity of the AIMLE Split Operation Node Register. It shall comply with the provisions defined in table 6.1.14.6.3.3-1.

Table 6.1.14.6.3.3-1: Enumeration SuppMlTaskType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FL_CLIENT | Identifies the supported AIML role of AIMLE Split Operation Node Register is used as FL client. |  |
| FL_SERVER | Identifies the supported AIML role of AIMLE Split Operation Node Register is used as FL server. |  |

# *** End of Changes ***