3GPP TSG CT WG3 Meeting #145	C3-260273

# Goa, India, 9 - 13 February, 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1780 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of AddressingParamProvision API data model for static IP addresses and GPSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_IP_SP_EXP |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | As required in TS 23.502 and also defined in TS 29.503, static UE IP addresses indicate one IPv4 address, one IPv6 prefix, or both, for the subscriber identified by GPSI; hence, GPSI is mandatory for provisioning towards UDM. TS 29.522, however, allows the AF to provide more than two IPv4/IPv6 addresses or combinations via the staticIpAddresses attribute and defines GPSI as optional. This creates inconsistencies on how the NEF shall select valid IP address information, derive GPSI if not provided by the AF, and invoke the Nudm_ParameterProvision service API. - Snippet from 23.502: Table 4.15.6.3i-1: Description of Static IP address assignment parameters Parameters; Description. GPSI; GPSI for which the IP address/prefix applies.. DNN; DNN for which the IP address information applies.. S-NSSAI; S-NSSAI for which the IP address information applies.. IP address; Indicates one static UE IPv4 address or one IPv6 prefix or both, for the subscriber identified by GPSI.. - Snippets from 29.503: Table 6.5.6.2.2-1: Definition of type PpData Attribute name; Data type; P; Cardinality; Description; Applicability. staticUeIpAddressParams; array(StaticUeIpAddressParams); O; 1..N; A list of static IP address assignment parameters, per-DNN, per-S-NSSAI, for the GPSI identified by the {ueId} within the resource URI. (NOTE 4). NOTE 4: In the staticUeIpAddressParams array, there shall be at most one entry for a given DNN and S-NSSAI pair.. Table 6.5.6.2.35-1: Definition of type StaticUeIpAddressParams Attribute name; Data type; P; Cardinality; Description. dnn; Dnn; M; 1; DNN for which the IP address information applies.. snssai; Snssai; M; 1; S-NSSAI for which the IP address information applies.. ipAddresses; array(IpAddress); M; 1..2; Contains one static UE IPv4 address or one IPv6 prefix or both.. |
| Summary of change: |  | This CR proposes to correct the cardinality of the staticIpAddresses attribute to 1..2 and make gpsi a mandatory attribute in the StaticIpAddrParams data type. In addition, a NOTE is also added to the staticIpAddrParams attribute in the AddrPpData data type to clarify that the static IP address parameter can be provided only as one entry for a given DNN and S-NSSAI pair, ensuring alignment with TS 29.503 (refer NOTE 4 in Table 6.5.6.2.2-1 above). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data model misalignment, potentially leading to interoperability errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.38.5.2.2, 5.38.5.2.3, 5.38.5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29522_AddressingParamProvision.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 5.38.5.2.2	Type: AddrPpData

Table 5.38.5.2.2-1: Definition of type AddrPpData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| staticIpaddrParams | array(StaticIpAddrParams) | C | 1..N | Contains the Static IP Addressing Parameters data that the AF requests to provision. This attribute shall be present only when the AF requests to provision static IP addressing parameters. (NOTE) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.38.6. This attribute shall be present only when feature negotiation needs to take place. |  |
| NOTE: In the staticIpaddrParams array, there shall be at most one entry for a given DNN and S-NSSAI pair for the gpsi identified by the gpsi attribute. |  |  |  |  |  |

# *** Second Change ***

##### 5.38.5.2.3	Type: StaticIpAddrParams

Table 5.38.5.2.3-1: Definition of type StaticIpAddrParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnn | Dnn | M | 1 | Contains the DNN to which the provisioned static IP addressing parameters data is related. |  |
| snssai | Snssai | M | 1 | Contains the S-NSSAI to which the provisioned static IP addressing parameters data is related. |  |
| gpsi | Gpsi | M | 1 | Contains the identifier of the UE to which the provisioned static IP addressing parameters data is related. |  |
| staticIpAddresses | array(IpAddr) | M | 1..2 | Contains the provisioned static IP address(es) of the UE. |  |

# *** Third Change ***

##### 5.38.5.2.4	Type: AddrPpDataPatch

Table 5.38.5.2.4-1: Definition of type AddrPpDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| staticIpaddrParams | array(StaticIpAddrParams) | O | 1..N | Contains the modified Static IP address parameters that the AF requests to provision. (NOTE) |  |
| NOTE: The "dnn", "snssai" and “gpsi” attributes within each array element of the "staticIpaddrParams" attribute shall not be modified. In the staticIpaddrParams array, there shall be at most one entry for a given DNN and S-NSSAI pair for the gpsi identified by the gpsi attribute. |  |  |  |  |  |

# *** Forth Change ***

# A.36	AddressingParamProvision API

openapi: 3.0.0

info:

title: 3gpp-addr-pp

version: 1.0.0

description: |

API for Addressing Parameters Provisioning.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V19.5.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

servers:

- url: '{apiRoot}/3gpp-addr-pp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/pp:

get:

summary: Request to retrieve all the active Addressing Parameters Provisionings at the NEF.

operationId: GetAddrParamsProvisionings

tags:

- Addressing Parameters Provisionings (Collection)

responses:

'200':

description: >

OK. All the Individual Addressing Parameters Provisioning resources managed by

the NEF are returned.

If there are no existing Individual Addressing Parameters Provisioning resources

managed by the NEF, an empty array is returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AddrPpData'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new Addressing Parameters Provisioning.

tags:

- Addressing Parameters Provisionings (Collection)

operationId: CreateAddrParamsProvisioning

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AddrPpData'

responses:

'201':

description: >

Created. A representation of the created Individual Addressing Parameters

Provisioning resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AddrPpData'

headers:

Location:

description: >

Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/pp/{ppId}:

parameters:

- name: ppId

in: path

description: >

Represents the identifier of the Individual Addressing Parameters Provisioning

resource.

required: true

schema:

type: string

get:

summary: Request to retrieve an existing Individual Addressing Parameters Provisioning resource.

operationId: GetIndAddrParamsProvisioning

tags:

- Individual Addressing Parameters Provisioning (Document)

responses:

'200':

description: >

OK. The requested Individual Group Parameters Provisioning resource is returned in the

response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AddrPpData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Addressing Parameters Provisioning resource.

tags:

- Individual Addressing Parameters Provisioning (Document)

operationId: UpdateIndAddrParamsProvisioning

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AddrPpData'

responses:

'200':

description: >

OK. The Individual Addressing Parameters Provisioning resource is successfully

updated and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AddrPpData'

'204':

description: >

No Content. The Individual Addressing Parameters Provisioning resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Addressing Parameters Provisioning resource.

tags:

- Individual Addressing Parameters Provisioning (Document)

operationId: ModifyIndAddrParamsProvisioning

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AddrPpDataPatch'

responses:

'200':

description: >

OK. The Individual Addressing Parameters Provisioning resource is successfully

modified and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AddrPpData'

'204':

description: >

No Content. The Individual Addressing Parameters Provisioning resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Addressing Parameters Provisioning resource.

tags:

- Individual Addressing Parameters Provisioning (Document)

operationId: DeleteIndAddrParamsProvisioning

responses:

'204':

description: >

No Content. The Individual Addressing Parameters Provisioning resource is

successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

AddrPpData:

description: Represents the Addressing Parameters data.

type: object

properties:

afId:

type: string

staticIpaddrParams:

type: array

items:

$ref: '#/components/schemas/StaticIpAddrParams'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

StaticIpAddrParams:

description: Represents the Static IP Address Parameters data.

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

staticIpAddresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

minItems: 1

maxItems: 2

required:

- dnn

- snssai

- gpsi

- staticIpAddresses

AddrPpDataPatch:

description: >

Represents the requested modification to an existing Addressing Parameters Provisioning.

type: object

properties:

staticIpaddrParams:

type: array

items:

$ref: '#/components/schemas/StaticIpAddrParams'

minItems: 1

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# *** End of Changes ***