3GPP TSG-CT WG3 Meeting #145		C3-260494

Goa, India, 9 - 13 February, 2026

Title:	LS on BDT Policy re-negotiation conditions considering Energy Indicator

Release:	Rel-19

Work Item:	EnergySys

Source:	3GPP CT WG3

To:	SA2

Cc:

Contact person:	Juan Manuel Fernandez

Juan.manuel.fernandez@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

# 1	Overall description

3GPP CT3 Working Group has observed that clause 5.51.4 in 3GPP TS 23.501 is stating that a re-negotiation process can take into consideration the Energy Indicator and that re-negotiation can be triggered based on energy related information as highlighted below:

### 5.51.4	BDT based on network energy related information

5GS supports BDT (background data transfer) policy selection and re-negotiation process by taking energy indicator from AF into consideration as specified in clause 6.1.2.4 of TS 23.503 [45]. The PCF may make BDT policy decisions based on operator's policy. The PCF may also trigger the re-negotiation of BDT policy with the AF.

Furthermore, as it is specified by 3GPP TS 23.502 in clause 4.16.7.3 Procedure for BDT warning notification, the only existing trigger for BDT policy re-negotiation is network performance degradation, which might not be directly related to energy consumption.

CT3 needs clarification on the following question:

Question 1:

- Based on the provisions of clause 5.51.4 in 3GPP TS 23.501, is it the correct understanding that the change of the Energy Indicator should be considered as a new trigger for BDT policy re-negotiation with the AF, similarly to the exisiting network performance degradation trigger?

# 2	Actions

To SA2

ACTION: 	3GPP CT WG3 asks SA2 to answer the questions and adjust its specifications accordingly, if needed for next CT3 meeting.

# 3	Dates of next TSG CT WG 3 meetings

CT3#146				13th April- 17th April 2026			Malta

CT3#147				18th May- 22th May 2026				China