3GPP TSG-CT WG3 Meeting #142	C3-253293

# Gothenburg, SE, 25 - 29 August 2025

Source:	Samsung

Title:	Pseudo-CR on data model definition for AIMLES_MLModelTraining API.

Spec:	3GPP TS 29.482

Agenda item:	19.41

Document for:	Decision

1. Introduction

The pseudo CR defines the data model for AIMLES_MLModelTraining API.

2. Reason for Change

The data model for AIMLES_MLModelTraining API is FFS. This CR defines the data model for the API.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 v1.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[8]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	IETF RFC 9113: "HTTP/2".

[11]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[12]	IETF RFC 9457: "Problem Details for HTTP APIs".

[13]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[14]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[16]	3GPP TS 24.560: "Artificial Intelligence Machine Learning (AIML) Services - Service Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3".

[17]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[18]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[19]	3GPP TS 29.558: "5G System; Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

* * * Second Change * * * *

#### 6.1.8.6	Data Model

##### 6.1.8.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.8.6.1-1 specifies the data types defined for the AIMLES_MLModelTraining API.

Table 6.1.8.6.1-1: AIMLES_MLModelTraining API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MemberInfo | 6.1.8.6.2.7 | Represents the list of AIMLE clients selected or de-selected for the ML model training. |  |
| MemberSelCriteria | 6.1.8.6.2.4 | Represents the criteria to be continuously monitored for selecting the member clients. |  |
| MlModelInfo | 6.1.8.6.2.5 | Represents the ML model information. |  |
| MlModelTrainNotif | 6.1.8.6.2.3 | Represents the ML Model training notification |  |
| ModelDomain | 6.1.8.6.3.6 | Represents the domain for which the model can be used. |  |
| TrainRequest | 6.1.8.6.2.2 | Represents the ML Model training request |  |
| PerfParam | 6.1.8.6.2.8 | Represents the output of training, e.g., ML model performance parameters for the training. |  |

Table 6.1.8.6.1-2 specifies data types re-used by the AIMLES_MLModelTraining API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIMLES_MLModelTraining API.

Table 6.1.8.6.1-2: AIMLES_MLModelTraining API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| LocationArea5G |  | 3GPP TS 29.122 [5] | Used to indicate a location area represented as list of geographic areas, civic addresses and network area. |  |  |
| Uri | 3GPP TS 29.122 [2] | Represent an URI, used to indicate the notification URI. |  |
| ClientCapability | 3GPP TS 29.560 [16] | Represents the client capability information. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Indicates the time window. |  |
| Endpoint | 3GPP TS 29.558 [19] | Represents the endpoint information. |  |
| Float |  | 3GPP TS 29.571 [15] | Used to represent the fractional part of the proximity range in the reference UE details. |  |  |
| MlModel | Clause 6.2.1.6.2.4 |  |  | Represents the ML model that has to be distributed to the selected member clients for training. |  |

##### 6.1.8.6.2	Structured data types

6.1.8.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 6.1.8.6.2.2	Type: TrainRequest

Table 6.1.8.6.2.2-1: Definition of type TrainRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trnType | TrainingType | M | 1 | Contains the type of training to be performed. |  |
| members | array(AimleClientId) | C | 1..N | Contains the list of member clients to be utilized for training the ML model. (NOTE 1) |  |
| memSelCrit | MemberSelCriteria | C | 0..1 | Contains the criteria that needs to be continuously monitored for selecting the member clients. (NOTE 1) |  |
| modelInf | MlModelInfo | C | 0..1 | Contains the ML model that has to be distributed to the selected member clients for training. (NOTE 2) |  |
| modelReq | MlModel | C | 0..1 | Contains the requirement for selecting a model to be trained and the filtering criteria for selecting the model. (NOTE 2) |  |
| memUpdNotif | Boolean | O | 0..1 | Indicates whether the requestor needs to be notified whenever there is update related to new member clients selected or de-selected. Set to "true" to indicate that requestor needs to be notified whenever there is update related to new member clients selected or de-selected. Default value is "false" if omitted. |  |
| notifUri | Uri | C | 0..1 | Contains the notification URI where notifications should be sent. This attribute shall be present when the attribute "memUpdNotif" is set to "true". |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.3	Type: MlModelTrainNotif

Table 6.1.8.6.2.3-1: Definition of type MlModelTrainNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| members | array(MemberInfo) | C | 1..N | Contains the list of AIMLE clients selected or de-selected for the ML model training. (NOTE) |  |
| trainOut | PerfParams | C | 0..1 | Contains the output of training, e.g., ML model parameters for the training. (NOTE) |  |
| trainErr | TrainingErr | C | 0..1 | Contains the list of errors, if any, encountered during training process. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

.

###### 6.1.8.6.2.4	Type: MemberSelCriteria

Table 6.1.8.6.2.4-1: Definition of type MemberSelCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientLoc | LocationArea5G | C | 0..1 | Contains the location of the AIMLE client for the AIML service that needs to be monitored. |  |
| clientAvailability | TimeWindow | C | 0..1 | Contains the required availability duration of the AIMLE client. |  |
| clientCapability | ClientCapability | C | 0..1 | Contains the required client capability information. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.5	Type: MlModelInfo

Table 6.1.8.6.2.5-1: Definition of type MlModelInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelId | string | C | 0..1 | Contains the identifier for ML model. |  |
| mlModelLoc | Endpoint | C | 0..1 | Contains the URI, fqdn or address that maps to the resource where ML model is stored. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.6	Type: MemberInfo

Table 6.1.8.6.2.6-1: Definition of type MemberInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientSel | boolean | C | 0..1 | Indicates whether the AIMLE client is selected or not. 0 represents the client is not selected. 1 represents the client is selected. |  |
| clientUri | Uri | C | 0..1 | Contains the URI information of the AIMLE clients. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.7	Type: PerfParam

Table 6.1.8.6.2.7-1: Definition of type PerfParam

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| modelAccuracy | integer | C | 0..1 | Indicates the achieved ML model accuracy, expressed as a percentage. Minimum: 0, maximum: 100. |  |
| modelPrecision | Float | C | 0..1 | Represents the accuracy for the positive predictions made by the model. Minimum: 0, maximum: 1. |  |
| modelRecall | Float | C | 0..1 | Represents model's ability to identify all the actual positive instances within a dataset. Minimum: 0, maximum: 1. |  |
| modelF1Score | Float | C | 0..1 | Represents the combined metric for precision and recall. Minimum: 0, maximum: 1. |  |
| errorMeanSquare | Float | C | 0..1 | Represents the mean squared error between predicted values and actual values. |  |
| errorMeanAbs | Float | C | 0..1 | Represents the mean absolute error between predicted values and actual values. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 6.1.8.6.3	Simple data types and enumerations

###### 6.1.8.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 6.1.8.6.3.2	Simple data types

The simple data types defined in table 6.1.8.6.3.2-1 shall be supported.

Table 6.1.8.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| AimleModelId | string | Represents the ML model identifier. |  |

###### 6.1.8.6.3.3	Enumeration: TrainingType

The enumeration TrainingType represents the type training requested. It shall comply with the provisions defined in table 6.1.8.6.3.3-1.

Table 6.1.8.6.3.3-1: Enumeration TrainingType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TRAIN_VFL | Indicates that the training type is horizontal federated learning |  |
| TRAIN_HFL | Indicates that the training type is vertical federated learning |  |

###### 6.1.8.6.3.4	Enumeration: TrainingErr

The enumeration TrainingErr represents the training error encountered during ML model training. It shall comply with the provisions defined in table 6.1.8.6.3.4-1.

Table 6.1.8.6.3.4-1: Enumeration TrainingErr

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UNDERFITTING | Indicates that the trained model is underfitting the training data. |  |
| OVERFITTING | Indicates that the trained mode is overfitting the training data. |  |
| PERFORMANCE_ERRORS | Indicates that the trained model is unable to meet the desired performance. |  |
| DATA_LEAKAGE | Indicates that there is data leakage from evaluation data set to the training data. |  |

* * * End of Changes * * * *