3GPP TSG-CT WG4 Meeting #130	C3-253xxx

Göteborg, Sweden; 25th – 29th August 2025

# Source:	China Mobile

# Title:	New WID on Protocol for AI Data Collection from UPF

# Document for:	Approval

# Agenda Item:	19.2

3GPP TSG-CT WG4 Meeting #130	C4-253062

Göteborg, Sweden; 25th – 29th August 2025

# Source:	China Mobile

# Title:	New WID on Protocol for AI Data Collection from UPF

# Document for:	Approval

# Agenda Item:	19.2.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	WID on Protocol for AI Data Collection from UPF

Acronym:	PAIDC_UPF

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_PAIDC_UPF | CT4 | 1040005 | Study on Protocol for AI Data Collection from UPF |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

3	Justification

The 5G service-based architecture enables flexible information exchange via standardized SBIs, providing a framework for AI/ML functions (e.g., NWDAF) to collect data from diverse sources.

As AI/ML adoption increases for network automation and analytics, an optimized protocol is needed to efficiently collect data from user-plane (e.g., Nupf per TS 29.564) and control-plane (e.g., Nnwdaf per TS 29.520) sources over SBIs. This is critical for high-volume/frequent data transfers to ensure AI/ML system responsiveness and scalability without degrading network performance.

Based on the background above, the SID FS_PAIDC_UPF (CP-251039) was approved to study the Protocol for AI Data Collection from UPF. In this SID, the following two key issues were studied:

-	Key Issue#1: Identifying and lowering the network performance impacts of intensive data collection from UPF;

-	Key Issue#2: Data collection protocol selection.

TR 29.889 was sent for information to TSG#108 and for approval to TSG#109. A WID on Protocol for AI Data Collection from UPF is needed to standardize the solutions agreed for normative work according to the conclusions of the study documented in Clause 8 of TR 29.889.

4	Objective

The aim of this work item is to specify the solutions concluded to be standardized in TR 29.889 Clause 8 Conclusions.

Stage 3 objectives:

CT4:

- Enhancement of Nupf_EventExposure service to enable the NF service consumer to create a UPF event exposure subscription with reduced reporting instructions, and to skip generating event reports according to these instructions;

- Enhancement of Nupf_EventExposure service to enable bundling of event reports of different UPF event exposure subscriptions;

- Enhancement of Nupf_EventExposure service to enable the NF service consumer to create a UPF event exposure subscription targeting ANY UE with information that identifying the 5G VN group, PDU session type or RAT;

- Enhancement of PFCP session establishment/modification requests to support SMF sending the 5G VN group identification for a 5G VN group member's PFCP session (PDU session).

CT3:

- Enhancement of Nsmf_EventExposure service to enable the NF service consumer to create an event exposure subscription with reduced reporting instructions, and to forward these instructions to the UPF;

- Enhancement of Nsmf_EventExposure service to enable bundling of event reports of different UPF event exposure subscriptions, and to forward these parameters to the UPF;

5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A |  |  |  |  |  |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.564 | - Enhancement of Nupf_EventExposure service to support the event reports reduction, event reports bundling, event subscription optimization using 5G VN group identification, PDU session type or RAT. | TSG#110 (Dec, 2025) | CT4 |
| 29.508 | - Enhancement of Nsmf_EventExposure service to support the event reports reduction, event reports bundling. | TSG#110 (Dec, 2025) | CT3 |
| 29.571 | - Potential enhancement on common data type definition. | TSG#110 (Dec, 2025) | CT4 |
| 29.244 | - Enhancement on PFCP session establishment/modification requests to support SMF sending the 5G VN group identification. | TSG#110 (Dec, 2025) | CT4 |

6	Work item Rapporteur(s)

Wang Rong, China Mobile, wangrongyjy@chinamobile.com

7	Work item leadership

CT4

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

China Mobile

CATT

China Telecom

Deutsche Telekom

Huawei

Nokia

Orange

SK Telecom

vivo