3GPP TSG-CT WG4 Meeting #131	C4-254039

# Sophia Antipolis, France; 13th – 17th October 2025

Source:	Huawei

Title:	Discussion on study on Protocols enhancement and evolution within Core Network of 6G system

Agenda item:	20.2.1

Document for:	Discussion

1. Introduction

With the rapid advancement of emerging technologies such as the Artificial Intelligence (AI), Integrated Sensing and Communication (ISAC) and etc., the 6G system is required to deliver enhanced capabilities, including ultra-high bandwidth, ultra-low latency, massive connectivity, and high reliability. As a critical component of the 6G system, the core network undertakes key functions such as user access control, data transmission, and service management. Protocols within the core network serve as the foundation for its normal operation. However, existing core network protocols in the 5th Generation (5G) system can no longer fully meet the new requirements of the 6G system. Therefore, in-depth research on the enhancement and evolution of 6G core network protocols is of great significance.

SA1 has kicked off the FS_6G_REQ work item to map out 6G use cases and requirements, while SA2 has started the FS_6G_ARC task to define the architecture of 6G systems. To guarantee that stage-3 protocols can satisfy 6G service-driven requirements and architectural constraints, CT4 needs an early exploratory study.

2. Discussion

SA1’s FS_6G_REQ and SA2’s FS_6G_ARC have been launched to deliver stage-1 use-cases/requirements and stage-2 architecture respectively. At SA#108 the TSGs approved the 3GPP 6G SA2 study item (SP-250806). The SA2 SID Study on Architecture for 6G System has defined as following:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

It is considered that there are several aspects should be studied in CT4: control plane (CP) protocol evolution, user plane (UP) protocol evolution, and data framework protocol evolution. It aims to identify technical gaps in existing 5G CN protocols against 6G service requirements and study new protocol selections to fulfill the new 6G service requirements (e.g. data framework), analyze protocol optimization directions, and provide foundational technical inputs for subsequent 6G CN protocol standardization. CT4, as the responsible body for core network protocol design, needs to address the following challenges to meet the outcomes of SA1 and SA2:

- Control Plane Protocol Gaps: Existing 5G control plane protocols (e.g., HTTP/2 for SBA) may not fully meet 6G’s requirements for low-latency signaling, high reliability, and flexible function expansion. For instance, it requires evaluating new protocol options (e.g., HTTP/3) and optimizing existing interfaces.

- User Plane Protocol Limitations: 5G’s GTP-U protocol, while suitable for traditional traffic, may need enhancements to support 6G’s new user plane requirements, such as UE-Network collaboration for dynamic traffic adjustment and QoS mechanisms tailored for AI traffic (e.g., priority scheduling, bandwidth guarantee).

- Lack of Data Framework Protocols: 5G does not have a dedicated data framework protocol system to support large-scale data transmission for AI and sensing. The data framework, as defined in WT#5, requires studying protocol principles, key capabilities (e.g., efficient transmission, security), and interface definitions to ensure end-to-end data handling efficiency and security.

The expected work includes:

- Study on the Control Plane Protocol enhancement and evolution, including:

- Study the potential protocols for Control Plane, e.g. HTTP/3

- Study and evaluation of the protocols of the Control Plane interface (e.g. SBI, AI Agent-based Interface)

- Study on the User Plane Protocol enhancement and evolution, including:

- Study the potential protocols for User Plane, e.g. GTP-U enhancement

- Study the potential protocols for UE-Network Collaboration of User Plane.

- Study the potential protocols for Content Delivery enhancement, e.g. MoQ, HTTP

- Study the QoS mechanism to support the new traffic (e.g. AI traffic)

- Study on the Data Framework related protocols, including:

- Study principles and guidelines for Data Framework related protocols definition.

- Study key capabilities of the Data Framework related protocols, e.g. Efficient and reliable transmission, Security and etc.

CP protocol, UP protocol, and Data Framework-related Protocols are not isolated in the 6G core network; they rely on and constrain each other to achieve end-to-end service support. Separate study items will lead to redundant work in multiple links, while a merged project can significantly improve efficiency, a merged study item can fully consider these cross-protocol technical connections, ensuring coordinated design and avoiding functional gaps or contradictions.

The study of 6G core network CP Protocols, UP Protocols, and Data Framework-related Protocols as a single merged project is more appropriate. It could ensure technical coordination between protocols, improve research efficiency, optimize resource utilization, and avoid risks such as inconsistent standards.

It proposed to have one study items in CT4 on protocol enhancement and evolution within CN of 6G systems.

3 Conclusion

This paper has provided an overview on the study work on Protocols enhancement and evolution within Core Network of 6G system to estimate the content of a CT SID for the expected work.

It proposed to have one study items in CT4 on protocol enhancement and evolution within CN of 6G systems.