3GPP TSG-CT WG4 Meeting #154	C4-254104

Sophia Antipolis, France; 13th – 17th October 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on P-CSCF Failure recovery with UAC Access Category 9

Spec:	3GPP TR 29.867 v0.2.0

Agenda item:	21.4

Document for:	Approval

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Solution of the Technical Report is missing.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 29.867 v0.2.0.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.380: "IMS Restoration Procedures".

[3]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on Session Control Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[7]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[8]	3GPP TR 29.866: "Study on IMS Disaster Prevention and Restoration Enhancement".

[9]	IETF RFC 5626: "Managing Client Initiated Connections in the Session Initiation Protocol (SIP)".

[10]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[11]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[12]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[x]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1"

* * * Next Change * * * *

6.X	Solutions

Solution #X: Access restriction with UAC Access Category 9

6.X.1	Description

This solution is proposed to address Key Issue #1: "Preventing UE SIP Registration attempts during P-CSCF Failures." In KI#1, UEs continue to initiate the SIP registration request when the IMS is unable to respond, resulting in sustained traffic congestion. Therefore, it is required that the solution prevents repeated registration attempts by UEs under severe P-CSCF failure conditions.

To address the issue, this proposal solution proposes delegating by using Unified Access Control (UAC, defined in 3GPP TS 22.261 [x]). UAC has the access categories and access identifiers to control the restriction of UE accessing more flexible.

For this KI#1, this solution uses the access category 9 which is reserved for MO IMS registration related signaling and defined as “Includes IMS registration related signaling, e.g. IMS initial registration, re-registration, and subscription refresh.”. Access control may be applied to communications from UEs using access category 9, and the barring rate may be gradually relaxed over time to avoid concentrated load.

Since KI#1 indicates the situation of severe P-CSCF Failures and it triggers the total IMS service down, affecting for whole PLMN is allowed. In addition, since access category 9 targets only IMS registration-related signalling, barring can be applied without impacting other types of communications.

6.X.1.1	Manual recovery

The manual recovery flow of P-CSCF failure is based on the regular operation flow of UAC barring.

- Upon detection of a P-CSCF failure, the operator initiates UAC barring for UEs of access category 9.

- UAC barring information is immediately broadcast to all UEs via SIB1 in RRC layer.

- Based on the configured barring rate, IMS initial registration, re-registration, and subscription refresh requests from affected UEs are suppressed prior to transmission. Affected UEs cannot retry until the barring timer is ended.

- When the situation changes, e.g. P-CSCF recovery, the operator updates the barring rate to gradually increase the number of UEs permitted to access IMS services.

Since UAC restricts on RRC layer, ongoing transactions	are not affected and continue / retry to connect with established NAS session.

6.X.1.2	Automatic recovery

The operator may automate UAC recovery procedures based on operator’s policy. The detection of P-CSCF failure may be performed based on KPI data, for example, alarms, increased failure rates of IMS traffic or status monitor on surrounding node e.g. PCRF/S-CSCF/PGW/UPF.

6.X.2	Impacts on services, entities, and interfaces

No impact.

6.X.3	Pros

The pros of the solution:

- This solution only consists of the existing features.

- IMS registration is immediately restricted once UAC barring is configured.

6.X.4	Cons

The cons of the solution:

- Automatic recovery requires operator’s policy, and is out of 3GPP scope.

- There is no restriction on the signalling that already establishes an RRC connection or the procedure for restoration.

- Need to consider the enhancement of access identifiers to support roaming / MVNO scenarios.

* * * End of Changes * * * *